# CONTRACT STANDARDS AND DEVELOPMENT

NORTH CAROLINA DEPT OF TRANSPORTATION

TECHNICAL SERVICES - CENTURY CENTER B

1020 BIRCH RIDGE DRIVE, RALEIGH NC  27610

MAIL SERVICE CENTER 1591, RALEIGH NC  27699

DIRECT TELEPHONE NUMBER:  919-707-6953

### 1st PLAN REVIEW

#### CONTRACT #  	47138.3.1	    		TIP # I -6003         .

WBS # / FA #	C204880	     		FORSYTH COUNTY

PREPARED BY	ROGERS/JAMES		DATE 08/21/2023

CHECKED BY  	MONTY BROWN  		DATE 08/23/2023

LETTING  	21 NOV 2023       		SHEET 1 OF 1

- Title Sheet: Please remove the “Final Plans” note. “Final Plans” note has been removed

- Sheet 1A: Please add Standards 560.02, 840.00, 840.25, 840.27, 840.66, 850.10, 852.04, 852.05, Requested standards have been incorporated. - Please remove Standards 840.22, 840.51, 840.52, 840.53. Unnecessary standards have been removed. - On the Index of Sheets, please revise the 2C-Special Details sheet numbers based upon the 2C-series comments below. Total number of 2C sheets has remained the same. - On Index of Sheets, please confirm M series (Metal Pole Sheets) is in the correct order. Currently, it follows the SIG (Signal Plan Sheets) plan set and index of sheets shows it follows SIGN set. Index revised to show M series following SIG series. - On General Notes, please include Std. No. 560.01 in the “Shoulder Construction” Standard. Standard Number 560.01 has been added.

- 2A series: Please include the Final Pavement Schedule on each 2A sheet. Final pavement schedule has been incorporated on bottom right of each sheet. - Sheet 2A-1: Please clarify why Typical Sections for -Y15- and -T16- show with guardrail but not included on Plan Sheet No. 4 nor on in the Guardrail Summary. There is no guardrail to be placed on either section, the dimension callouts have been removed from the typical sections to prevent confusion.

- 2C-series: Please include all NCDOT Curb Ramp Standards as shown on the 2018 Roadway Standard Drawings Connect site. Curb ramp details have been included for the relevant alternative curb ramp designs from the 2018 Roadway Standard Drawings Connect site, and they reference std. no 848.05 included on the list of standard drawings on sheet 1A. Unused details are not included to reduce confusion for Contractors. - Please include NCDOT Standards for 2’-9” curb and gutter as shown on 2018 Roadway Standard Drawings Connect Site. The 2018 Standard Drawings do not include 2’-9” Curb and Gutter, a detail on 2C-3 is present to provide guidance. - Please have all NCDOT Standards signed and approved by Standards Engineer, Nicole Hackler, PE. All standards shall be signed and approved by Nicole Hackler, PE.

- Sheet 3B-1: Please include -Y15- and -Y16- in the Guardrail Summary, if Typical Sections are correct. There is no guardrail to be placed on either section, the dimension callouts have been removed from the typical sections to prevent confusion.

- Sheet 3B-2 and Earthwork Balance Sheet: On Earthwork Balance Sheet, please include the Rock Excavation quantities in your calculations. Rock Excavation has been incorporated from updated recommendations. - Please include all station ranges from the Geotech Report that are considered Acceptable Unclassified Excavation. Station ranges have been incorporated below adjustments. - Please review Total Unclassified Excavation and Total Embankment amount in cross-sections to assure they correspond correctly to Earthwork Balance Sheet totals. Quantities have been reviewed and revised to correspond to balance sheet. - Since there is Unsuitable Unclassified Excavation, please include it to be unsuitable waste material as well. (See attachment) Unsuitable Unclassified material has been incorporated into the waster material. - Please revise “Selcet Granular Material” to “Select Granular Material”. Text has been revised. - On Earthwork Balance Sheet, please include the full statement “Acceptable Unclassified Excavation = 26000 C.Y. Not to be Used in Top 3’ of Embankment or Backfill”. Full statement has been included under adjustments. - Please revise note to say “Note: Earthwork quantities are calculated by Roadway Designer….” instead of Roadway Design Unit. Note revised. - On Sheet 3B-2 and Earthwork Balance Sheet, please remove rows where there are no volumes for these materials because this will help avoid any questions from others looking at the plans. Empty rows have been omitted. - Show earthwork quantities measured in cubic yards under the “Summary of Earthwork” table. All quantities shown under the table are in cubic yards.

- 3D series: Please rename “Berm Ditch Outlet” to “Berm Drainage Outlet” as shown in Roadway Standard Drawings 850.10. Berm Drainage Outlet is measured in SY. Please revise measurements from “per each” to SY. DSS updated. - On Sheet 3D-1, please include and revise information for Str. 0424 and 0426 since both are DI. Also include pipe length for the 15” RCP-III that connects Str. 0424 to 0426. DSS updated. - On Sheet 3D-2, please remove quantity from M.H. Std 840.51, Std. 840.52, or 840.53 since this is a junction box. DSS updated.

- Plan Sheets: Please label DDE quantities for each location. DDE quantities added at each location.

- Cross Section Sheets: On sheet X-1A, please show embankment column “does not include” undercut backfill. Embankment Column note has been updated. - On X-1A, please review the Total Unclassified Excavation and Total Embankment amount for each station range to assure they correspond to the Earthwork Balance Sheet. Quantities have been reviewed and revised to correspond to balance sheet.

- TMP Plans: On sheet TMP-1, please remove the “Type of Work” since that is part of the Roadway plans. Removed. - On TMP-1A, please verify if Std. No. 1101.06 should be included or removed. Std. Removed. - Please include the Intermediate Contract Time (ICT) document and ensure they match the Time and Holiday Restrictions found in General Notes. ICTs match General Notes and copies placed in SP folder. - SIGN Plans: Please place the M series (Metal Poles Sheets) in the correct order. Sheet M4 has been renamed on SharePoint to 260_315…. To follow sheet M3. - UC Plans: Please complete the review and seal and sign the Utility Construction plans when approved. UC sheets shall be signed and sealed upon approval. - Project Letters: Please provide an email or letter from Right of Way for the 200-series items…replace, demol., building, gate, fence, etc. Documentation is supposed to be on file, even if there appears to be no items. A letter for the 200 series items has been placed on SharePoint in the ROW discipline. Division ROW is continuing to address the outstanding items that need to be removed, so the list may be taken care of prior to construction. They are requesting a delay of availability for construction, except for tree clearing, until March 15, 2024. - Please respond to each review comment and return a copy of the responses. Provide corrections to the plans by Monday, August 28th. Note: Revisions to the Earthwork Summary, Drainage Summary, and PMP include quantities that will need to be updated in the PIQ.