# MEMORANDUM

| To: | Gary Jordan and Mike Wicker |
| --- | --- |
| From: | Cameron Long, P.E. |
| Date: | September 20, 2022 |
| Subject: | Culvert Replacement to Enhance Aquatic Connectivity Resiliency and Flood Capacity in the Black River Watershed |
| M&N Job No.: | 10292-01 |

Introduction

Moffatt & Nichol is under contract to provide preliminary design, survey data collection, final design, permitting and construction support services for the Cape Fear Resource Conservation & Development (CFRC&D) as part of the Enhancing Aquatic Connectivity Resiliency and Flood Capacity in the Black River Watershed project. The objective of the project is to increase access to historic migratory fish habitat and to enhance flooding resiliency in the Black River Watershed. This culvert replacement was selected from a group of 200 culverts in the watershed as one of the five most sever barriers to aquatic connectivity.

Summary of Objectives

- Aquatic Connectivity

To accomplish the first objective of the project in increasing access to historic migratory fish habitat the culvert invert for the two primary barrels is proposed to be lowered approximately 1.99 ft. on the upstream end and 1.35 ft. on the downstream end. This lowers the overall culvert elevation for the primary barrels and puts it in line with the channel slope both upstream and downstream of the culvert. The proposed culvert slope is 0.32% and the channel slope measured using QL2 LIDAR over 4000 ft. was 0.28%. In addition, the primary barrels are both buried 1.0 ft. to further facilitate aquatic passage.

One factor in the decision to have a configuration of 48”-72”-72”-48” with the two central 72” pipes being buried 1.0 ft. was to attempt to maintain a main channel width similar to the upstream and downstream natural channel widths. The two central 72” will serve as the main channel during normal flow and the two outer 48” barrels placed on the floodplain bench would only convey flow during storm events. With the current proposed configuration, the main channel at the culvert will measure 18 ft. If all four barrels were at the same elevation the proposed main channel width would be 33 ft. The natural main channel width is between 10 to 24 ft. on the downstream end. The upstream main channel width varies between 8 to 14 ft. and is smaller as the stream is braided upstream of the existing culvert. The secondary braided channel width varies between 5 to 8 ft. The decision to change the two outside barrels to smaller pipes that are placed on a floodplain bench was also done to mitigate potential concerns about over burial of the culvert raised by Craig Lee of the NCDOT Hydraulics Unit.

- Flooding Resiliency

In addition to improving the crossing for aquatic connectivity the project still had to meet the NCDOT and FEMA requirements for a replacement project. The effective FEMA base flood elevation upstream of the culvert is 21.31’. The corrected effective FEMA base flood elevation is 22.05’ which accounts for improved survey and topographical information near the crossing. The proposed structure decreases the base flood elevation to 21.16’ which is less than both the corrected effective and the effective models. There are no increases in the base flood elevations anywhere in the model and as such there are no insurable structures impacted by an increase in base flood elevations. The NCDOT requirements are that the 25-year storm event headwater elevation be less than or equal to the existing headwater elevation for the same event. The existing 25-year headwater elevation is 18.7’ while the proposed headwater elevation is 17.9’.