Greene County Hydrologic Analysis

Detailed and Limited Detail Study

The hydrologic analyses are based on the methods described in the US Geological Survey (USGS) Water Resources Investigations Report 01-4207, “Estimating the Magnitude and Frequency of Floods in Rural Basins of North Carolina.”1  Flood discharges were estimated using regression equations developed by USGS from stream gages in various regions of the North Carolina. The hydrologic region for Greene County is the Coastal Plains region of North Carolina.  The only variable in the equations is the drainage area of the basin in question. USGS Report 01-4207 also includes methods of adjustment of the regression equation discharges on streams with gages. The adjustments to streams with gages were applied based on these procedures. An adjustment factor was developed for application of the regression equations to certain un-gaged streams because the regression equations produce discharges less than the stream gage discharges in those areas.

Delineation of drainage basins and drainage areas was based on digital elevation models (DEM) produced from light detection and ranging (LIDAR) data supplied by the NCFMP.

The hydrologic analysis also includes a review and update of flood-flow frequency estimates for active stream gages within the basin to include the most current stream flow records available. The flood-flow frequency data was updated using methods described in USGS “Bulletin 17B guidelines for Determining Flood Flow Frequency.”  The updated information was utilized for adjustment of regression equation discharges.

Hydrologic analysis of urban areas was based on methods described in USGS Water Resources Investigations Report 96-4084, “Estimation of Flood-Frequency Characteristics of Small Urban Streams in North Carolina.”2 Flood discharges were estimated using regression equations developed by USGS from rainfall runoff models of urban watersheds throughout North Carolina. The variables in the equations include the drainage area and the percent of impervious area within the basin in question. Basins were considered urban when the impervious area exceeded 10 percent.  The urban regression equations were also used to calculate future conditions based on 10%, 20%, and 35% impervious cover.

Greene County Hydraulic Analysis

- Limited Detail Study

The step-backwater computer program U.S. Army Corps of Engineers Hydraulic Engineering Center River Analysis System, version 4.0.0 (HEC-RAS 4.0.0)3 was used as the hydraulic model for the limited detail studies.  Topographic data was derived from LiDAR mass points.  Relevant features of structures were field measured or verified.  Orthophotos were also used in creation of the HEC-RAS model.  1% annual chance of exceedance (100-year flood) elevations were modeled, as well as an approximate non-encroachment width.  The non-encroachment width was determined by running a Method 4 floodway encroachment model.  The 100-year flood elevation was used in conjunction with topographic data derived from LiDAR mass points to estimate the 100-year flood inundation.

The HEC-RAS models assumed subcritical flow with a downstream boundary condition.  The boundary condition was typically established using the normal depth determined from the channel invert slope.  If the study tied into another study downstream, the calculated water surface elevation was used as the boundary condition. Orthophotos were used to determine Manning’s n-values for each modeled cross section.

- Detail Study

10%, 2%, 1%, and 0.2% annual chance of exceedance flood elevations were modeled using HEC-RAS 4.0 for detail study streams.  Floodways were modeled using the Method 1 floodway encroachment.  Structures were field surveyed, as well as sample cross sections.  Cross sections were derived from a combination of field cross section surveys and LiDAR elevation data.  The survey of the channel geometry was used for the modeled channel and was blended with LiDAR mass points to make up the overbank portion of the cross section.

The boundary condition was typically established using the normal depth determined from the channel invert slope.  If the study tied into another study downstream, the calculated water surface elevation was used as the boundary condition. Orthophotos were used to determine Manning’s n-values for each modeled cross section.

References

1U.S. Department of the Interior, Geological Survey, Water Resources Investigations Report 01-4207, Estimating the Magnitude and Frequency of Floods in Rural Basins of North Carolina, Raleigh, North Carolina, 2001

Rural Regression Equation

Q10=72.9(DA0.697)

Q25=98.1(DA0.693)

Q50=120(DA0.691)

Q100=143(DA0.688)

Q500=210(DA0.684)

where,

Qx = Discharge with return period x

DA = Drainage area

2U.S. Department of the Interior, Geological Survey, Water Resources Investigations Report 96-4084, Estimation of Flood-Frequency Characteristics of Small Urban Streams in North Carolina, Raleigh, North Carolina, 1996

Urban Regression Equation

Q10=75.6(DA0.677)*(IA0.515)

Q25=128(DA0.644)*(IA0.436)

Q50=170(DA0.637)*(IA0.396)

Q100=217(DA0.63)*(IA0.358)

where,

Qx = Discharge with return period x

DA = Drainage area

IA = Impervious area

3U.S. Army Corps of Engineers, Hydrologic Engineering Center, HEC-RAS Version 4.0.0, March, 2008