| Table 9—Summary of Discharges |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Flooding Source | Location | Drainage Area (square miles) | Discharges (cfs) |  |  |  |
|  |  |  | 10% Annual Chance | 2% Annual Chance | 1% Annual Chance | 0.2% Annual Chance |
| Appletree Swamp | At the confluence with Nahunta Swamp | 8.9 | 720 | 1,300 | 1,620 | 2,520 |
|  | Approximately 1,710 feet upstream of Shingleton Road | 8.5 | 700 | 1,270 | 1,570 | 2,450 |
|  | Approximately 1.0 mile downstream of Apple Tree Road (SR 1215) | 7.5 | 650 | 1,180 | 1,470 | 2,290 |
|  | Approximately 1,850 feet downstream of Apple Tree Road (SR 1215) | 6.9 | 610 | 1,120 | 1,390 | 2,180 |
|  | Just upstream of Apple Tree Road (SR 1215) | 6.3 | 580 | 1,060 | 1,320 | 2,080 |
|  | Approximately 1,640 feet upstream of Apple Tree Road (SR 1215) | 5.9 | * | * | 1,270 | * |
|  | Approximately 0.5 mile upstream of Apple Tree Road (SR 1215) | 5.7 | * | * | 1,250 | * |
|  | Approximately 1,800 feet downstream of Pope Farm Rd | 5.3 | * | * | 1,200 | * |
|  | Approximately 370 feet upstream of Pope Farm Rd | 3.4 | * | * | 937 | * |
|  | Approximately 420 feet downstream of Greene County boundary | 3.0 | * | * | 878 | * |
| Appletree Swamp Tributary | At the confluence with Appletree Swamp | 1.5 | * | * | 587 | * |
| Bear Creek | Approximately 1,320 feet downstream of Parkstown Road | 13.3 | 920 | 1,640 | 2,020 | 3,120 |
|  | Approximately 0.7 mile upstream of Parkstown Road | 11.1 | 830 | 1,480 | 1,830 | 2,830 |
|  | Approximately 0.8 mile downstream of Watershed Road | 6.9 | 610 | 1,120 | 1,390 | 2,180 |
|  | Approximately 0.6 mile downstream of Watershed Road | 5.1 | 510 | 940 | 1,170 | 1,850 |
|  | Approximately 0.5 mile downstream of Watershed Road | 4.7 | 490 | 900 | 1,120 | 1,780 |
|  | Approximately 1,570 feet downstream of Oakdale Road (SR 1140) | 2.5 | 330 | 620 | 780 | 1,250 |
|  | Approximately 250 feet upstream of Oakdale Road (SR 1140) | 2.0 | 290 | 550 | 690 | 1,130 |
| Button Branch | At the confluence with Nahunta Swamp | 5.4 | * | * | 1,220 | * |
|  | Approximately 1,000 feet upstream of Saulston Rd | 4.6 | * | * | 1,110 | * |
|  | Approximately 0.8 mile upstream of Greene/Wayne County boundary | 2.9 | * | * | 849 | * |
| Contentnea Creek | Approximately 1.1 mile downstream of the confluence of Little Contentnea Creek | 980.0 | 12,400 | 19,400 | 22,800 | 31,900 |
|  | Just above the confluence of Little Contentnea Creek | 795.8 | 10,100 | 16,600 | 19,800 | 28,700 |
|  | Approximately 4.0 mile downstream from the confluence of Wheat Swamp | 785.2 | 9,800 | 16,000 | 19,120 | 27,620 |
|  | Approximately 2.6 mile downstream from the confluence of Wheat Swamp | 775.6 | 9,680 | 15,840 | 18,950 | 27,430 |
|  | Approximately 2.2 mile downstream from the confluence of Wheat Swamp | 775.1 | 9,680 | 15,830 | 18,940 | 27,420 |
|  | At the confluence of Wheat Swamp | 745.3 | 9,300 | 15,350 | 18,410 | 26,800 |
|  | Approximately 1,970 feet upstream from the confluence of Wheat Swamp | 745.2 | 9,300 | 15,350 | 18,410 | 26,800 |
|  | Approximately 0.9 mile upstream from the confluence of Wheat Swamp | 744.2 | 9,290 | 15,330 | 18,390 | 26,780 |
|  | Approximately 0.6 mile downstream from East Main Street/State Highway 123 | 732.0 | 9,150 | 15,140 | 18,180 | 26,520 |
| Contentnea Creek (continued) | Just upstream of East Main Street/State Highway 123 | 726.4 | 9,130 | 15,100 | 18,120 | 26,420 |
|  | At the confluence of Rainbow Creek | 711.5 | 9,080 | 14,980 | 17,960 | 26,150 |
|  | Approximately 1.1 mile upstream from the confluence of Rainbow Creek | 710.6 | 9,080 | 14,970 | 17,960 | 26,130 |
|  | Approximately 1.7 mile upstream from the confluence of Rainbow Creek | 709.9 | 9,080 | 14,970 | 17,950 | 26,120 |
|  | Approximately 2.4 mile upstream from the confluence of Rainbow Creek | 709.4 | 9,080 | 14,960 | 17,940 | 26,110 |
|  | Approximately 2.9 mile upstream from the confluence of Rainbow Creek | 708.2 | 9,070 | 14,950 | 17,930 | 26,090 |
|  | Approximately 3.7 mile downstream from the confluence of Poorhouse Run | 705.1 | 9,060 | 14,930 | 17,900 | 26,030 |
|  | Approximately 2.7 mile downstream from the confluence of Poorhouse Run | 703.0 | 9,060 | 14,910 | 17,870 | 25,990 |
|  | At the confluence of Poorhouse Run | 700.2 | 9,050 | 14,890 | 17,840 | 25,940 |
| Contentnea Creek (continued) | Approximately 0.5 mile upstream from the confluence of Poorhouse Run | 694.9 | 9,030 | 14,850 | 17,790 | 25,840 |
|  | At the confluence of Contentnea Creek Tributary 8 | 693.2 | 9,020 | 14,830 | 17,770 | 25,810 |
|  | At the confluence of Contentnea Creek Tributary 9 | 690.5 | 9,010 | 14,810 | 17,740 | 25,760 |
|  | Approximately 0.5 mile upstream from the confluence of Contentnea Creek Tributary 9 | 689.4 | 9,010 | 14,800 | 17,730 | 25,740 |
|  | At the confluence of Tyson Marsh | 672.1 | 8,950 | 14,660 | 17,540 | 25,410 |
|  | Approximately 1.3 mile upstream from the confluence of Tyson Marsh | 669.3 | 8,940 | 14,630 | 17,510 | 25,360 |
|  | Approximately 2.7 mile downstream from the confluence of Fort Run | 666.8 | 8,930 | 14,610 | 17,480 | 25,310 |
|  | Approximately 1.4 mile downstream from the confluence of Fort Run | 666.0 | 8,920 | 14,600 | 17,470 | 25,300 |
|  | Approximately 0.7 mile downstream from the confluence of Fort Run | 664.7 | 8,920 | 14,590 | 17,450 | 25,270 |
| Contentnea Creek (continued) | At the confluence of Fort Run | 653.7 | 8,880 | 14,500 | 17,330 | 25,060 |
|  | Approximately 1,350 feet upstream from the confluence of Fort Run | 651.6 | 8,870 | 14,480 | 17,310 | 25,020 |
|  | Approximately 450 feet upstream from Sheppard Ferry Road (SR 1222) | 651.5 | 8,870 | 14,480 | 17,300 | 25,020 |
|  | Approximately 0.9 mile upstream from Sheppard Ferry Road (SR 1222) | 650.2 | 8,860 | 14,470 | 17,290 | 24,990 |
|  | At the confluence of Nahunta Swamp | 550.6 | 8,390 | 13,500 | 16,060 | 22,970 |
|  | Approximately 1.5 mile upstream from the confluence of Nahunta Swamp | 549.8 | 8,390 | 13,500 | 16,050 | 22,960 |
|  | Approximately 2.5 mile upstream from the confluence of Nahunta Swamp | 547.6 | 8,380 | 13,470 | 16,020 | 22,910 |
|  | At the confluence of Beaman Run | 539.0 | 8,330 | 13,380 | 15,900 | 22,720 |
|  | Approximately 1.7 mile downstream from Speights Bridge Road | 535.5 | 8,310 | 13,340 | 15,860 | 22,650 |
|  | Approximately 1.5 mile downstream from Speights Bridge Road | 533.5 | 8,300 | 13,320 | 15,830 | 22,610 |
| Contentnea Creek (continued) | Approximately 620 feet upstream from Speights Bridge Road | 532.0 | 8,290 | 13,300 | 15,810 | 22,570 |
|  | Approximately 0.5 mile upstream from Speights Bridge Road | 531.5 | 8,290 | 13,300 | 15,800 | 22,560 |
|  | Approximately 2.3 mile upstream from Speights Bridge Road | 530.7 | 8,280 | 13,290 | 15,790 | 22,550 |
|  | Approximately 2.5 mile downstream from the confluence of Watery Branch | 523.5 | 8,240 | 13,210 | 15,690 | 22,390 |
|  | Approximately 1.1 mile downstream from the confluence of Watery Branch | 522.6 | 8,230 | 13,200 | 15,680 | 22,370 |
|  | At the confluence of Watery Branch | 515.7 | 8,190 | 13,120 | 15,580 | 22,220 |
| Contentnea Creek Tributary 8 | At the confluence with Contentnea Creek | 1.2 | 210 | 410 | 520 | 850 |
| Contentnea Creek Tributary 9 | At the confluence with Contentnea Creek | 1.6 | 260 | 490 | 620 | 1,010 |
| Cow Branch | At the confluence with Nahunta Swamp | 4.3 | 460 | 850 | 1,070 | 1,690 |
|  | Approximately 0.4 mile upstream from Fort Run Road (SR 1058) | 3.8 | 430 | 800 | 1,000 | 1,590 |
| Cow Branch (continued) | Approximately 400 feet downstream from Cow Branch Road | 2.9 | 360 | 680 | 850 | 1,370 |
|  | Approximately 0.5 mile upstream from Cow Branch Road | 1.9 | 280 | 530 | 670 | 1,090 |
|  | Approximately 1.2 mile upstream from Cow Branch Road | 1.0 | 190 | 370 | 470 | 770 |
| Fort Run | At the confluence with Contentnea Creek | 10.8 | * | * | 1,800 | * |
|  | Approximately 1,800 feet upstream of State Hwy 58 | 9.2 | * | * | 1,650 | * |
|  | Approximately 0.5 mile downstream of Saulston Rd | 8.2 | * | * | 1,540 | * |
|  | Approximately 2,060 feet upstream of Saulston Rd | 7.7 | * | * | 1,490 | * |
|  | At the confluence with Lewis Branch | 5.5 | * | * | 1,220 | * |
|  | Approximately 0.6 mile upstream of the confluence with Lewis Branch | 5.1 | * | * | 1,170 | * |
|  | Approximately 1.0 mile upstream of the confluence with Lewis Branch | 3.6 | * | * | 960 | * |
| Fort Run (continued) | Approximately 1,270 feet downstream of Gurganus Road (SR 1204) | 3.2 | * | * | 904 | * |
|  | Approximately 0.07 mile upstream of Gurganus Road (SR 1204) | 2.2 | * | * | 733 | * |
| Jacks Fork | At the confluence with Tyson Marsh | 3.6 | * | * | 971 | * |
|  | Approximately 1.08 mile upstream of the confluence with Tyson Marsh | 2.6 | * | * | 809 | * |
| Lewis Branch | At the confluence with Fort Run | 1.8 | * | * | 652 | * |
|  | The confluence with Contentnea Creek | 184.1 | 4,648 | 7,520 | 8,960 | 12,856 |
|  | Approximately 1.0 mile upstream of the confluence with Contentnea Creek | 182.7 | 4,625 | 7,485 | 8,920 | 12,800 |
|  | Approximately 0.7 mile downstream of NC Highway 903 | 178.8 | 4,565 | 7,393 | 8,813 | 12,655 |
| Little Contentnea Creek | Approximately 0.3 mile upstream of NC Highway 903 | 170.8 | 4,438 | 7,200 | 8,588 | 12,346 |
|  | Approximately 5.6 miles downstream of Waterside Church Road | 161.9 | 4,294 | 6,980 | 8,332 | 11,996 |
|  | Approximately 1.9 miles downstream of Waterside Church Road | 153.6 | 4,156 | 6,769 | 8,085 | 11,657 |
|  | Approximately 0.6 mile downstream of Waterside Church Road | 96.7 | 3,127 | 5,178 | 6,222 | 9,083 |
|  | Approximately 1,290 feet downstream of U.S. Highway 264 | 20.3 | 1,116 | 1,882 | 2,273 | 3,183 |
|  | Approximately 0.8 mile upstream of Bell Road (SR 1311) | 17.4 | 1,015 | 1,717 | 2,076 | 2,911 |
| Little Contentnea Creek (continued) | Approximately 1.4 miles upstream of Bell Road (SR 1311) | 16.0 | 963 | 1,632 | 1,974 | 2,771 |
|  | Approximately 1,120 feet downstream of Moseley Road | 10.0 | 723 | 1,234 | 1,497 | 2,112 |
|  | Approximately 0.7 mile upstream of Moseley Road | 7.6 | 610 | 1,046 | 1,271 | 1,799 |
|  | Approximately 0.8 mile upstream of Moseley Road | 5.8 | 514 | 885 | 1,077 | 1,529 |
|  | At the confluence with Little Contentnea Creek | 55.9 | 2,083 | 3,454 | 4,147 | 5,742 |
|  | Approximately 0.5 mile upstream of the confluence with Little Contentnea Creek | 54.3 | 2,047 | 3,396 | 4,078 | 5,648 |
| Middle Swamp | Approximately 1,260 feet downstream of Darden Farm Road | 51.3 | 1,975 | 3,281 | 3,940 | 5,461 |
|  | Approximately 0.9 mile downstream of Darden Farm Road | 19.7 | 1,094 | 1,847 | 2,231 | 3,124 |
|  | Approximately 1.0 mile upstream of U.S. Highway 13 | 17.0 | 999 | 1,691 | 2,044 | 2,868 |
|  | Approximately 1.4 miles upstream of U.S. Highway 13 | 15.0 | 923 | 1,566 | 1,895 | 2,662 |
|  | Approximately 700 feet upstream of U.S. Highway 258 | 11.6 | 791 | 1,348 | 1,633 | 2,301 |
|  | Approximately 0.8 mile upstream of US Highway 258 | 8.0 | * | * | 1,519 | * |
| Middle Swamp (continued) | Approximately 6.2 miles upstream of US Highway 258 | 7.0 | * | * | 1,410 | * |
|  | Approximately 1,430 feet downstream of U.S. Highway 264 | 31.8 | 1,578 | 2,720 | 3,316 | 4,988 |
|  | At the confluence with Contentnea Creek | 98.7 | 3,200 | 5,560 | 6,820 | 10,430 |
|  | Approximately 1,390 feet upstream of the confluence with Contentnea Creek | 98.0 | 3,180 | 5,550 | 6,810 | 10,430 |
|  | Approximately 1,060 feet upstream of State Highway 58 | 97.1 | 3,170 | 5,530 | 6,800 | 10,430 |
| Nahunta Swamp | Approximately 0.7 mile downstream of the confluence of Appletree Swamp | 96.2 | 3,150 | 5,520 | 6,780 | 10,430 |
|  | At the confluence of Appletree Swamp | 86.2 | 2,960 | 5,310 | 6,600 | 10,360 |
|  | At the confluence of Cow Branch | 80.9 | 2,850 | 5,200 | 6,490 | 10,310 |
|  | Just downstream of Bullhead Road | 77.7 | 2,840 | 5,160 | 6,420 | 10,160 |
|  | Approximately 1,100 feet upstream of Bullhead Road | 77.1 | 2,830 | 5,120 | 6,380 | 10,080 |
|  | Approximately 0.5 mile upstream of Bullhead Road | 76.4 | 2,810 | 5,090 | 6,330 | 10,000 |
|  | Approximately 1.5 mile upstream of Bullhead Road | 73.8 | 2,740 | 4,940 | 6,140 | 9,670 |
| Poorhouse Run | At the confluence with Contentnea Creek | 1.7 | 550 | 1,010 | 1,180 | 1,710 |
|  | At Kingold Boulevard | 1.2 | 210 | 410 | 530 | 870 |
|  | At the confluence with Contentnea Creek | 14.9 | 990 | 1,750 | 2,160 | 3,310 |
|  | Approximately 0.5 mile downstream of Shady Grove Church Road | 14.3 | 970 | 1,720 | 2,110 | 3,250 |
|  | Approximately 1,930 feet upstream of Shady Grove Church Road | 13.4 | 930 | 1,650 | 2,040 | 3,140 |
| Rainbow Creek | Approximately 0.7 mile downstream of State Highway 58 | 12.6 | 890 | 1,590 | 1,960 | 3,020 |
|  | Approximately 0.5 mile downstream of State Highway 58 | 11.6 | 850 | 1,510 | 1,870 | 2,890 |
|  | Approximately 360 feet upstream of State Highway 58 | 9.3 | 740 | 1,330 | 1,650 | 2,570 |
|  | Approximately 0.5 mile upstream of State Highway 58 | 7.9 | 670 | 1,210 | 1,510 | 2,350 |
|  | Approximately 0.7 mile downstream of Lloyd Harrison Road (SR 1101) | 7.4 | 640 | 1,170 | 1,450 | 2,270 |
| Rainbow Creek (continued) | Approximately 1,230 feet upstream of Lloyd Harrison Road (SR 1101) | 6.0 | 570 | 1,040 | 1,300 | 2,040 |
|  | Approximately 0.5 mile upstream of the confluence with Sowell Run | 5.3 | * | * | 1,200 | * |
|  | Approximately 530 feet upstream of U.S. Hwy 258 | 4.9 | * | * | 1,150 | * |
|  | Approximately 1,690 feet upstream of U.S. Hwy 258 | 3.4 | * | * | 929 | * |
| Reeders Fork | At the confluence with Tyson Marsh | 1.7 | * | * | 628 | * |
|  | Approximately 1,370 feet upstream of Grant Sand Hole Rd | 1.4 | * | * | 572 | * |
|  | At the confluence with Middle Swamp | 31.1 | * | * | 3,280 | * |
|  | Approximately 1,370 feet upstream of the confluence with Middle Swamp | 30.4 | * | * | 3,230 | * |
|  | Approximately 1.3 mile upstream of the confluence with Middle Swamp | 26.3 | * | * | 2,980 | * |
|  | Approximately 1.6 mile upstream of the confluence with Middle Swamp | 25.6 | * | * | 2,930 | * |
| Sandy Run | At the confluence with Middle Swamp | 31.1 | * | * | 3,234 | * |
|  | Approximately 0.4 mile upstream of the confluence with Middle Swamp | 30.4 | * | * | 2,980 | * |
|  | Approximately 1.8 miles upstream of the confluence with Middle Swamp | 25.6 | * | * | 2,930 | * |
|  | Approximately 0.9 mile downstream of U.S. Hwy 13 | 23.9 | * | * | 2,820 | * |
|  | Approximately 0.6 mile downstream of U.S. Hwy 13 | 20.0 | * | * | 2,550 | * |
|  | Approximately 0.5 mile downstream of U.S. Hwy 13 | 15.7 | * | * | 2,220 | * |
| Sandy Run (continued) | Approximately 1,440 feet upstream of U.S. Hwy 13 | 14.2 | * | * | 2,100 | * |
|  | Approximately 1.2 mile upstream of U.S. Hwy 13 | 10.3 | * | * | 1,750 | * |
|  | Approximately 1.5 mile upstream of U.S. Hwy 13 | 9.6 | * | * | 1,680 | * |
|  | Approximately 1,630 feet downstream of Friendship Church Rd | 8.8 | * | * | 1,610 | * |
|  | Approximately 670 feet upstream of Friendship Church Rd | 8.7 | * | * | 1,590 | * |
|  | Approximately 0.8 mile upstream of Friendship Church Rd | 6.4 | * | * | 1,330 | * |
| Sandy Run (continued) | Approximately 1.1 mile upstream of Friendship Church Rd | 1.9 | * | * | 674 | * |
| Toisnot Swamp | At the confluence of Contentnea Creek | 126.7 | 8,240 | 13,030 | 15,360 | 21,710 |
|  | At the confluence with Contentnea Creek | 17.2 | * | * | 2,340 | * |
|  | Approximately 1,370 feet upstream of U.S. Hwy 13 | 15.0 | * | * | 2,160 | * |
|  | Approximately 1.4 mile upstream of U.S. Hwy 13 | 14.0 | * | * | 2,080 | * |
|  | Approximately 1.8 mile upstream of U.S. Hwy 13 | 12.5 | * | * | 1,950 | * |
| Tyson Marsh | At the confluence with Reeders Fork | 10. 6 | * | * | 1,780 | * |
|  | Approximately 950 feet upstream of the confluence with Reeders Fork | 6.8 | * | * | 1,390 | * |
|  | At the confluence with Jacks Fork | 3.1 | * | * | 884 | * |
|  | Approximately 0.4 mile upstream of the confluence with Jacks Fork | 2.9 | * | * | 850 | * |
|  | Approximately 1,270 feet upstream of Gray Turnage Rd | 2.0 | * | * | 691 | * |
| Watery Branch | Approximately 0.6 mile upstream of the confluence with Contentnea Creek | 6.0 | * | * | 2,286 | * |
|  | Approximately 2,010 feet upstream of State Hwy 58 | 5.7 | * | * | 2,218 | * |
|  | Approximately 0.9 mile upstream of State Hwy 58 | 4.7 | * | * | 1,968 | * |
|  | Approximately 1.1 mile upstream of State Hwy 58 | 4.5 | * | * | 1,900 | * |
|  | At the confluence with Contentnea Creek | 28.9 | * | * | 3,140 | * |
|  | At Shady Grove Church Road | 27.2 | * | * | 3,040 | * |
|  | Approximately 0.6 mile downstream of Oats Road | 24.9 | * | * | 2,890 | * |
| Wheat Swamp | Approximately 790 feet downstream of Oats Road | 20.5 | * | * | 2,580 | * |
|  | Approximately 1,370 feet upstream of Oats Road | 18.7 | * | * | 2,450 | * |
|  | Approximately 0.5mile upstream of Stocks Rd | 15.7 | * | * | 2,220 | * |
|  | Approximately 1.1 mile upstream of Stocks Rd | 13.7 | * | * | 2,060 | * |
| Wheat Swamp (continued) | Approximately 1,640 feet upstream of State Hwy 58 | 10.6 | * | * | 1,780 | * |
|  | Approximately 1,640 feet upstream of Sugg Rd | 8.9 | * | * | 1,610 | * |
|  | Approximately 0.6 mile upstream of Sugg Rd | 7.6 | * | * | 1,470 | * |
|  | Approximately 0.9 mile downstream of U.S. Hwy 258 | 5.6 | * | * | 1,240 | * |
|  | Approximately 1,950 feet downstream of U.S. Hwy 258 | 4.9 | * | * | 1,150 | * |

*Data not available