# NORTH CAROLINA DEPARTMENT OF TRANSPORTATION

# FIELD SCOPING MEETING WORKSHEET

Design Build Process

| WBS No.: 17BP.6.R.44 | #### Field Scoping Meeting Date: July 22, 2014 |
| --- | --- |
| Division: 6 | #### Location: On-site |
| Route (US/NC/SR): NC-410 | County: COLUMBUS |
| Project Description: Bridge No. 230013 over Beaver Dam Swamp on NC-410 (Joe Brown Hwy S) |  |
| Tier: Regional |  |
| #### Municipality: No |  |

| Attendees |  |  |  |
| --- | --- | --- | --- |
|  | Name | Phone No. | E-mail |
| Division Bridge Manager | Brice Bell | 910-486-1493 | bricebell@ncdot.gov |
| Division Environmental Off. | Jim Rerko | 910-437-0207 | jjrerko@ncdot.gov |
| Hydraulics Engineer | Jerry Lindsey | 919-707-6734 | jlindsey@ncdot.gov |
| Hydraulics Engineer | Jon Moore | 919-707-6738 | Jlmoore6@ncdot.gov |
| TPMU | Pam Williams | 919-707-6608 | prwilliams@ncdot.gov |
| TPMU | Rachel Evans | 919-707-6609 | rcevans@ncdot.gov |
| TPMU | Jamie Lancaster | 919-707-6620 | jlancaster@ncdot.gov |
| Bridge Management | Paul Garrett | 919-707-6457 | pgarrett@ncdot.gov |
| Resident Engineer | Rusty Marsh | 910-642-2489 | rmarsh@ncdot.gov |
| Asst. Resident Engineer | Terry Church | 910-642-2489 | tachurch@ncdot.gov |
| Div. 6 Locating Engineer | John Kaukola | 910-486-1338 | jkaukola@ncdot.gov |
| Div. 6 Maintenance Engineer | Ken Murphy | 910-486-1493 | rkmurphy@ncdot.gov |
| Div. 6 Bridge Engineer | Darren Pittman | 910-829-6345 | dpittman@ncdot.gov |
| Div. 6 Bridge Maint. Supervisor | Barry Gelezinsky | 910-642-4388 | bwgelezinsky@ncdot.gov |
| Division 6 Bridge Maint. | Kevin Worley | 910-642-4388 | kjworley@ncdot.gov |
| Div. 6 Dist. 3 Engineer | Drew Cox | 910-642-3760 | hlcox@ncdot.gov |
| DENR-DWR | Mason Herndon | 910-308-4021 | Mason.herndon@ncdenr.gov |
| USACE | Liz Hair | 910-251-4049 | sarah.e.hair@usace.army.mil |

Existing Features

| Feature Bridged: 230013 over Beaver Dam Swamp on NC-410 (Joe Brown Hwy S) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ex. Bridge Clear Deck Width: 24 (Ft.) |  |  | Roadway width: 20 (Ft.) |  |  |  |  |  |
| (Bridge / Culvert) Length: 41 (Ft.) |  |  | Deck Width (Out To Out): 25.33(Ft.) |  |  |  |  |  |
| Water Depth: 7(Ft.) |  |  | Height Bed-To-Crown: 14(Ft.) |  |  |  |  |  |
| Year Built: 1948 |  |  | Posted: SV NP |  |  |  | TTST: NP |  |
| Superstructure: ST M-BM |  | Abutment: TM |  |  |  | Pier Type: TM |  |  |
| Temp. Shored: Yes |  |  |  |  |  |  |  |  |
| Historic High Water (Elev. To The Existing Structure ): (Ft) |  |  |  |  |  |  |  |  |
| PRI: 72.2 DP: SR: 16.5 |  |  |  | School Bussing crossings per day: 2 buses/4 trips |  |  |  |  |
| Posted Speed Limit Vicinity: 55 (mph) |  |  |  | Existing Road Surface: Paved |  |  |  |  |
| Detour Off-Site | Stage Construction |  |  | New Alignment | On-site |  |  |  |
| If Off-Site, Description Of Detour Route: SR 1004, SR 1005, SR 1324 |  |  |  |  |  |  |  |
| Approximate Length Of Detour? 4.5 ( Miles ) |  |  |  |  |  |  |  |
| Improvements Needed To Road(s) On Detour? No |  |  |  |  |  |  |  |
| Div. Traffic Eng. Recommend Off-site detour signing? Trailblazing (with St./Rd. names) |  |  |  |  |  |  |  |
| Improvements Needed To Bridges On Detour? No |  |  |  |  |  |  |  |
| Are Bridges On Detour Currently Programmed? No |  |  |  |  |  |  |  |
| Are There EMS Or Business Access Issues? Length of Detour route would increase response times for fire, EMS and law enforcement |  |  |  |  |  |  |  |
| Are There Any Railroad Crossings On Detour? No |  |  |  |  |  |  |  |
| Should Work Zone Pedestrian Access Be Maintained During Construction? No |  |  |  |  |  |  |  |
| Overhead Utility Lines In Conflict |  |  |  |  |  |  |  |
| Power Transmission Lines In Conflict |  |  |  |  |  |  |  |
| Telephone In Conflict |  |  |  |  |  |  |  |
| Cable Lines In Conflict Time Warner |  |  |  |  |  |  |  |
| Fiber Optic In Conflict |  |  |  |  |  |  |  |
| Water In Conflict No information, possible |  |  |  |  |  |  |  |
| Sewer In Conflict |  |  |  |  |  |  |  |
| Natural Gas In Conflict |  |  |  |  |  |  |  |
| Other In Conflict |  |  |  |  |  |  |  |
| Based On The Past History Near This Project Site, What Is The Estimated Time Required To Complete Utility Adjustments 3 Months |  |  |  |  |  |  |  |
| Is There Any Future Utility Construction Anticipated In The Project Area No |  |  |  |  |  |  |  |
| Is A FEMA Buy-Out Property Being Impacted Yes; Avoid Columbus Cty Property in the NE quad |  |  |  |  |  |  |  |

Environmental

| Wetlands At Site Yes |  |  |  | Comments: NES delineated |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Endangered Species In County: American alligator, Atlantic sturgeon, Bald Eagle (BGPA), Red-cockaded woodpecker, Shortnose sturgeon, Waccamaw silverside, Wood stork, Cooley’s meadowrue & Rough-leaved loosestrife. |  |  |  |  |  |  |  |
| T&E species effect: Waccamaw silverside – No effect |  |  |  |  |  |  |  |
| Trout County : No |  |  | TVA County: No |  |  |  |  |
| CAMA County: No |  |  | Primary Nursery Area: No |  |  |  |  |
| Moratoria: No |  | Which species: |  |  |  | Duration : |  |
| Permits: |  |  |  |  |  |  |  |
| Water Quality Classification: C; Sw |  |  |  |  |  |  | 303d: No |
| Coast Guard Permit? No | Drainage Basin: Lumber |  |  |  | Riparian Buffer Rules: No |  |  |
| Is The Project Site In Or Near Any Of The Following: |  |  |  |  |  |  |  |
| National Forest: No |  |  |  | Wildlife Refuge: No |  |  |  |
| State, County, Or Local Park: No |  |  |  | Wild And Scenic River: No |  |  |  |
| Airport: No |  |  |  | Recreation Of Power Generation: No |  |  |  |
| Water Supply Reservoir: No |  |  |  | Nutrient Sensitive Waters: No |  |  |  |
| Public Use Boat Ramp: No |  |  |  | Cemeteries: No |  |  |  |
| Known Or Potential Historic Properties In The Area: |  |  |  |  |  |  |  |
| Architecture: No Survey Required |  |  |  | Archaeology: No Survey Required |  |  |  |
| Is The Bridge Structure Itself, Or Any Part Thereof, Considered Historic: No |  |  |  |  |  |  |  |
| Impacts to a Church, Community Center, Or Other Public Facility? No |  |  |  |  |  |  |  |
| Is this a Statewide Bicycle Route or a Local Non-Marked Bicycle Route: No |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |

Geotechnical

Are There Any Historical And/Or Vibration Sensitive Structures Near By Comments:

Are There Any Known Landfills And / Or Geo-environmental Hazard Sites At Or Within Close Proximity To The Project Site Comments:

Are Any Impacts Anticipated To Natural Springs Or Artesian Wells: Comments:

Possible Foundation Type: @ End Bents @ Interior

Division prefers H piles or pipe piles and no concrete piles.

Hydraulics

| FEMA Approval? Limited DS (MOA) |  |  | State Stormwater Permit? No |  |  |
| --- | --- | --- | --- | --- | --- |
| Is There Unusual Scour Potential? No |  |  | Is Protection Needed? Standard |  |  |
| Are Banks Stable? Yes |  |  | Is Protection Needed? Standard |  |  |
| Appreciable Amount Of Large Debris? Moderate |  |  |  |  |  |
| Placement Of Bents In The Water Be Allowed: Yes |  |  |  | Where: 5’ from Interior Bent on North side |  |
| Superstructure Type: Bridge |  | If Bridge: CS |  |  | If girder; why? |
| Length Of Structure: 105’ | Min. Number Of Spans: 2 |  |  |  | Span Arrangement: 1@55, 1 @ 50 |
| Waive offset: No. Shall be as close to 10’ setback as practicable. | Cap: 2’6” |  |  |  | Skewed: 90 |

Items To Be Discussed / Resolved At FSM By Attendees

| Off-Site Detour | Stage: |  |  |  |  | New Alignment: |  |  | On-site detour |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environmental Document Prepared by DBE: Low Impact Data Spreadsheet Minimum Criteria Checklist |  |  |  |  |  |  |  |  |  |
| ADT: 4400 per SIR |  | NHS: No |  | Tier: Regional |  |  | Func. Class: Major Collector, Rural |  |  |
| Bridge Min. Clear Deck Width Recommended by Div.: 40’ CRW, 42’ OTO with 42” Vertical Face Rail |  |  |  |  |  |  |  |  |  |
| Roadway lane width 12’ with 8’ shoulders including 4’ paved shoulders |  |  |  |  |  |  |  |  |  |
| Extension of Paved shoulders: end of construction limits and blunt end. |  |  |  |  |  |  |  |  |  |
| Pave to face of guardrail and taper 8:1 to travel lane/paved shoulder? Existing guardrail extends 220’ north and 220’ south. |  |  |  |  |  |  |  |  |  |
| Min. Pavement Surface Course: 3” inches: |  |  |  |  | Existing Roadway: 20 |  |  |  |  |
| Pavement Marking: Thermoplastic |  |  |  |  | Pavement markers: Raised |  |  |  |  |
| Will Railroad Involvement Be Required: No |  |  |  |  | Method of Clearing: II modified (hand clearing from ss out 5’) |  |  |  |  |
| Truck Percentage: : 7% per BIR |  |  |  |  | Reasonable Safe Speed per SIR: 55 mph |  |  |  |  |
| Liquidated damages: will be imposed after 135 days of closure/construction. Amt: $ _500___ per day |  |  |  |  |  |  |  |  |  |
| CEI by? NCDOT |  |  | R/W by? DBT |  |  |  |  | R/W monuments? concrete monuments |  |
| Salvageable materials: Bridge Approach rail (Type III) and transition pieces |  |  |  |  |  |  |  |  |  |
| Contact Person/number: Barry Gelezinsky / 910-642-4388 |  |  |  |  |  |  |  |  |  |
| Deliver to: 31 Airport Road, Whiteville, NC , contractor is to provide a weeks’ notice to the Bridge Maintenance Supervisor prior to delivery |  |  |  |  |  |  |  |  |  |
| Removal of previous structure remnant abutments/piers: No |  |  |  |  |  |  |  |  |  |
| Sequencing priority in construction? Construction from Labor Day to Memorial Day (No construction during summer months). |  |  |  |  |  |  |  |  |  |
| Comments: Division recommends asphalt overlay on bridge. Division requests new pavement a minimum of 50 feet from both ends of bridge and approach slabs perpendicular to roadway. Approach work to extend 30’ past the existing limits of existing guardrail at both ends. |  |  |  |  |  |  |  |  |  |