FINAL SURVEY REPORT									 January 8, 2016

PROJECT No.      17BP.11.R.119

TIP No.                 960072

COUNTY:             WILKES

ROUTE:               SR 1320

LS ID No.:

DESCRIPTION:    BRIDGE NO. 72 OVER TUCKER HOLE CREEK ON SR 1320 (CONGO ROAD)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT GPS MONUMENT

"960072-2" WITH A NORTHING OF 884721.625’ AND AN EASTING OF 1352513.712’.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99995154.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID G12A) AND BASED ON

NCDOT GPS "960072-2" (ELEV 1093.63’)

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (BL-4) and runs northerly along SR 1320 to station 10+69.79 (960072-2).

There is no baseline BY alignment.

The existing centerlines were calculated but not staked for Line EL SR 1320.

Elevations were obtained for each baseline control point and two benchmarks were established.

Baseline control data transmitted in the graphics FS file and the baseline text file.

=========================================================================================

PLANIMETRIC DATA:

The planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 55 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

Underground utilities were encountered within the project limits and are noted as shown in the FS file.  CenturyLink appears to be the owner of telephone and was contacted to receive information.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes and sanitary sewer were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The apparent utility pole owner is Duke Power.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A Right-of-Way abstract was provided for this project dated January 19, 2016 and received by TGS January 25, 2016 via email.  Existing right of way is shown by maintenance limits and mapped at 9’ each side of existing centerline as computed by TGS Engineers.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word SitePhoto.doc

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 960072_ls_fs.dgn

* 960072_ls_dtl.dgn

* 960072_ls_tnl.tin

* 960072_ls_gpk.gpk

* 960072_ls_Deeds.pdf

* 960072_ls_sr.doc

PROJECT SITE PHOTOGRAPHS

Looking East                                                                      Looking West

Looking North (Upstream)                                                 Looking South (Downstream)

South Elevation                                                                   North Elevation