FINAL SURVEY REPORT									 January 11, 2016

PROJECT No.

TIP No.                B-5385

COUNTY:             CALDWELL

ROUTE:               SR 1356 (OLD JOHN’S RIVER RD)

LS ID No.:            b5385-con-tgs

DESCRIPTION:    REPLACE BRIDGE NO. 130123 OVER ELIJAH ESTES MILL CREEK ON SR 1356

(OLD JOHN’S RIVER RD)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT GPS MONUMENT

"B5385-2" WITH A NORTHING OF 816272.2191' AND AN EASTING OF 1199377.5486'.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99989903.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID G12NC) AND BASED ON

NCDOT GPS "B5385-2" (ELEV 1162.35')

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00.00 (BL-3) and runs easterly along SR 1356 to station 14+78.22 (B5385-2).

There is no baseline BY alignment.

The existing centerlines were calculated but not staked for Line EL SR 1356.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file and the GPK file.

=========================================================================================

PLANIMETRIC DATA:

The shell planimetric mapping was supplied by photogrammetry with supplemental surveys and field

classification by TGS Engineers.  Data merged into the graphics FS file.

=========================================================================================

DTM DATA:

The initial DTM data was supplied by photogrammetry with supplemental and obscured DTM’s collected and

Compiled by TGS Engineers.  Data merged into the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 55 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

Underground utilities were encountered within the project limits and are noted as shown in the FS file.  AT&T (SBT&T) telephone poles are present near the bridge. Danny Little with Telics (704-764-7343) confirmed a buried 50 pair copper cable shown level “C” in the FS file.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The apparent utility pole owners are Blue Ridge Electric Membership Corp. and AT&T (SBT&T).

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

The project Propcon database was created and property owner contacts were made by TGS Engineers.

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the B5385_ls_Deed&PlatBook_160111.pdf file.  Property lines were established based upon surveyed and located field evidence and recorded documents.

Existing right of way lines were mapped based on the following:

A Right of Way Abstract was provided on January 27th, 2016 from Division 11 Right of Way.  Maintenance limits are mapped at 18’ Total (9’ left and right of centerline) as shown in the FS file.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in this Microsoft Word document.

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* B5385_ls_fs_160111.dgn

* B5385_ls_dtl_160111.dgn

* B5385_ls_tnl_160111.tin

* B5385_ls_gpk_160111.gpk

* B5385_ls_deed&platbook_160111.pdf

* B5385_ls_sr_160111.docx

* B5385_ls_poc_160111.mdb

PROJECT SITE PHOTOGRAPHS

North Elevation					  Looking South

Looking West						Looking East

South Elevation					  Looking North