===========================================================================

PROJECT NUMBER: 17BP.9.R.71	                     DAVIE COUNTY                             TIP NO: 29-0078

DESC: BRIDGE #78 OVER HUNTING CREEK ON SR 1338 COUNTY LINE ROAD

DATE OF SURVEY: NOVEMBER 21, 2016

LAST UPDATE: DECEMBER 16, 2016

===========================================================================

SPECIAL NOTES:

A THIS IS AN ENGLISH PROJECT

B ALL BASELINE POINTS ARE 18" REBAR WITH CAP (UNLESS OTHERWISE NOTED)

C PROJECT MAPPING SCALE 1” = 50’

D ALL BASELINE POINTS ARE SET TO BENCHMARK SPECIFICATION

(UNLESS OTHERWISE NOTED)

===========================================================================

PERSONNEL:

DEWBERRY

9300 HARRIS CORNERS PARKWAY

SUITE 220

CHARLOTTE, NC 28269-3797

(704) 509-9918

TIMOTHY A. RUDOLPH, PLS		CHIEF OF SURVEYS

TERRY DENNIS, PLS			ASSISTANT CHIEF OF SURVEYS

RICHARD MCCANNELL, PLS		CAD TECH

GAVIN PROFFIT, LSIT			CAD TECH

DAREN STOKER				PARTY CHIEF

CHARLES SCHWAB			PARTY CHIEF

TYLER MCCANNELL			INSTRUMENT OPERATOR

SHANE CHAMBERS			INSTRUMENT OPERATOR

===========================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT

IS BASED ON STATE PLANE COORDINATES ESTABLISHED BY

NCDOT FOR MONUMENTS “29-0078-2”

WITH NAD 83/NA 2011 STATE PLANE GRID COORDINATES OF

NORTHING:	801885.2239(ft) EASTING: 	1499727.3423(ft)

ELEVATION:	728.65(ft)

THE AVERAGE COMBINED GRID FACTOR USED ON THIS PROJECT

(GROUND TO GRID) IS: 0.9999152857

THE N.C. LANBERT GRID BEARING AND

LOCALIZED HORIZONTAL GROUND DISTANCE FROM

“29-0078-2” TO –L- STATION IS

ALL LEINEAR DIMENSIONS ARE LOCALIZED HORIZONTAL DISTANCES

VERTICAL DATUM USED IS NAVD 88

===========================================================================

BASELINE: BL = BASELINE ALONG SR 1533 (COUNTY LINE ROAD).

THE BASELINE WAS SET PER NCDOT L&S GUIDELINES AND CAN BE FOUND IN GRAPHICS

FILE (290078_NCDOT_FS_161216.dgn) AND AS CHAIN (BL) IN GPK FILE (290078_LS_GPK_161216.gpk).

===========================================================================

BESTFIT ALIGNMENT:

EL = CENTERLINE ALIGNMENT FOR SR 1533 (COUNTY LINE ROAD).

THE BESTFIT ALIGNMENT WAS CREATED FROM THE DATA SHOTS AS LOCATED IN THE

FIELD AND CAN BE FOUND IN GRAPHICS FILE (290078_NCDOT_FS_161216.dgn)

AND AS A CHAIN (EL) IN GPK FILE (290078_LS_GPK_161216.gpk).

===========================================================================

DTM DATA:

COMPLETE DTM GROUND TOPO WAS LOCATED IN THE FIELD BASED ON THE LIMITS OF

770' SOUTH AND 412’ NORTH OF THE CENTER OF BRIDGE #78, 75' EAST AND WEST OF THE CENTERLINE OF THE ROADWAY NORTH OF BRIDGE OVER HUNTING CREEK, 72’ NORTH AND SOUTH OF CL OF HUNTING CREEK EAST OF SR1338, SOUTH OF HUNTING CREEK BRIDGE 660’ TO SOUTHER LIMITS, 75’ EAST TO EASTERLY LIMITS, AND 315’ WEST TO WESTERLY LIMITS, FIELD COLLECTED DTMS CAN BE FOUND IN GRAPHICS FILE (290078_LS_DTL_161216.dgn).

TIN FILE (290078_LS_TNL_161216.tin) WAS CREATED FROM DTMS WITH:

DISSOLVE OPTION: SIDE = 100 feet

MINIMUM LINEAR DISTANCE = 30 feet

ARC STROKE TOLERANCE =0.15 feet

===========================================================================

HYDROGRAPHIC DATA:

HYDRO DATA WAS COLLECTED IN THE FIELD ACCORDING TO NCDOT L&S HYDRO GUIDELINES

AND CAN BE FOUND IN GRAPHICS FILE (290078_NCDOT_FS_161216.DGN). NO NEW INFO

IS AVAILABLE AT THIS TIME TO DETERMINE IF THE STREAM IS JURISDICTIONAL.

===========================================================================

PROPERTY:

PROPERTY WAS CREATED FROM DEEDS AND PLATS OF RECORD BASED ON EXISTING

MONUMENTATION AS LOCATED IN THE FIELD AND CAN BE FOUND IN GRAPHICS

FILE (290078_NCDOT_FS_161216.dgn).

THE BEST FIT ALIGNMENT, IN CONJUNCTION WITH WIDTH PROVIDED PER PLAT BOOK 12 PAGE 143, BY GRADY L. TUTTEROW (SW 4682’ FROM CL OF BRIDGE) WAS USED TO PLACE THE 60' WIDE R/W ALONG SR 1338 (COUNTY LINE RD). R/W ABSTRACT NOT PROVIDED BY NCDOT.

PROPERTY OWNER INFORMATION CAN BE FOUND IN (290078_LS_POC_161216.mdb).

==========================================================================

POLE DATA:

NO ID TAGS OR BRANDS WERE PRESNT ON POLES AT SITE

WIRE HEIGHT INFORMATION

LOWEST WIRE NEAR BRIDGE (POWER LINE OVER HUNTING CREEK)

NORTHING 801740.9’ EASTING 1499775.1’ WIRE ELEVATION 743.2’

===========================================================================

UNDERGROUND UTILITY DATA:

SUE DESIGNATION WAS PERFORMED FOR UNDERGROUND UTILITIES BY NC ONECALL AND LOCATED BY DEWBERRY.  THE STORM SEWER, AND THE ABOVE GROUND EVIDENCE OF UNDERGROUND UTILITIES WERE LOCATED BY DEWBERRY AND CAN BE FOUND IN GRAPHICS FILE

(290078_NCDOT_FS_161216.dgn).

# From: North Carolina 811 <automated@nc811.org>
Sent: Mon 10/31/2016 11:27 AM
To: Rudolph, Matthew <MRudolph@Dewberry.com>
Subject: NCOC EMLCFM 2016/10/31  #01835 C163051283-00C NORM NEW DSGN

EMLCFM 01835 NCOCc 10/31/16 11:26:45 C163051283-00C NORM NEW GRID DSGN

North Carolina 811

SURVEY/DESIGN

Ticket : C163051283 Date: 10/31/16 Time: 11:25 Oper: CLB Chan:999

Old Tkt: C163051283 Date: 10/31/16 Time: 11:26 Oper: CLB Rev :00C

State: NC Cnty: DAVIE Place: CALAHALN In/Out: B

Subdivision:

Address :

Street  : COUNTY LINE RD   Intersection: N

Cross 1 : SNOW BECK TRL

Location: MEMBER OPERATORS SHOULD RESPOND BY 11/15/2016

DISTANCE FROM CROSS STREET IS: 0.51 MILES/SOUTH

BRIDGE OVER YADKIN RIVER

LOCATE NORTH OF THE BRIDGE FOR 350 FT AND SOUTH OF THE BRIDGE FOR 700 FT, BOTH

SIDES/MIDDLE OF THE RD.

:

Grids   : 3556B8041C   3556C8041C

Work type:SURVEY/DESIGN

Work date:11/15/16  Time: 23:59  Hours notice: 240/240  Priority: NORM

Ug/Oh/Both: U  Blasting: N  Boring: N  Railroad: N     Emergency: N

Duration: UNKNOWN  Done for: NCDOT

Company : DEWBERRY  Type: CONT

Co addr : 6135 LAKEVIEW RD SUITE 150

City    : CHARLOTTE State: NC Zip: 28269

Caller  : MATT RUDOLPH Phone: 704-264-1257

Contact : MATT AT ALT NUMBER Phone:

BestTime:

Cellular: 704-200-8025

Email   : mrudolph@dewberry.com

Remarks : CUSTOMER STATES THAT THIS IS THE YADKIN RIVER, NC811 MAP SHOWS THAT TO

BE CALLED HUNTING CREEK.

:

Submitted date: 10/31/16 Time: 11:26

Good thru date: 11/22/16 Time: 23:59  Update by date: 11/17/16 Time: 23:59

Members:

CLS01  = USIC LOCATING SERVICES, INC. CRE01* = ENERGYUNITED

CSR02  = WTR/SWR/STR DRAIN ONLY LOCAT DPU01  = DAVIE COUNTY PUBLIC UTILITIE

MAP01  = MAPPING CORRECTIONS          TST06  = TEST MEMBER

YVT01* = YADTEL

Member      Type    Contact                         Phone

CRE01*      Field   DISPATCH                        800-778-9140

Damage  KEVIN REEVES                    704-880-7534

DPU01       Field   ANDY ATWOOD                     336-909-4034

Damage  ANDY ATWOOD                     336-909-4034

YVT01*      Field   1 - REPAIR LINE                 336-463-5047

Field   2 - CINDY FOSTER                336-463-5075x5194

Damage  GREG DAVIS                      336-428-0730

If any damage involves a release of product or injury, please call 911

before contacting the owner.

Water, sewer and storm drain lines are marked within the right of way or

to the meter, and not on private property. Lines from the right of way or

meter to the residence or business are private lines and you will need to

contact a private line locator to have these lines located. Member facility

owners are not responsible for marking private lines. For a listing of

private line locators, please visit our website, www.nc811.org.  You must

contact any non-member facility owners not listed on your location request

directly to request their facilities to be located and marked.

==========================================================================

THE FOLLOWING FILES WERE CREATED AND TRANSMITTED:

290078_NCDOT_FS_161216.dgn (2D GRAPHICS TOPO FILE)

290078_LS_DTL_161216.dgn (3D GRAPHICS DTM FILE)

290078_LS_TNL_161216.tin (TIN FILE)

290078_LS_GPK_161216.gpk (GPK FILE)

290078_LS_POC_161216.MDB (PROPERTY OWNER CONTACT FILE)

290078_LS_SR_161216.docx (THIS SURVEY REPORT FILE)

290078_LS_BASELINE_161216 (BASELINE REPORT)

290078 DEED BOOK (PROPCON DEED REPORT)

LOOKING NORTH ON COUNTY LINE RD. (SR 1338)

LOOKING SOUTH ON COUNTY LINE RD. (SR 1338)

EAST SIDE OF SR 1562 (UPSTREAM) LOOKING UNDERNEATH (SW).

ON BRIDGE OVER HUNTING CREEK, SR 1338  LOOKING EAST.

Level Loop Adjustment