Project#:	45734.1.1			TIP#:	B-5778			ID#:	B5778-con-ame

Desc:		Bridge No. 61 over Fourth Creek on NC 801

Survey Date:		April 21, 2016

Last update:		April 22, 2016

Scale:			1”=50’

File Format:		MicroStation V8i

SPECIAL NOTES:

This is an English project.

All baseline points are 24” rebar with caps (unless noted otherwise)

Project mapping scale 1” = 50’

All baseline points are set to benchmark specification (unless noted otherwise)

Units of measurement are US survey feet

PERSONNEL:

Amec Foster Wheeler

4021 Stirrup Creek Drive, Suite 100

Durham, NC 27703

Contact Persons:

Andy DeGroat, PLS		(704) 357-5508

Todd Adams, SI 		(704) 357-5596

William “Corey” Kohnle

Christian Kohnle

EDM CALIBRATION:

Amec Foster Wheeler followed Location & Surveys Procedure 2015-3, “NCGS Baseline Calibrations” for this project. The baseline calibration data has been provided in the text file.

BESTFIT ALIGNMENT:

The best fit alignment was created from data shot in the field and can be found in the graphics file and as chains in the gpk file.

DTM DATA:

DTM was created from ground topo located in the field based on the limits of 650’ southwest, 600’ northeast of bridge 61 over Fourth Creek, on NC 801. Collected DTM’s can be found in the graphics file.

The TIN file was created from the DTM file with settings of:

Dissolve Option: Side = 100 feet

Minimum Linear Distance = 30 feet

Arc Stroke Tolerance = 0.15 feet

PROPERTY:

The property shown in the FS file was checked from information collected at the county of Rowan court house. Using this information the property data base was created and used to create and send out property owner notifications. The property information has been provided.

POLE DATA:

Pole data information was checked in the field and updated in the pole data file.

UNDERGROUND UTILITY INFORMATION:

SUE features were marked by NC 811 One Call. Amec Foster Wheeler field located these marks. The SUE information can be found in the graphics file.

THE FOLLOWING FILES WERE CREATED AND TRANSIMITTED:

b5778_NCDOT_FS_160424.dgn	           (2D graphics full field survey file)

b5778_ls_dtl_160423.dgn		           (3D graphics DTM file)

b5778_ls_tnl_160423.tin			(TIN File)

b5778_ls_gpk_160424.gpk		(Geopak coord. File)

b5778_ls_pdf_160426.txt			(Pole data file)

b5778_ls_sr_160426.doc			(Survey report file)

b5778_ls_cert_160426.pdf		           (Certification letter)

b5778_ls_ecr_160408.rpt			(EDM Calibration report)

b5778_ls_input_160426.txt		(Input file)

b5778_ls_baseline_160426.txt		(Baseline text file)

Figure 1 - Standing at the bridge looking southwest

Figure 2  - Standing at the bridge looking northeast

Figure 3 - Standing at the bridge looking upstream

Figure 4 - Standing at the bridge looking downstream

Figure 5 – Standing on creek bank looking under bridge.