FINAL SURVEY REPORT									 July 16, 2014

PROJECT No.      17BP.11.R.95

TIP No.                 940046

COUNTY:             WATAUGA

ROUTE:               SR 1351

LS ID No.:            DIV11TGSBG#1CONV

DESCRIPTION:    BRIDGE NO. 46 OVER GRASSY CREEK ON SR 1351 (CASTLE FORD ROAD)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

John J. Thomas, SI

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT GPS MONUMENT

"46-1" WITH A NORTHING OF 925382.478' AND AN EASTING OF 1233717.606'.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99988940.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID G12NC) AND BASED ON

NCDOT GPS "46-1" (ELEV 3029.44')

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00.00 (BL-3) and runs northerly along SR 1351 to station 15+04.57 (46-2).

There is no baseline BY alignment.

The existing centerlines were calculated but not staked for Line EL SR 1351.

Elevations were obtained for each baseline control point and two benchmarks were established.

Baseline control data transmitted in the graphics FS file and the baseline text file.

=========================================================================================

PLANIMETRIC DATA:

The planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 55 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

There is a SBT&T 200 pair direct buried copper telephone cable.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The apparent utility pole owners are Blue Ridge Electric Membership Corp. and SBT&T.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Property owner contacts were made by Wilkesboro Location & Surveys.

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the 940046_ls_deeds.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A right of way agreement was obtained from the Register of Deeds. The basis of right of way was found in Deed book 549 page 896, grantor Joel D. Cook, Deed book 549 page 897, grantor Marc A. Yorkson, Deed book 549 page 898, grantor Della M. Cook, Deed book 549 page 899, grantor Louise C. Howard and Deed book 549 page 900, grantor Ralph Norris.  These deeds were scanned and transmitted in the Adobe RW_Abstract.pdf file.

Existing right of way lines were mapped utilizing the said agreement.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word SitePhoto.doc

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 940046_ls_fs.dgn

* 940046_ls_dtl.dgn

* 940046_ls_tnl.tin

* 940046_ls_gpk.gpk

* 940046_ls_deeds.pdf

* 940046_ls_sr.docx

* 940046_ls_poc.mdb

PROJECT SITE PHOTOGRAPHS

Looking North and Upstream 940046		  Looking South and Downstream 940046

Looking West 940046					Looking East 940046

South Elevation 940046				   North Elevation 940046