FINAL SURVEY REPORT									 July 3, 2014

PROJECT No.      17BP.11.R.109

TIP No.                 940149

COUNTY:             WATAUGA

ROUTE:               SR 1351

LS ID No.:            DIV11TGSBG#1CONV

DESCRIPTION:    BRIDGE NO. 149 OVER GRASSY CREEK ON SR 1351 (CASTLE FORD ROAD)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT GPS MONUMENT

"149-2" WITH A NORTHING OF 927439.741' AND AN EASTING OF 1236131.200'.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99989291.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID G12NC) AND BASED ON

NCDOT GPS "149-2" (ELEV 2980.26')

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (BL-4) and runs northerly along SR 1351 to station 17+61 (149-2).

There is no baseline BY alignment.

The existing centerlines were calculated but not staked for Line EL SR 1351.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file and the baseline text file.

=========================================================================================

PLANIMETRIC DATA:

The planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 100 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

No underground utilities were encountered.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The apparent utility pole owner is Blue Ridge Electric Membership Corp.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Property owner contacts were made by Wilkesboro Location & Surveys.

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A right of way agreement was obtained from the Register of Deeds. The basis of right of way was found in Deed book 550 page 6, grantor was James E. Cook.  No reference was found for Charles D. Norris.  This deed was scanned and transmitted in the Adobe RW_Abstract.pdf file.

Existing right of way lines were mapped utilizing the said agreement.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word SitePhoto.doc

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 940149_ls_fs.dgn

* 940149_ls_dtl.dgn

* 940149_ls_tnl.tin

* 940149_ls_gpk.gpk

* 940149_ls_Deeds.pdf

* 940149_ls_sr.doc

PROJECT SITE PHOTOGRAPHS

Looking North

Looking South

Looking South and Upstream

Looking North and Downstream

South Elevation

North Elevation