July 2, 2013

MEMORANDUM TO:	Mr. Gregory J. Thorpe, Ph.D., Manager

Project Development and Environmental Analysis Unit

FROM:			Mr. Charles W. Brown, PE, PLS

State Location and Surveys Engineer

SUBJECT:			Scoping Comments for Replacement of

Bridge No. 8 & 9 over Ivy River on NC 251

WBS 38548.1.2, Madison County, TIP B-4777

Utilities:

French Broad Electric Coop has aerial power lines the cross over NC 251 just South of the bridges and then run North along the West side of NC 251.

Frontier Telephone Co. has aerial phone lines on the same poles that cross over NC 251 South of the bridges but transition to a separate pole line that crosses over the Ivy River from the South end of bridge number 9 and runs North along the West side of NC 251.

Environmental:

Area has above ground propane and heating oil tanks, groundwater wells, and septic tank systems.  The Ivy River is a slow moving River that flows towards and into the French Broad River West of bridge number 9.  The Northeast quadrant is a steep slope wooded area.  The West side of the bridges is bounded by the French Broad River.

Page Two

Scoping Comments for B-4777

Cultural:

The area is a rural mountain community mixed with residential and commercial properties.  The Southwest quadrant has a tax preparation business located in a converted gas station with a small church next door at the intersection of SR 1589 (Ivy Hill Road) and NC 251.  There is also a block automobile repair business in the Southeast quadrant on the North side of the intersection of NC 251 and SR 1589 situated between the Ivey River and SR 1589 in close proximity to bridge number 8.

Engineering:

The bridges are located in a horizontal curve to the West on the North and South approaches with a vertical incline in the North.  They are on a divided two lane roadway reduced from four lanes approaching the bridges from the South.  North of the bridges the road is two land divided with median crossovers at both ends of the bridges.  Both bridges have been reduced to one lane possibly due to disintegration of the deck and girders.  Bridge number 8 shows the most sign of disintegration from crumbling girders and deck around the weep holes that drain directly into the Ivey River.  The posted weight limits are 32 tons single and 36 tons truck, tractor, semi-trailer. Bridge number 8 is a five span all concrete structure with three of the four bents in the river.  Bridge number 9 is a four span all concrete structure with two of the bents in the river.  Both bridges have concrete guardrails and wheel guards

Recommendation:

Replace bridges in existing footprint with temporary detour between the two structures.

PLT

CC:  Mr. Jay Bennett, PE, State Roadway Design Engineer

Mr. David Chang, PE, State Hydraulics Engineer

Mr. John Pilipchuk, PE, State Geotechnical Engineer

Mr. Phil Harris, PE, State Natural Environment Engineer

Mr. John Williams, PE, Project Planning Engineer Supervisor