===========================================================================

PROJECT NUMBER:	38406.1.2		B-4544	    HARNETT COUNTY

DESC: BRIDGE# 133 & 134 OVER BLACK RIVER & BLACK RIVER OVERFLOW ON SR 1722

(THREE BRIDGE RD).

DATE OF SURVEY: MAY 2015

LAST UPDATE: JUNE 15, 2015 (RAM)

===========================================================================

SPECIAL NOTES:

* THIS IS AN ENGLISH PROJECT

* ALL BASELINE POINTS ARE 24" REBAR WITH CAP (UNLESS OTHERWISE NOTED)

===========================================================================

PERSONNEL:

DEWBERRY

6135 LAKEVIEW ROAD, SUITE 150

CHARLOTTE, NC 28269-2618

(704) 509-9918

TIMOTHY RUDOLPH, PLS

RICHARD MCANNELL, PLS

DAREN STOKER

SHANE CHAMBERS

===========================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON

THE NAD 83/NA2011 STATE PLANE COORD. ESTABLISHED BY NCDOT FOR MONUMENT:

"B4544-1" WITH STATE PLANE GRID COORDINATES OF:

NORTHING: 5282973.240	EASTING: 2109420.617

AVG COMBINED GRID FACTOR FOR THIS PROJ. (GROUND TO GRID) = 0.9998691150

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

VERTICAL DATUM FOR THIS PROJ. IS (NAVD 88) AND BASED ON NCDOT MONUMENT

"B4544-1" (ELEV.170.85’).

===========================================================================

BASELINE: BL = BASELINE ALONG SR 1722 THREE BRIDGE RD.

THE BASELINE WAS SET PER NCDOT L&S GUIDELINES AND CAN BE FOUND IN GRAPHICS

FILE (B4544_NCDOT_FS_150615.dgn) AND TEXT FILE (B4544_LS_BASELINE_150615.TXT)

AND AS CHAIN (BL) IN GPK FILE (B4544_LS_GPK_150615.gpk).

===========================================================================

===========================================================================

BESTFIT ALIGNMENT:

EL = CENTERLINE ALIGNMENT FOR SR 1722 THREE BRIDGE RD.

THE BESTFIT ALIGNMENT WAS CREATED FROM THE DATA SHOTS AS LOCATED IN THE

FIELD AND CAN BE FOUND IN GRAPHICS FILE (B4544_NCDOT_FS_150615.dgn) AND AS

CHAIN (EL) IN GPK FILE (B4544_LS_GPK_150615.gpk).

===========================================================================

DTM DATA:

THIS JOB WAS FLOWN BY PHOTOGRAMMETRY SO ONLY EOP, CROWNLINE, CREEK AND

OBSCURE DTMS WERE LOCATED IN THE FIELD BASED ON THE LIMITS OF 500' WEST OF BRIDGE# 134 AND 500’ EAST OF THE BRIDGE# 133, 75' NORTH AND 75’ SOUTH OF THE CENTERLINE OF THE ROADWAY AND 75’ UP AND DOWN THE STREAM FROM CENTERLINE BRIDGES# 133 & 134.  FIELD COLLECTED DTMS WERE MERGED TOGETHER WITH PHOTOGRAMMETRY DTMS AND CAN BE FOUND IN GRAPHICS FILE (B4544_LS_DTM_150615.dgn).

TIN FILE (B4544_LS_TIN_150615.tin) WAS CREATED FROM DTMS WITH:

DISSOLVE OPTION: SIDE = 100 feet

MINIMUM LINEAR DISTANCE = 30 feet

ARC STROKE TOLERANCE =0.15 feet

=============================================================================

HYDROGRAPHIC DATA:

HYDRO DATA WAS COLLECTED IN THE FIELD ACCORDING TO NCDOT L&S HYDRO GUIDELINES

AND CAN BE FOUND IN GRAPHICS FILE (B4544_NCDOT_FS_150615.dgn). NO INFO IS

AVAILABLE AT THIS TIME TO DETERMINE IF THE STREAM IS JURISDICTIONAL.

=============================================================================

PROPERTY:

PROPERTY WAS CREATED FROM DEEDS AND PLATS OF RECORD BASED ON EXISTING

MONUMENTATION AS LOCATED IN THE FIELD AND CAN BE FOUND IN GRAPHICS

FILE (B4544_NCDOT_FS_150615.dgn). THE BEST FIT ALIGNMENT WAS USED TO PLACE

THE 60' WIDE MAINTENANCE R/W.  R/W WIDTH PER FIELD EVIDENCE FOUND.  PROPERTY OWNER

INFORMATION CAN BE FOUND IN (B4544_LS_POC_150615.mdb).

=============================================================================

=============================================================================

POLE DATA:

POLE DATA INFORMATION CAN BE FOUND IN (B4544_LS_PDF_150615.txt).

OVERHEAD POWER LINE RUNS ALONG SOUTH SIDE OF SR 1722 THREE BRIDGE RD.

ELECTRIC POWER LINE OWNER IS PROGRESS ENERGY COAROLINA INC. (WOODEN POLES SEE EXAMPLE PICTURE NEAR BRIDGE BELOW).

TELEPHONE WIRE HEIGHT INFORMATION

LOWEST WIRE NEAR BIDGE#133 (TELEPHONE ATTACHED TO WOODEN POWER POLES)

NORTHING 581692.7’ EASTING 2112059.7’ WIRE ELEVATION 190.0’

OVERHEAD TELEPHONE LINE RUNS ALONG SOUTH SIDE OF SR 1722 THREE BRIDGE RD.

TELEPHONE POLE OWNER IS CAROLINA TELEPHONE AND TELEGRAPH COMPANY (WOODEN POLES SEE EXAMPLE PICTURE NEAR BRIDGE BELOW).

TELEPHONE WIRE HEIGHT INFORMATION

LOWEST WIRE NEAR BIDGE# 134 (TELEPHONE LINE ATTACHED TO WOODEN POLE)

NORTHING 581625.7’ EASTING 2111547.7’ WIRE ELEVATION 190.1’

=============================================================================

UNDERGROUND UTILITY DATA:

SUE FEATURES WERE MARKED BY NCONECALL.  (TICKET ##03732 C151311262-00C &#03736 C151311258-00C) DEWBERRY FIELD LOCATED THE ABOVE GROUND EVIDENCE OF UNDERGROUND UTILITIES.  . SUE DATA WAS FIELD DESIGNATED.  UNDERGROUND TELEPHONE AND WATER WAS LOACATED PER DESIGNATED PAINTED LINES THROUGHOUT THE PROJECT LIMITS.   SHOWN IN

GRAPHICS FILE (B4544_NCDOT_FS_150615.dgn).

=============================================================================

THE FOLLOWING FILES WERE CREATED AND TRANSMITTED:

B4544_NCDOT_FS_150615.dgn (2D GRAPHICS TOPO FILE)

B4544_LS_DTM_150615.dgn (3D GRAPHICS DTM FILE)

B4544_LS_TIN_150615.tin (TIN FILE)

B4544_LS_BASELINE_150615.TXT (BASELINE TEXT FILE)

B4544_LS_GPK_150615.gpk (GPK FILE)

B4544_LS_POC_150615.mdb (PROPERTY OWNER CONTACT FILE)

B4544_LS_SR_150615.docx (THIS SURVEY REPORT FILE)

B4544_LS_PDF_150615.txt (POLE FILE)

SR 1722 THREE BRIDGE ROAD LOOKING EAST

SR 1718 THREE BRIDGE ROAD LOOKING WEST

BRIDGE #133 LOOKING UPSTREAM NORTH

BRIDGE #133 LOOKING DOWN STREAM SOUTH

BRIDGE #134 LOOKING UPSTREAM NORTH

BRIDGE #134 LOOKING DOWN STREAM SOUTH

# From: North Carolina 811 [mailto:automated@nc811.org] 
Sent: Thursday, May 14, 2015 6:06 PM
To: Rudolph, Matthew
Subject: NCOC EMLCFM 2015/05/14 #03732 C151311262-00C NORM RESP LR

EMLCFM 03732 NCOCc 05/14/15 18:05:56 C151311262-00C NORM RESP GRID LR

Thank you for calling North Carolina 811!

This is an automatically generated response from the utilities

who received your notice of excavation.  For your safety,

please respect and protect the marks, and excavate carefully

around the marked utility lines.

This email comes from an automated program that is NOT MONITORED.

DO NOT REPLY BACK TO THIS EMAIL.

Ticket : C151311262 Rev: 00C Taken: 05/11/15 12:18 PM

State: NC Cnty: HARNETT Place: DUNN

Address :  THREE BRIDGE RD

Work date: 05/14/15 11:59 PM

Good thru: 06/02/15 11:59 PM  Update by: 05/28/15 11:59 PM

Member      Description               Response

Code

CPL40*      DUKE ENERGY

POWER                     05/14/15 06:05 PM   40

Could not gain access to property, locator will contact excavator

CTT03*      CTLCL-CENTURYLINK

PHONE AND FIBER OPTIC     05/14/15 06:05 PM   40

Could not gain access to property, locator will contact excavator

NEW01       HARNETT COUNTY DEPT. OF PUBLIC UTILITIES

WATER AND SEWER           05/14/15 08:12 AM   20

Marked

UTV01*      CHARTER

CABLE TV                  05/14/15 06:05 PM   40

Could not gain access to property, locator will contact excavator

Member      Type    Contact                         Phone

CPL40*      Field   DISPATCH                        800-778-9140

Damage  DUKE ENERGY REPRESENTATIVE      800-452-2777

CTT03*      Field   DISPATCH                        800-778-9140

Damage  GENERAL NUMBER                  800-788-3600

NEW01       Field   NO FIELD CONTACT NUMBER LISTED  000-000-0000

Damage  SHANE CUMMINGS                  910-893-7575

UTV01*      Field   USIC LOCATING SERVICES, INC.    800-778-9140

Damage  Not available

If any damage involves a release of product or injury, please call 911

before contacting the owner.

# From: North Carolina 811 [mailto:automated@nc811.org] 
Sent: Thursday, May 14, 2015 6:06 PM
To: Rudolph, Matthew
Subject: NCOC EMLCFM 2015/05/14 #03736 C151311258-00C NORM RESP LR

EMLCFM 03736 NCOCc 05/14/15 18:06:05 C151311258-00C NORM RESP GRID LR

Thank you for calling North Carolina 811!

This is an automatically generated response from the utilities

who received your notice of excavation.  For your safety,

please respect and protect the marks, and excavate carefully

around the marked utility lines.

This email comes from an automated program that is NOT MONITORED.

DO NOT REPLY BACK TO THIS EMAIL.

Ticket : C151311258 Rev: 00C Taken: 05/11/15 12:08 PM

State: NC Cnty: HARNETT Place: DUNN

Address :  THREE BRIDGE RD

Work date: 05/14/15 11:59 PM

Good thru: 06/02/15 11:59 PM  Update by: 05/28/15 11:59 PM

Member      Description               Response

Code

CPL40*      DUKE ENERGY

POWER                     05/14/15 06:05 PM   40

Could not gain access to property, locator will contact excavator

CTT03*      CTLCL-CENTURYLINK

PHONE AND FIBER OPTIC     05/14/15 06:05 PM   40

Could not gain access to property, locator will contact excavator

NEW01       HARNETT COUNTY DEPT. OF PUBLIC UTILITIES

WATER AND SEWER           05/14/15 08:11 AM   20

Marked

UTV01*      CHARTER

CABLE TV                  05/14/15 06:05 PM   40

Could not gain access to property, locator will contact excavator

Member      Type    Contact                         Phone

CPL40*      Field   DISPATCH                        800-778-9140

Damage  DUKE ENERGY REPRESENTATIVE      800-452-2777

CTT03*      Field   DISPATCH                        800-778-9140

Damage  GENERAL NUMBER                  800-788-3600

NEW01       Field   NO FIELD CONTACT NUMBER LISTED  000-000-0000

Damage  SHANE CUMMINGS                  910-893-7575

UTV01*      Field   USIC LOCATING SERVICES, INC.    800-778-9140

Damage  Not available

If any damage involves a release of product or injury, please call 911

before contacting the owner.

Process Compass Results                                Tue May 26 20:11:17 2015

Raw File: C:\rick\NCDOT\B-4544\FIELD FILES\5-25-15\ADJ TRAV\B4544 5-25-15.RW5

Coordinate File: C:\rick\NCDOT\B-4544\V-CARLSON\B4544 FIELD.crd

Scale Factor: 1.00000000

Correct for Earth Curvature: OFF

Closure Results (Before Angle Balance)

Starting Point B4544-2: N 582324.522 E 2110021.374 Z 203.658

Closing Reference Point B4544-2: N 582324.522 E 2110021.374 Z 203.658

Ending Point B4544-2 CHK: N 582324.401 E 2110021.298 Z 203.046

Azimuth Of Error: 211°59'44"

North Error     : -0.12111

East Error      : -0.07566

Vertical Error  : -0.61203

Hz Dist Error   : 0.14280

Sl Dist Error   : 0.62847

Traverse Lines  : 12

SideShots       : 2

Store Points    : 0

Horiz Dist Traversed: 5687.793

Slope Dist Traversed: 5688.320

Closure Precision: 1 in 39829

Starting Point B4544-2: N 582324.522 E 2110021.374 Z 203.658

Backsight Point B4544-1: N 582973.310 E 2109420.664 Z 224.665

Point          Horizontal 		Zenith     Slope        Northing          Easting

No.             Angle      		Angle        Dist

Description

BL-1           AR178.4446 		91.2447  625.760  581874.907  2110456.327

4020

BL-2           AR159.3856 		90.3943  575.060  581626.462  2110974.906

4020

BL-3           AR153.3740 		89.5654  533.300  581633.663  2111508.157

4020

BL-4           AR172.5749 		90.0319  544.865  581707.704  2112047.968

4020

BL-5           AR180.0358 		90.2109  575.560  581785.258  2112618.268

4020

6                 AR5.0527   		89.4036  320.115  581770.440  2112298.501

4020

7                AR174.4749 		89.4420  468.800  581706.362  2111834.106

4020

8                AR181.2906 		90.1559  646.740  581634.594  2111191.367

4020

9                       AR195.0902 	89.4433  318.020  581683.133  2110877.077

4020

10                      AR196.1942 	89.2029  339.040  581826.985  2110570.092

4020

11                      AR195.4711 	88.5533  317.820  582035.014  2110329.888

4020

B4544-2 CHK    AR182.1600 	88.1718  423.240  582324.401  2110021.298

4017

Sideshots

P7A       	            AR160.1729 	89.3501  188.040  581871.944  2112785.129

P7 CHK

C45441             AR184.0217 	88.3934  884.405  582973.125  2109420.548

CHK B45441

Angle Balance

Angular Error: -0°00'17.29" for 13 traverse sides

Adjusting Each Angle: -0°00'01.33"

Closure Results (After Angle Balance)

Starting Coordinates : N 582324.522 E 2110021.374 Z 203.658

Closing Reference Point B4544-2: N 582324.522 E 2110021.374 Z 203.658

Ending   Coordinates : N 582324.494 E 2110021.339 Z 203.046

Azimuth Of Error: 231°38'55"

North Error     : -0.02781

East Error      : -0.03515

Hz Dist Error   : 0.04482

Sl Dist Error   : 0.61367

Traverse Lines  : 12

SideShots

Total Hz Dist Traversed: 5687.79270

Total Sl Dist Traversed: 5688.32000

Closure Precision: 1 in 126894

Point          	Horizontal   Zenith      Slope         Northing        Easting

No.            	Angle            Angle        Dist

Description

BL-1           	AR178.4447 91.2447  625.760  581874.904  2110456.324

4020

BL-2           	AR159.3857 90.3943  575.060  581626.452  2110974.900

4020

BL-3           	AR153.3741 89.5654  533.300  581633.643  2111508.151

4020

BL-4           	AR172.5750 90.0319  544.865  581707.671  2112047.964

4020

BL-5           	AR180.0359 90.2109  575.560  581785.206  2112618.266

4020

6             	 AR5.0528   89.4036  320.115  581770.400  2112298.499

4020

7              	AR174.4750 89.4420  468.800  581706.343  2111834.101

4020

8              	AR181.2907 90.1559  646.740  581634.609  2111191.359

4020

9              	AR195.0903 89.4433  318.020  581683.166  2110877.071

4020

10             	AR196.1943 89.2029  339.040  581827.038  2110570.096

4020

11             	AR195.4712 88.5533  317.820  582035.083  2110329.906

4020

B4544-2 CHK     AR182.1601 88.1718  423.240  582324.494  2110021.339

4017

Sideshots

P7             	AR160.1729 89.3501  188.040  581871.887  2112785.130

CHK

B4544-1 CHK     AR184.0218 88.3934  884.405  582973.269  2109420.643

4017

Compass Closure

Adjusted Point Comparison

Original                                          Adjusted

Point#        	  Northing           Easting                Northing         Easting           Dist       Bearing

BL-1          	 581874.904  2110456.324 	581874.907  2110456.328 0.005  N 51°38'55" E

BL-2           	 581626.452  2110974.900 	581626.458  2110974.907 0.009  N 51°38'55" E

BL-3          	 581633.643  2111508.151 	581633.651  2111508.162 0.014  N 51°38'55" E

BL-4          	 581707.671  2112047.964 	581707.682  2112047.978 0.018  N 51°38'55" E

BL-5           	 581785.206  2112618.266 	581785.219  2112618.284 0.022  N 51°38'55" E

6               	 581770.400  2112298.499 	581770.416  2112298.519 0.025  N 51°38'55" E

7                 	 581706.343  2111834.101	581706.361  2111834.124 0.029  N 51°38'55" E

8                      	 581634.609  2111191.359	581634.630  2111191.385 0.034  N 51°38'55" E

9                       	 581683.166  2110877.071 	581683.188  2110877.099 0.036  N 51°38'55" E

10                     	 581827.038  2110570.096 	581827.062  2110570.126 0.039  N 51°38'55" E

11                     	 582035.083  2110329.906 	582035.109  2110329.938 0.041  N 51°38'55" E

B4544-2 CHK    	 582324.494  2110021.339 	582324.522  2110021.374 0.045  N 51°38'55" E

Max adjustment: 0.045

Starting Point B4544-2: N 582324.522 E 2110021.374 Z 203.658

Backsight Point B4544-1: N 582973.310 E 2109420.664 Z 224.665

Point          	Horizontal     Zenith      Slope    Northing         Easting

No.            	Angle              Angle        Dist

Description

BL-1          	 AR178.4446 91.2447  625.759  581874.907  2110456.328

4020

BL-2           	AR159.3857 90.3943  575.061  581626.458  2110974.907

4020

BL-3           	AR153.3742 89.5654  533.303  581633.651  2111508.162

4020

BL-4           	AR172.5750 90.0319  544.869  581707.682  2112047.978

4020

BL-5           	AR180.0359 90.2109  575.564  581785.219  2112618.284

4020

6              	AR5.0530     89.4036  320.113  581770.416  2112298.519

4020

7              	AR174.4750 89.4420  468.797  581706.361  2111834.124

4020

8             	 AR181.2907 90.1559  646.735  581634.630  2111191.385

4020

9              	AR195.0904 89.4433  318.018  581683.188  2110877.099

4020

10             	AR196.1944 89.2029  339.039  581827.062  2110570.126

4020

11             	AR195.4712 88.5533  317.820  582035.109  2110329.938

4020

B4544-2  CHK     AR182.1601 88.1718  423.241  582324.522  2110021.374

4017

Sideshots

P7             	AR160.1729 89.3501  188.040  581871.902  2112785.147

CHK

B4544-1 CHK     AR184.0218 88.3934  884.405  582973.301  2109420.683

4017

***********************************************************************************

***********************************************************************************

MISCLOSURE FROM DEWBERRY TRAVERSE TO NCDOT CONTROL AFTER ADJUSTMENT

BEARING      DISTANCE

P7        N 15°32'04" E    0.286'

B4544-1   S 47°12'12" W    0.008'

***********************************************************************************

***********************************************************************************

BECAUSE LOCAL SITE POINT B4544-1 WAS POSSIBLY DISTURBED DURING WATERLINE CONSTRUCTION

NEW COORDINATES VALUES WERE ESTABLISHED HOLDING B4544-2 AND ROTATING INTO (panel) P7.

THE ROTATION ANGLE FOR THE COORDINATE ADJUSTMENT IS -00°00'21"

NEW COORDINATE VALUES:

POINT#		NORTHING         EASTING               ELEVATION        DESCRIPTION

B4544-1	582973.240	2109420.617  	224.662	   4017

B4544-2	582324.522	2110021.374  	203.658	   4017

BL-1		581874.951	2110456.373 	188.608	   4020

BL-2		581626.555	2110974.978 	182.043	   4020

BL-3		581633.802	2111508.232  	182.577	   4020

BL-4		581707.887	2112048.040  	182.023	   4020

BL-5		581785.483	2112618.339  	178.839	   4020

P7		581872.182	2112785.193  	180.066	   PANEL

6		581770.647	2112298.575	180.253	   4020

7		581706.545	2111834.186	182.509  	   4020

8		581634.748	2111191.455	179.528	   4020

9		581683.275	2110877.164  	181.097	   4020

10		581827.118	2110570.177  	185.078	   4020

11		582035.140	2110329.968  	191.434	   4020

***********************************************************************************

***********************************************************************************

Start-Line	 1 ROD 			DIST.		 ELEV.		 ADJ. ELEV. 	NOTES

1 B4544-1	 1 					Z 224.662 			FLOATED

1 B4544-1	 1 Rb 1.9899 		HD 176.64

2 TURN 	 1 Rf 7.9622 		HD 154.88

2 TURN		 1					 Z 218.690 	218.690

2 TURN		 1 Rb 0.8571	  	 HD 166.63

3 TURN		 1 Rf 9.1075		 HD 163.17

3 TURN		 1 					Z 210.440 	210.439

3 TURN		 1 Rb 2.0128		 HD 104.40

4 B4544-2	 1 Rf 8.7944		 HD 117.18

4 B4544-2	 1 					Z 203.658	 203.658 	FIXED

4 B4544-2	 1 Rb 2.3941		 HD 103.14

5 TURN		 1 Rf 8.0300		 HD 106.18

5 TURN		 1 					Z 198.022 	 198.022

5 TURN		 1 Rb 1.9281		 HD 117.20

6 TURN		 1 Rf 8.1426		 HD 113.62

6 TURN		 1 					Z 191.808 	 191.807

6 TURN		 1 Rb 3.1343		 HD 101.03

7 BL-1		 1 Rf 6.3332		 HD 84.18

7 BL-1		 1 					Z 188.609 	188.608 	FLOATED

7 BL-1		 1 Rb 2.0502		 HD 230.19

8 TURN		 1 Rf 7.4903		 HD 218.48

8 TURN		 1 					Z 183.169 	 183.168

8 TURN		 1 Rb 4.5921		 HD 58.51

9 BL-2		 1 Rf 5.7169		 HD 74.81

9 BL-2		 1 					Z 182.044 	 182.043	 FLOATED

9 BL-2		 1 Rb 4.2502		 HD 172.27

10 TURN	 1 Rf 4.8077		 HD 164.57

10 TURN	 1 					Z 181.486 	 181.486

10 TURN	 1 Rb 5.7538		 HD 94.40

11 BL-3		 1 Rf 4.6620		 HD 103.26

11 BL-3		 1 					Z 182.578 	 182.577	 FLOATED

11 BL-3		 1 Rb 5.3102		 HD 161.32

12 TURN	 1 Rf 5.2958		 HD 161.53

12 TURN	 1 					Z 182.593 	 182.592

12 TURN	 1 Rb 5.0939		 HD 114.44

13 BL-4		 1 Rf 5.6627		 HD 116.86

13 BL-4		 1 					Z 182.024 	 182.023	 FLOATED

13 BL-4		 1 Rb 4.3406		 HD 188.18

14 TURN	 1 Rf 7.7819		 HD 191.43

14 TURN	 1 					Z 178.582 	 178.581

14 TURN	 1 Rb 5.5980		 HD 94.29

15 BL-5		 1 Rf 5.3400		 HD 113.68

15 BL-5		 1 					Z 178.840 	 178.839	 FLOATED

15 BL-5		 1 Rb 6.1862		 HD 101.17

16 P7		 1 Rf 4.9593		 HD 87.02

16 P7		 1 					Z 180.067 	 180.066	 FLOATED

16 P7		 1 Rb 4.9443 HD 86.97

17 TURN	 1 Rf 5.4065		 HD 96.41

17 TURN	 1 					Z 179.605 	179.604

17 TURN	 1 Rb 4.2924	 HD 183.35

18 6		 1 Rf 3.6436	 HD 140.29

18 6		 1 					Z 180.254 	 180.253 	FLOATED

18 6		 1 Rb 7.3226	 HD 258.93

19 7		 1 Rf 5.0664	 HD 213.42

19 7		 1 					Z 182.510 	 182.509	 FLOATED

19 7		 1 Rb 5.0832	 HD 170.81

20 TURN	 1 Rf 5.2729	 HD 155.74

20 TURN	 1 					Z 182.320 	 182.319

20 TURN	 1 Rb 3.8565	 HD 135.29

21 8		 1 Rf 6.6476	 HD 189.00

21 8		 1 					Z 179.529 	 179.528	 FLOATED

21 8		 1 Rb 7.0257	 HD 166.79

22 9		 1 Rf 5.4566	 HD 161.66

22 9		 1 					Z 181.098 	 181.097	 FLOATED

22 9		 1 Rb 8.3331	 HD 158.98

23 10		 1 Rf 4.3513	 HD 190.20

23 10		 1 					Z 185.080 	 185.078	 FLOATED

23 10		 1 Rb 8.6764	 HD 133.36

24 11		 1 Rf 2.3202	 HD 194.05

24 11		 1 					Z 191.436 	 191.434	 FLOATED

24 11		 1 Rb 9.0201	 HD 129.75

25 TURN	 1 Rf 1.7306	 HD 120.66

25 TURN	 1 					Z 198.726 	 198.724

25 TURN	 1 Rb 7.7427	 HD 90.69

26 B4544-2	 1 Rf 2.8148	 HD 82.96

26 B4544-2	 1 					Z 203.654 	 203.652	 FLOATED

26 B4544-2	 1 Rb 9.1066	 HD 141.62

27 TURN	 1 Rf 0.7033	 HD 140.29

27 TURN	 1 					Z 212.057 	 212.055

27 TURN	 1 Rb 7.9274	 HD 128.60

28 TURN	 1 Rf 2.1485	 HD 124.65

28 TURN	 1 					Z 217.836 	 217.834

28 TURN	 1 Rb 8.5734	 HD 167.70

29 B4544-1	 1 Rf 1.7448	 HD 180.31

29 B4544-1	 1 					Z 224.665 	 224.662 FLOATED

29 B4544-1	 1 Sh 0.0023 	dz -0.0023 		Z 224.662

29 B4544-1	 1 Db 3936.65 	Df 3960.49 		Z 224.665

End-Line 1

RATIO OF CORRECTION = -0.00000029