NCFMP and NCDOT MOA Project Review

| GENERAL INFORMATION |  | LOCATION INFORMATION |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project TIP / ID: | B-4544 |  | Stream Name : |  | Black River |  |
| WBS: | 38406.1.2 |  | River Basin : |  | Cape Fear |  |
| Initial Submittal Date: | 2/7/2017 |  | Regulating City: |  | N/A |  |
| MOA Approval Target Date: | 5/8/2017 |  | Regulating County: |  | Harnett |  |
| Project LET DATE: | 2/20/2018 |  | Division: |  | 06 |  |
| DOT Project Manager: | Paul Atkinson |  | Community ID Number: |  | 370328 |  |
| DOT Design Engineer: | Craig A. Freeman |  | Latitude (in decimal degrees): |  | 35.24782 |  |
| In-House or PEF project? | PEF |  | Longitude (in decimal degrees): |  | -78.62413 |  |
| PEF (Name of Firm) | RS&H |  | Route Number: |  | SR 1722 |  |
| DOT STR. INV. # (6-digit): | 420134 and 420133 |  | Road Name (on FIRM Panel): |  | Three Bridge Road |  |
|  |  |  | Effective FIS date: |  | Rev. 7/17/2007 |  |
|  |  |  | Panel Number (4-digit): |  | 1518 |  |
|  |  |  | Panel Effective Date: |  | 10/3/2006 |  |  |
|  |  |  | Published section # up/down stream |  | 71364.8/79126.2 |  |  |

FEMA/FMP STATUS

| Status (MOA TYPE: 1, 2a, 2b, 3a, 3b): | 2a |
| --- | --- |
| If MOA Type 3a or 3b, CLOMR case number: |  |
| Type Of Study (detailed, limited, redelineated): | Limited |
| Effective Model on File? (Yes or No): | Yes |
| Effective Model Format (HEC-RAS, HEC2): | HEC-RAS |
| Model Version No. | 3.1.3 |
| Proposed Model Format (HEC-RAS, HEC2): | HEC-RAS |
| Model Version No. | 4.1.0 |

PROJECT SPECIFIC NOTES (pre-model review)

This project has a maximum decrease of 0.3-ft, and qualifies under Type 2a MOA.

.

FMP/DOT NOTES AND REVIEW COMMENTS:

[DOT staff: Add any pertinent notes from FMP/DOT coordination meetings/correspondence.]

- Please add “Revised 7/17/2007” to the Effective FIS date in the FEMA Coordination Documentation Form.

Comment Incorporated

- When the HEC-RAS model was run using the Duplicate Effective (DE) plan, an error message was displayed that a flow has been specified for RS 71440, but this RS does not exist in the geometric data.  The submitted HEC-RAS 4.1.0 model uses a single steady flow file (100-Year Single) for the DE, Corrected Effective (CE) and Revised (REV) models. Since the flow at RS 71440 in the DE model has been reassigned to RS 71410.8 in CE & REV models, please include an additional steady flow file for the DE model.

Comment Incorporated

- The span of the existing Bridge #1 (Bridge No. 133) is 53.50-ft per the BSR, but the model has it as 55.25-ft. Similarly, the span of the existing Bridge #2 (Bridge No. 134) is 53.00-ft per the BSR, but the model has it as 55.75-ft.  The Narrative text indicates the span for both bridges as 55-ft.  Please verify the spans and effective spans of both existing bridges and revise the Narrative text and/or model as necessary.

Per Bridge Inspection Report, both bridges have a total length of 54’, rather than 55’ as had been previously stated. The effective length of Bridge 1 (133) is 53’ (from the BSR / Bridge Inspection Report), so it is now modeled as a 54’ bridge with 0.5’ abutments. The effective length of Bridge 2 (134) is 53.5’ (from the BSR / Bridge Inspection Report), so it is now modeled as a 54’ bridge with 0.25’ abutments. The narrative has been revised to reflect these total and effective lengths. The effective lengths changed by 0.25’ for each bridge.

- The calculated expansion ratio for the ineffective flow station placement on the left side of existing bridge #2 is 1.84:1. This differs from the right side expansion coefficient of 1.93:1. Please verify and revise the CE model if necessary.

The ineffective flow boundaries were updated to reflect the minor changes in the bridge width. They are set at a 1:1 contraction and a 2:1 expansion ratio.

- It appears that the right ineffective flow area elevation needs to be lowered at RS 71321 in the CE model to show that the 100 year is overtopping (see figure below).  Please verify and revise as necessary to ensure that right overbank flow around the bridge is consistent with the weir flow.

The ineffective flow area elevation was lowered so that the right over bank has effective flow to match the weir flow at RS 71321.

Figure: Cross section 71321 in CE model

FINAL DETERMINATION:

A final determination will be established once the above comments are addressed.

______________________	_

Rahish Khatri, PE	   Date	3/14/2017

Project Engineer