Low Impact Bridge Survey Report 04-0023

- Structure # 04-0023 on NC 194 over Old Field Creek

- September 5th, 2017

- TGS Engineers

- DATUM DESCRIPTION

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE STATE PLANE COORDINATES FOR MONUMENT “23-2" WITH NAD 83/NA 2011 STATE PLANE COORDINATES OF NORTHING 1010021.150’, EASTING OF 1261789.945’.  THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99998761.  ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES. THE VERTICAL DATUM IS BASED ON MONUMENT “23-2" ELEV. 2657.60’ (NAVD 88).

- (WGS-84) Lat=36°29’57.4" N Long=81°30’40.7"W.

- There were no utilities attached to the bridge. There are Skyline 400 & 600 Pair copper telephone lines along with 16 strand F/O line buried on the south side of the bridge. Also there is a Town of Lansing owned 4” PVC water line and 8” PVC sewer line buried on the south side.

- There are utility poles located in the project limits with apparent owner as Blue Ridge Electric Membership Corp. The underground utilities as mapped were designated by TGS Engineers.

- BRIDGE PHOTOS:

Looking North	Upstream			              Looking South Downstream

Looking East 			         		                Looking West

North (Pedestrian) Elevation			               South Elevation

- A Right of Way Abstract has been provided by Division 11 Right of Way Agent dated July 31, 2017.  The right of way as mapped according to this abstract.

- Bridge Seat Elevation Data:

| Quadrant | Elevation |
| --- | --- |
| Southwest | 2656.77’ |
| Southeast | 2656.42’ |
| Northwest | 2656.73’ |
| Northeast | 2656.46’ |

Overview:

This is a single span two lane bridge with an asphalt filled steel plank deck on steel I-beams on concrete abutments.  The bridge also has steel curbs, guardrails and posts.

Cultural:

Bridge is located on NC 194 in the Town of Lansing in Ashe County.