HEC-RAS Model Narrative – Bridge B4950

Duplicate Effective Model

The effective HEC-RAS model version 3.1.0 for South River was obtained from NCDOT.  The effective model was run to produce the duplicate effective model using HEC-RAS version 4.1.0. There is no difference between the duplicate effective and the effective models except at XS’s 414175.4, 410000, 406000, 403358.3, 374000 and 361148.9 (0.01 foot difference) with respect to the WSEL’s. These differences were attributed to model version differences. The discrepancy in the surcharge was updated in the corrected effective model.  Topographic data supplied by NCDOT was referenced to NAVD 88.

Corrected Effective Model

The duplicate effective model was used to develop the corrected effective model.  The existing bridges were not modeled in the effective model as they currently exist in the field.  There are two bridge crossings in the floodplain at this location, not one as shown in the effective model.  One bridge (250172) conveys the main channel and another bridge is an overflow/relief bridge (250171).   Multiple opening bridge conveyance methodology will accurately predict the water surface elevations at this crossing.  Thus the bridge crossing was changed to a multiple opening option in the bridge selection project. Internal bridge data for the two crossings were obtained from field survey, soundings data and supplemental NCDOT survey data. The total length of Bridge 250171 is 90.75’. It has 1’ wide abutments, making the effective bridge length 88.75’. The total length of Bridge 250172 is 150.75’. It has 1’ wide abutments, making the effective bridge length 148.75’. Bridge 250171 has two interior bents and bridge 250172 has three interior bents. The interior bents of both bridges consist of dual columns in the direction of flow that are each 1.17’ in diameter. The road elevation was modified to match the highest roadway elevations obtained from NCDOT survey.

Effective XS’s 346503.7 and 346457.7 were inside the influence of the existing bridge contraction/expansion zone of influence and embankment slope.  These two cross-sections were moved away from the influence to toe of the embankment areas.  The new no.3 and no.2 stations at the bridge crossing are 346534 and 346437 respectively.  Also, two new cross-sections stations 346629.3 and 346295 were added to better reflect the approach and expanded sections.  The geometry of the existing cross-sections and the new stations were revised based on the survey supplied by NCDOT and field survey/reconnaissance (346629.3, 346534, 346437, and 346295). The encroachment widths for the additional cross-sections were selected per method 4 and converted to method 1 (HEC-RAS floodway methodology).  The Manning’s n values for the additional cross-sections were coded to reflect the existing conditions based on the effective model, our field survey, and aerial imagery provided by NCDOT. The ineffective flow boundaries were adjusted on the upstream side to reflect a 1:1 contraction ratio and the ineffective flow boundaries were adjusted on the downstream side to reflect a 2:1 expansion ratio.

Revised Model

The corrected effective model was used to develop the revised model.  No new cross-sections were added.  The proposed bridge geometric data was revised based on the proposed roadway profile and bridge alignment.  The ineffective areas at XS’s 346629.3, 346534, 346437 and 346295 were revised based on the proposed bridge geometry. The total length of Bridge 250171 is 120’. It has end bents that project 1.625’ into the opening, making the effective bridge length 116.75’. The total length of Bridge 250172 is 185’. It has end bents that project 1.625’ into the opening, making the effective bridge length 181.75’. Both bridges have interior bents consisting of one row of 3’ diameter columns.

The proposed bridge rail was coded in the model (Rail height = 3.5 ft. minus the cross-slope at the bridge) as directed in the NCDOT document “Bridge Rail Modeling Guidelines in HEC-RAS” for the bridge opening area.  In accordance with this document, the proposed bridge rail is modeled for the length of the bridge and for the first 12’ of guardrail anchor unit at each end of the bridge (height of rail = 2.5 ft.). Also, the proposed bridge abutments were coded into the model based on the profile data. The upstream and downstream channel geometry in the internal bridge sections was modified to reflect the proposed bridge opening.  The ineffective flow area elevations were set based on the overtopping elevations.  The encroachment widths in the vicinity or within the project limits were set appropriately such that the surcharges are within the FEMA specified limit of 1.0 foot.

Conclusion

The proposed bridge results in decreases of the BFE along the reach (XS 346295 through 378000). The maximum decrease of 0.12’ was reported at stations 346629.3 and 348000.