# NORTH CAROLINA DEPARTMENT OF TRANSPORTATION

# FIELD SCOPING MEETING WORKSHEET

Design Build Process

| TIP No.: | WBS No.: DF15004.2096128 | #### Field Scoping Meeting Date: 11/28/16 |
| --- | --- | --- |
| Division: 4 |  | #### Location: On-site |
| Route (US/NC/SR): SR 1101 |  | County: Wayne |
| Project Description: Bridge No. 950156 over Ranyor Mill Branch on SR 1101 (Raynor Mill Road) |  |  |
| Tier: SUB REGIONAL |  | Funding: Federal |
| #### Municipality: N/A |  |  |

| Attendees |  |  |  |
| --- | --- | --- | --- |
|  | Name | Phone No. | E-mail |
| Division Bridge Program Manager | Corey McLamb | 252-237-6164 | cdmclamb@ncdot.gov |
| Division Environmental Officer | Chad Coggins | 252-237-6164 | tccoggins@ncdot.gov |
| Division Locating Engineer | Brent Bass | 252-334-1118 | bwbass@ncdot.gov |
| Regional Geological Engineer | Dean Argenbright | 252-355-9054 | dargenbright@ncdot.gov |
| Hydraulics Unit | Andy Jordan | 919-707-6729 | pajordan@ncdot.gov |
| Hydraulics Unit | David Holmes |  | dkholmes@ncdot.gov |
| PPU | Virginia Mabry | 919-707-6604 | vmabry@ncdot.gov |
| PPU | Rachel Evans | 919-707-6609 | rcevans@ncdot.gov |

Existing Features

| Feature Bridged: Bridge No. 950156 over Ranyor Mill Branch on SR 1101 (Raynor Mill Road) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ex. Bridge Clear Deck Width: 24.9167 (Ft.) |  |  |  |  |  |  |  | Roadway width: 24 (Ft.) |  |  |  |  |  |
| (Bridge / Culvert) Length: 53 (Ft.) |  |  |  |  |  |  | Deck Width (Out To Out): 25.417 (Ft.) |  |  |  |  |  |  |
| Water Depth: 3 (Ft.) |  |  |  |  |  |  | Height Bed-To-Crown: 20 (Ft.) |  |  |  |  |  |  |
| Year Built: 1960 |  |  |  | Posted SV: 20 |  |  |  |  |  |  | TTST: 29 |  |  |
| Superstructure: reinforced concrete deck girders |  |  |  |  |  | Abutment: EBTS&IBTS: RC caps/ timber piles |  |  |  |  |  |  | Pier Type: |
| Temp. Shored: No |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Historic High Water (Elev. To The Existing Structure ): (Ft) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRI: 26.43882 |  | DP: 2.5 |  |  | SR: 60.41 |  |  |  | School Bussing crossings per day: N/A |  |  |  |  |
| Posted Speed Limit Vicinity: 55 (mph) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Detour | Off-Site |  | Stage Construction |  |  |  |  |  |  | New Alignment |  | On-site |  |
| If Off-Site, Description Of Detour Route: SR 1009 - US 13 - SR 1105 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approximate Length Of Detour? 4.7 Miles |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvements Needed To Road(s) On Detour? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Div. Traffic Eng. Recommend Off-site detour signing? Use existing detour signing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Improvements Needed To Bridges On Detour? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Are future plans for upgrading this roadway either at or in the vicinity of this project? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Are Bridges On Detour Currently Programmed? Yes, B-4941, replace Bridge 950093 on SR 1009 over Town Creek, Let 2019 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Are There EMS Or Business Access Issues? N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Are There Any Railroad Crossings On Detour? No |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Should Work Zone Pedestrian Access Be Maintained During Construction? No |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Utility Lines In Conflict Yes, upstream (telephone) and downstream (power) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power Transmission Lines In Conflict |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Telephone In Conflict Yes, upstream |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cable Lines In Conflict |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fiber Optic In Conflict |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water In Conflict Yes, downstream |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sewer In Conflict |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Natural Gas In Conflict |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other In Conflict |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is There Any Future Utility Construction Anticipated In The Project Area: No |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is A FEMA Buy-Out Property Being Impacted: No |  |  |  |  |  |  |  |  |  |  |  |  |  |

Environmental

| Wetlands At Site: Yes | Comments: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Endangered Species In County: Bald eagle, Red-cockaded woodpecker |  |  |  |  |  |
| T&E species effect: No effect |  |  |  |  |  |
| Trout County: No |  |  | TVA County: No |  |  |
| CAMA County: No |  |  | Primary Nursery Area: No |  |  |
| Moratoria: No |  | Which species: |  |  | Duration: |
| Permits discussion: NWP 3 |  |  |  |  |  |
| Water Quality Classification: C, NSW |  |  |  |  | 303d: No |
| Coast Guard Permit? No |  | Drainage Basin: Neuse |  |  | Riparian Buffer Rules: Yes |
| Is The Project Site In Or Near Any Of The Following: |  |  |  |  |  |
| National Forest: No |  |  | Wildlife Refuge: No |  |  |
| State, County, Or Local Park: No |  |  | Wild And Scenic River: No |  |  |
| Airport: No |  |  | Recreation or Power Generation: No |  |  |
| Water Supply Reservoir: No |  |  | Nutrient Sensitive Waters: Yes |  |  |
| Public Use Boat Ramp: No |  |  | Cemeteries: No |  |  |
| VAD/Farmland Protection: No |  |  | Game Lands: No |  |  |
| Known Or Potential Historic Properties In The Area: No |  |  |  |  |  |
| Architecture: NSR |  |  |  | Archaeology: NSR |  |
| Is The Bridge Structure Itself, Or Any Part Thereof, Considered Historic: No |  |  |  |  |  |
| Impacts to a Church, Community Center, Or Other Public Facility? Piney Grove Church 0.07 miles south of bridge site; no impacts |  |  |  |  |  |
| Is this a Statewide Bicycle Route or a Local Non-Marked Bicycle Route: No |  |  |  |  |  |
| Comments: |  |  |  |  |  |

Geotechnical

Are There Any Historical and/or Vibration Sensitive Structures Near By? No Comments:

Are There Any Known Landfills and/or Geo-environmental Hazard Sites at/or Within Close Proximity To The Project Site? No Comments:

Are Any Impacts Anticipated To Natural Springs Or Artesian Wells? No Comments:

Possible Foundation Type: End Bents: piles Interior Bents: piles **recommend proposed piles be 5’ minimum from existing

Hydraulics

| FEMA Approval? No |  |  |  |  |
| --- | --- | --- | --- | --- |
| Is There Unusual Scour Potential? No (normally) |  | Is Protection Needed? Std |  |  |
| Are Banks Stable? Yes (normally) |  | Is Protection Needed? Std |  |  |
| Appreciable Amount Of Large Debris? Yes |  |  |  |  |
| Placement Of Bents In The Water Be Allowed: Yes |  |  | Where: not within 5’ of water’s edge |  |
| Structure Type: Bridge | If Bridge: CS |  |  | If girder; why? |
| Length Of Structure: 120’ | Min. Number Of Spans: 2 |  |  | Span Arrangement: 2 @ 60’ |
| Waive offset: No | EB Cap: 4'-0" |  |  | Skewed: 90 |

Items To Be Discussed / Resolved At FSM By Attendees

| Off-Site | Stage Construction |  |  |  | New Alignment |  |  |  | On-site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environmental Document Prepared by DEO: Low Impact Data Spreadsheet Minimum Criteria Checklist |  |  |  |  |  |  |  |  |  |
| ADT: 260 per BIR |  | NHS: No |  | Tier: SUBREGIONAL |  |  | Func. Class: Local |  |  |
| Bridge Min. Clear Deck Width Recommended by Div.: 30’ out to out |  |  |  |  |  |  |  |  |  |
| Roadway lane width: 10’ with 3’ shoulders, including 0’ paved shoulders *match existing shoulder |  |  |  |  |  |  |  |  |  |
| Extension of Paved shoulders: N/A |  |  |  |  |  |  |  |  |  |
| Pave to face of guardrail and taper 8:1 to travel lane/paved shoulder? Yes |  |  |  |  |  |  |  |  |  |
| Min. Pavement Surface Course: 3" |  |  |  |  |  | Existing Roadway: 19’ |  |  |  |
| Pavement Marking: Paint |  |  |  |  |  | Pavement Markers: None |  |  |  |
| Will Railroad Involvement Be Required: No |  |  |  |  |  | Method of Clearing: II modified |  |  |  |
| Truck Percentage: 6% per BIR |  |  |  |  |  | Reasonable Safe Speed per SIR: mph |  |  |  |
| Liquidated damages: will be imposed after 120 days of closure/construction. Amt: $500 per day |  |  |  |  |  |  |  |  |  |
| CEI by? NCDOT |  |  | R/W by? DBT |  |  |  |  | R/W monuments? Concrete |  |
| Salvageable materials: No |  |  |  |  |  |  |  |  |  |
| Contact Person/number: |  |  |  |  |  |  |  |  |  |
| Deliver to: |  |  |  |  |  |  |  |  |  |
| Removal of previous structure remnant abutments/piers: No |  |  |  |  |  |  |  |  |  |
| Sequencing priority in construction? Emergency Replacement |  |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |