# NORTH CAROLINA DEPARTMENT OF TRANSPORTATION

# FIELD SCOPING MEETING WORKSHEET

Design Build Process

| TIP No.: | WBS No.: 15006.1026039 | #### Field Scoping Meeting Date: 11/18/16 |
| --- | --- | --- |
| Division: 6 |  | #### Location: via email |
| Route (US/NC/SR): NC 53 |  | County: Cumberland |
| Project Description: Culvert No. 250252 over Indian Creek (Jessups Pond) on NC 53 (Cedar Creek Road) |  |  |
| Tier: SUB REGIONAL |  | Funding: Federal |
| #### Municipality: N/A |  |  |

| Attendees |  |  |  |
| --- | --- | --- | --- |
|  | Name | Phone No. | E-mail |

Existing Features

| Feature Bridged: Culvert No. 250252 over Indian Creek (Jessups Pond) on NC 53 (Cedar Creek Road) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ex. Bridge Clear Deck Width: 0 (Ft.) |  |  |  |  |  | Roadway width: 25’ (22.3’ roadway, left shoulder 1.2’ paved/ 5.5’ unpaved, right shoulder 1.5’ paved/ 5’ unpaved) |  |  |  |  |  |
| Culvert Length (centerline road): 21 (Ft.) |  |  |  |  |  |  | Deck Width (Out To Out): 0 (Ft.) |  |  |  |  |
| Culvert Length (centerline culvert): 50.5 (Ft.) |  |  |  |  |  |  |  |  |  |  |  |
| Water Depth: 4 (Ft.) |  |  |  |  |  |  | Height Bed-To-Crown: 13 (Ft.) |  |  |  |  |
| Year Built: 1955 |  |  |  | Posted SV: Not posted |  |  |  |  |  | TTST: Not posted |  |
| Superstructure: 2@10’x7’ RCBC Historic High Water (Elev. To The Existing Structure ): (Ft) |  |  |  |  |  |  |  |  |  |  |  |
| PRI: 0 |  | DP: 0 |  |  | SR: 88.31 |  |  | School Bussing crossings per day: N/A |  |  |  |
| Posted Speed Limit Vicinity: 55 (mph) |  |  |  |  |  |  |  |  |  |  |  |
| Detour | Off-Site |  | Stage Construction |  |  |  |  |  | New Alignment |  | On-site |
| If Off-Site, Description Of Detour Route: SR 2228(Cumberland) - SR 2023 (Cumberland) - SR 1318 (Bladen) |  |  |  |  |  |  |  |  |  |  |  |
| Approximate Length Of Detour? 18.2 Miles |  |  |  |  |  |  |  |  |  |  |  |
| Improvements Needed To Road(s) On Detour? No |  |  |  |  |  |  |  |  |  |  |  |
| Div. Traffic Eng. Recommend Off-site detour signing? Trailblazing with Street/Road names |  |  |  |  |  |  |  |  |  |  |  |
| Improvements Needed To Bridges On Detour? No |  |  |  |  |  |  |  |  |  |  |  |
| Are future plans for upgrading this roadway either at or in the vicinity of this project? ?SR |  |  |  |  |  |  |  |  |  |  |  |
| Are Bridges On Detour Currently Programmed? No (note: B-5411, replace Bladen County Bridge 080124 on SR 1318, is listed on STIP but was constructed in 2014/2015) |  |  |  |  |  |  |  |  |  |  |  |
| Are There EMS Or Business Access Issues? N/A |  |  |  |  |  |  |  |  |  |  |  |
| Are There Any Railroad Crossings On Detour? No |  |  |  |  |  |  |  |  |  |  |  |
| Should Work Zone Pedestrian Access Be Maintained During Construction? No |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Utility Lines In Conflict Yes, upstream and downstream, power lines cross road at culvert |  |  |  |  |  |  |  |  |  |  |  |
| Power Transmission Lines In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Telephone In Conflict yes, upstream and downstream |  |  |  |  |  |  |  |  |  |  |  |
| Cable Lines In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Fiber Optic In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Water In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Sewer In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Natural Gas In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Other In Conflict |  |  |  |  |  |  |  |  |  |  |  |
| Is There Any Future Utility Construction Anticipated In The Project Area: No |  |  |  |  |  |  |  |  |  |  |  |
| Is A FEMA Buy-Out Property Being Impacted: No |  |  |  |  |  |  |  |  |  |  |  |

Environmental

| Wetlands At Site: Yes | Comments: Agencies consider stream from 48” pipe from dam through culvert JS; the channel created from the breach of the dam is not considered JS. Agencies are anticipating dewatering, regrading of channel from breach in order to construct typical roadway section and rip-rap protection of said roadway section. NWP3 will be acquired through Emergency Permit Protocol where description of project and plans will need to be provided to agencies and construction can begin before permit is received. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Endangered Species In County: American Alligator, Red-cockaded woodpecker, Saint Francis’ satyr butterfly, American Chaffseed, Michaux’s sumac, Pondberry, Rough-leaved loosestrife |  |  |  |  |  |
| T&E species effect: No effect |  |  |  |  |  |
| Trout County: No |  |  | TVA County: No |  |  |
| CAMA County: No |  |  | Primary Nursery Area: No |  |  |
| Moratoria: No |  | Which species: |  |  | Duration: |
| Permits discussion: NWP 3 |  |  |  |  |  |
| Water Quality Classification: C |  |  |  |  | 303d: No |
| Coast Guard Permit? No |  | Drainage Basin: Cape Fear |  |  | Riparian Buffer Rules: No |
| Is The Project Site In Or Near Any Of The Following: |  |  |  |  |  |
| National Forest: No |  |  | Wildlife Refuge: No |  |  |
| State, County, Or Local Park: No |  |  | Wild And Scenic River: No |  |  |
| Airport: No |  |  | Recreation or Power Generation: No |  |  |
| Water Supply Reservoir: No |  |  | Nutrient Sensitive Waters: No |  |  |
| Public Use Boat Ramp: No |  |  | Cemeteries: No |  |  |
| VAD/Farmland Protection: Yes, 0.18 miles north on NC 53, VAD property owned by Wymam A Nichols, Jr and wide |  |  | Game Lands: Yes, Jessups Mill Pond is part of Suggs Mill Pond Game Land, Black Bear Sanctuary, 1212 acres |  |  |
| Known Or Potential Historic Properties In The Area: No |  |  |  |  |  |
| Architecture: NSR |  |  |  | Archaeology: NRHP |  |
| Is The Bridge Structure Itself, Or Any Part Thereof, Considered Historic: No |  |  |  |  |  |
| Impacts to a Church, Community Center, Or Other Public Facility? No |  |  |  |  |  |
| Is this a Statewide Bicycle Route or a Local Non-Marked Bicycle Route: No |  |  |  |  |  |
| Comments: - Jessup Mill Pond is Conservation Property owned by the State of NC; it is part of the Sandhills Area Land Trust. - Within a Cumberland County Soil and Water Conservation District |  |  |  |  |  |

Geotechnical

Are There Any Historical and/or Vibration Sensitive Structures Near By? Comments:

Are There Any Known Landfills and/or Geo-environmental Hazard Sites at/or Within Close Proximity To The Project Site? Comments:

Are Any Impacts Anticipated To Natural Springs Or Artesian Wells? Comments:

Possible Foundation Type: End Bents: Interior Bents:

Hydraulics

| FEMA Approval? No (MOA ) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Is There Unusual Scour Potential? No |  | Is Protection Needed? STD |  |  |
| Are Banks Stable? Yes |  | Is Protection Needed? STD |  |  |
| Appreciable Amount Of Large Debris? No |  |  |  |  |
| Placement Of Bents In The Water Be Allowed: No |  |  | Where: |  |
| Structure Type: Bridge | If Bridge: BB |  |  | If girder; why? |
| Length Of Structure: 90’ | Min. Number Of Spans: 1 |  |  | Span Arrangement: 1@90’ |
| Waive offset: Yes | EB Cap: 4'-0" |  |  | Skewed: 90 |
| - A new channel cut by the breach will require relocation. Embankment armor will be needed. |  |  |  |  |

Items To Be Discussed / Resolved At FSM By Attendees

| Off-Site | Stage Construction |  |  |  | New Alignment |  |  |  | On-site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environmental Document Prepared by DEO: Low Impact Data Spreadsheet Minimum Criteria Checklist |  |  |  |  |  |  |  |  |  |
| ADT: 1000 per CIR |  | NHS: No |  | Tier: SUBREGIONAL |  |  | Func. Class: Major Collector |  |  |
| Bridge Min. Clear Deck Width Recommended by Div.: 33’ OTO |  |  |  |  |  |  |  |  |  |
| Roadway lane width: 12’ with 4’ shoulders, including 2’ paved shoulders |  |  |  |  |  |  |  |  |  |
| Extension of Paved shoulders: end of construction limits and blunt ended |  |  |  |  |  |  |  |  |  |
| Pave to face of guardrail and taper 8:1 to travel lane/paved shoulder? Yes |  |  |  |  |  |  |  |  |  |
| Min. Pavement Surface Course: 3" |  |  |  |  |  | Existing Roadway: 25’ |  |  |  |
| Pavement Marking: Thermoplastic |  |  |  |  |  | Pavement Markers: Raised |  |  |  |
| Will Railroad Involvement Be Required: No |  |  |  |  |  | Method of Clearing: II modified |  |  |  |
| Truck Percentage: 7% per BIR |  |  |  |  |  | Reasonable Safe Speed per SIR: mph |  |  |  |
| Liquidated damages: will be imposed after 60 days of closure/construction. Amt: $1000 per day |  |  |  |  |  |  |  |  |  |
| CEI by? NCDOT |  |  | R/W by? DBT |  |  |  |  | R/W monuments? Pin & Cap |  |
| Salvageable materials: N/A |  |  |  |  |  |  |  |  |  |
| Contact Person/number: |  |  |  |  |  |  |  |  |  |
| Deliver to: |  |  |  |  |  |  |  |  |  |
| Removal of previous structure remnant abutments/piers: Yes |  |  |  |  |  |  |  |  |  |
| Sequencing priority in construction? Emergency Replacement |  |  |  |  |  |  |  |  |  |
| Comments: - dam at Jessup Mill Pond, just upstream of culvert, breached taking out culvert and 300’ roadway |  |  |  |  |  |  |  |  |  |