FINAL SURVEY REPORT									    January 3, 2017

PROJECT No.      17BP.11.R.129

TIP No.                 130262

COUNTY:             CALDWELL

ROUTE:               SR 1107

LS ID No.:

DESCRIPTION:    BRIDGE NO. 262 OVER GUNPOWDER CRREK LAKE ON SR 1107 (FALLS AVENUE)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT GPS MONUMENT

"262-1" WITH A NORTHING OF 756007.1143’ AND AN EASTING OF 1284113.0312’.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99987379.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID G12B) AND BASED ON

NCDOT GPS "262-1" (ELEV 1063.41’)

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (262-1) and runs northeasterly along SR 1107 to station 17+49.65 (262-2).

There is no baseline BY alignment.

The existing centerline was calculated but not staked for Line EL SR 1107.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file.

=========================================================================================

PLANIMETRIC DATA:

Planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 55 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

Underground utilities were encountered within the project limits and are noted as shown in the FS file.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes and sanitary sewer lines were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The apparent utility pole owner is Centel for telephone, and the Town of Granite Falls is the probable water line owner.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A Right-of-Way abstract was provided for this project dated November 16, 2016.  Existing right of way is shown by maintenance limits and mapped at 20’ each side of existing centerline as computed by TGS Engineers for SR 1107.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word 130262_ls_sr.doc

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 130262_ls_fs.dgn

* 130262_ls_dtl.dgn

* 130262_ls_tnl.tin

* 130262_ls_gpk.gpk

* 130262_ls_DeedBook.pdf

* 130262_ls_sr.doc

* 130262_ls_poc.mdb

PROJECT SITE PHOTOGRAPHS

Looking North                                                                        Looking South

Looking West (Upstream)                                                     Looking East (Downstream)

West Elevation                                                                       East Elevation