# NORTH CAROLINA DEPARTMENT OF TRANSPORTATION

# FIELD SCOPING MEETING WORKSHEET

Design Build Process

| TIP No.: | WBS No.: 17BP.11.R.136 | #### Field Scoping Meeting Date: 10/18/16 |
| --- | --- | --- |
| Division: 11 |  | #### Location: On-site |
| Route (US/NC/SR): SR1526 |  | County: ASHE |
| Project Description: Bridge No. 040321 over Helton Creek on SR1526 (John Griffith Road) |  |  |
| Tier: SUBREGIONAL |  | Funding: State |
| #### Municipality: N/A |  |  |

| Attendees |  |  |  |
| --- | --- | --- | --- |
|  | Name | Phone No. | E-mail |
| Division Bridge Program Manager | Jami Guynn | 336-903-9220 | jcguynn@ncdot.gov |
| Division BPM Assistant | Mark Johnson | 336-903-9208 | mgjohnson4@ncdot.gov |
| Division Environmental Officer | Heath Slaughter | 336-903-9202 | hslaughter@ncdot.gov |
| Division L&S | Landon Wagoner | 336-903-9501 | rlwagoner@ncdot.gov |
| Hydraulics Engineer | Marc Shown | 919-707-6751 | mshown@ncdot.gov |
| Hydraulics Engineer | Rusty Lassiter | 919-707-6732 | rlassiter@ncdot.gov |
| PPU | Rachel Evans | 919-707-6609 | rcevans@ncdot.gov |
| PPU | Pam Williams | 919-707-6608 | prwilliams@ncdot.gov |

Existing Features

| Feature Bridged: 040321 over Helton Creek on SR1526 (John Griffith Road) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ex. Bridge Clear Deck Width: 19.667 (Ft.) |  |  |  |  |  | Roadway width: 19 (Ft.) |  |  |  |  |
| (Bridge / Culvert) Length: 72 (Ft.) |  |  |  |  |  | Deck Width (Out To Out): 20.164 (Ft.) |  |  |  |  |
| Water Depth: 2 (Ft.) |  |  |  |  |  | Height Bed-To-Crown: 11 (Ft.) |  |  |  |  |
| Year Built: 1965 |  |  |  | Posted: SV 17 |  |  |  |  | TTST: 21 |  |
| Superstructure: 4 |  |  |  | Abutment: 4 |  |  |  |  | Pier Type: |  |
| Temp. Shored: No |  |  |  |  |  |  |  |  |  |  |
| Historic High Water (Elev. To The Existing Structure ): (Ft) |  |  |  |  |  |  |  |  |  |  |
| PRI: 56.07 |  | DP: 1.06 |  |  | SR: 34.80 |  | School Bussing crossings per day: No response |  |  |  |
| Posted Speed Limit Vicinity: 55 (mph) statutory |  |  |  |  |  |  |  |  |  |  |
| Detour | Off-Site |  | Stage Construction |  |  |  |  | New Alignment |  | On-site |
| If Off-Site, Description Of Detour Route: n/a |  |  |  |  |  |  |  |  |  |  |
| Approximate Length Of Detour? n/a |  |  |  |  |  |  |  |  |  |  |
| Improvements Needed To Road(s) On Detour? No |  |  |  |  |  |  |  |  |  |  |
| Div. Traffic Eng. Recommend Off-site detour signing? Trailblazing with Street/Road names |  |  |  |  |  |  |  |  |  |  |
| Improvements Needed To Bridges On Detour? No |  |  |  |  |  |  |  |  |  |  |
| Are future plans for upgrading this roadway either at or in the vicinity of this project? No |  |  |  |  |  |  |  |  |  |  |
| Are Bridges On Detour Currently Programmed? No |  |  |  |  |  |  |  |  |  |  |
| Are There EMS Or Business Access Issues? Yes, high impact for EMS. |  |  |  |  |  |  |  |  |  |  |
| Are There Any Railroad Crossings On Detour? No |  |  |  |  |  |  |  |  |  |  |
| Should Work Zone Pedestrian Access Be Maintained During Construction? No |  |  |  |  |  |  |  |  |  |  |
| Overhead Utility Lines In Conflict yes, upstream and downstream |  |  |  |  |  |  |  |  |  |  |
| Power Transmission Lines In Conflict |  |  |  |  |  |  |  |  |  |  |
| Telephone In Conflict yes, attached to bridge |  |  |  |  |  |  |  |  |  |  |
| Cable Lines In Conflict yes, attached to bridge |  |  |  |  |  |  |  |  |  |  |
| Fiber Optic In Conflict |  |  |  |  |  |  |  |  |  |  |
| Water In Conflict |  |  |  |  |  |  |  |  |  |  |
| Sewer In Conflict |  |  |  |  |  |  |  |  |  |  |
| Natural Gas In Conflict |  |  |  |  |  |  |  |  |  |  |
| Other In Conflict |  |  |  |  |  |  |  |  |  |  |
| Is There Any Future Utility Construction Anticipated In The Project Area: No |  |  |  |  |  |  |  |  |  |  |
| Is A FEMA Buy-Out Property Being Impacted: No |  |  |  |  |  |  |  |  |  |  |

Environmental

| Wetlands At Site: No | Comments: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Endangered Species In County: Bog turtle, Carolina northern flying squirrel, Northern long-eared bat, BlueRidge goldenrod, Heller's blazing star, Roan mountain bluet, Spreading avens, Swamp pink, Virginia spiraea, Rock gnome lichen |  |  |  |  |  |
| T&E species effect: May affect, but the activity is excepted under the Biological Opinion on NLEB Final 4d Rule: Northern long-eared bat; Gray Bat per USFWS review 10/18/16 no indications of any species of bat use, nor any bats present on the existing structure |  |  |  |  |  |
| Trout County: Yes |  |  | TVA County: No |  |  |
| CAMA County: No |  |  | Primary Nursery Area: No |  |  |
| Moratoria: No |  | Which species: |  |  | Duration: |
| Permits discussion: |  |  |  |  |  |
| Water Quality Classification: C; Tr, ORW, + |  |  |  |  | 303d: No |
| Coast Guard Permit? No |  | Drainage Basin: New |  |  | Riparian Buffer Rules: No |
| Is The Project Site In Or Near Any Of The Following: |  |  |  |  |  |
| National Forest: No |  |  | Wildlife Refuge: No |  |  |
| State, County, Or Local Park: No |  |  | Wild And Scenic River: No |  |  |
| Airport: No |  |  | Recreation or Power Generation: No |  |  |
| Water Supply Reservoir: No |  |  | Nutrient Sensitive Waters: No |  |  |
| Public Use Boat Ramp: No |  |  | Cemeteries: No |  |  |
| VAD/Farmland Protection: No |  |  | Game Lands: No |  |  |
| Known Or Potential Historic Properties In The Area: Historic Mill, southeast quadrant |  |  |  |  |  |
| Architecture: SR |  |  |  | Archaeology: NSR |  |
| Is The Bridge Structure Itself, Or Any Part Thereof, Considered Historic: No |  |  |  |  |  |
| Impacts to a Church, Community Center, Or Other Public Facility? No |  |  |  |  |  |
| Is this a Statewide Bicycle Route or a Local Non-Marked Bicycle Route: No |  |  |  |  |  |
| Comments: -Within Helton Historic District; Helton Roller Mill (southeast quadrant) on state historic study list |  |  |  |  |  |

Geotechnical

Are There Any Historical and/or Vibration Sensitive Structures Near By? Comments:

Are There Any Known Landfills and/or Geo-environmental Hazard Sites at/or Within Close Proximity To The Project Site? Comments:

Are Any Impacts Anticipated To Natural Springs Or Artesian Wells? Comments:

Possible Foundation Type: End Bents: Interior Bents:

Hydraulics

| FEMA Approval? Detailed Study (MOA ) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Is There Unusual Scour Potential? No |  | Is Protection Needed? STD |  |  |
| Are Banks Stable? Yes |  | Is Protection Needed? STD |  |  |
| Appreciable Amount Of Large Debris? No |  |  |  |  |
| Placement Of Bents In The Water Be Allowed: No |  |  | Where: |  |
| Structure Type: Bridge | If Bridge: CS |  |  | If girder; why? |
| Length Of Structure: 70’ | Min. Number Of Spans: 1 |  |  | Span Arrangement: 1 @ 70’ |
| Waive offset: yes, both ends | EB Cap: n/a |  |  | Skewed: yes |
| -vertical abutments -Alaska rail |  |  |  |  |

Items To Be Discussed / Resolved At FSM By Attendees

| Off-Site | Stage Construction |  |  |  | New Alignment |  |  |  | On-site |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment: West (upstream) |  |  |  |  |
| Environmental Document Prepared by DEO: Low Impact Data Spreadsheet Minimum Criteria Checklist |  |  |  |  |  |  |  |  |  |
| ADT: 100 per SIR |  | NHS: No |  | Tier: SUBREGIONAL |  |  | Func. Class: Local |  |  |
| Bridge Min. Clear Deck Width Recommended by Div.: 30’ out to out |  |  |  |  |  |  |  |  |  |
| Roadway lane width: 9’ with 3’ shoulders, including 0’ paved shoulders |  |  |  |  |  |  |  |  |  |
| Extension of Paved shoulders: n/a |  |  |  |  |  |  |  |  |  |
| Pave to face of guardrail and taper 8:1 to travel lane/paved shoulder? Yes |  |  |  |  |  |  |  |  |  |
| Min. Pavement Surface Course: 2½" |  |  |  |  |  | Existing Roadway: 14’ |  |  |  |
| Pavement Marking: None |  |  |  |  |  | Pavement Markers: None |  |  |  |
| Will Railroad Involvement Be Required: No |  |  |  |  |  | Method of Clearing: II modified |  |  |  |
| Truck Percentage: 6% per BIR |  |  |  |  |  | Reasonable Safe Speed per SIR: 20 mph |  |  |  |
| Liquidated damages: will be imposed after 120 days of closure/construction. Amt: $500 per day |  |  |  |  |  |  |  |  |  |
| CEI by? NCDOT |  |  | R/W by? DBT |  |  |  |  | R/W monuments? Pin & Cap |  |
| Salvageable materials: ALL STEEL I-BEAMS (18) 12" I-BEAMS AND (9) 15" I-BEAMS |  |  |  |  |  |  |  |  |  |
| Contact Person/number: CHAD COX 336-384-2562 - OFFICE 336-877-6481 - CELL |  |  |  |  |  |  |  |  |  |
| Deliver to: WARRENSVILLE BRIDGE MAINTENANCE YARD - 322 NORTHWEST SCHOOL ROAD, Warrensville, NC 28693 |  |  |  |  |  |  |  |  |  |
| Removal of previous structure remnant abutments/piers: No |  |  |  |  |  |  |  |  |  |
| Sequencing priority in construction? No |  |  |  |  |  |  |  |  |  |
| Comments: -resurface NC 194 (-Y-) 100’ either direction from proposed -L- centerline -pull existing concrete piers; remove abutments and rip rap with standard bank stablization (rip rap should match color of stone in stream) -The property owned by Mr. George Roe driveway shall be sufficient width to accommodate equipment and vehicles utilized by his business if relocated. His drive is used to haul heavy grading equipement . His driveway and home are south west of the bridge. He has voiced concerns with moving the bridge upstream and what his drive connection would be. |  |  |  |  |  |  |  |  |  |