FINAL SURVEY REPORT									    January 11, 2017

PROJECT No.      17BP.11.R.134

TIP No.                 050001

COUNTY:             AVERY

ROUTE:               SR 1349

LS ID No.:

DESCRIPTION:    BRIDGE NO. 1 OVER LINVILLE RIVER ON SR 1349 (RUFFIN STREET)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(CORS96) STATE PLANE COORDINATES FOR THE NCGS MONUMENT

"MARJONS 1971" WITH A NORTHING OF 882813.127’ AND AN EASTING OF 1172749.269’.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99986638.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID 09) AND BASED ON NCDOT MONUMENT “BL-1” (ELEV 3632.08’)

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (BL-1) and runs northerly along SR 1349 to station 9+40.43 (BL-3).

There is no baseline BY alignment.

The existing centerline was calculated but not staked for Line EL SR 1349, Line EY HWY 221 & Line EY1 SR 1342.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file.

=========================================================================================

PLANIMETRIC DATA:

Planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 60 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

Underground utilities were encountered within the project limits and are noted as shown in the FS file.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The owner of the poles within the project limits is unknown.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A Right-of-Way abstract was provided for this project dated November 9, 2016.  Existing right of way is shown by maintenance limits and mapped at 17.5’ each side of existing centerline as computed by TGS Engineers for SR 1349 to a point then by property line to the end of the limits.  Right of way is shown by recorded agreements for US Highway 221.  Right of way is shown by evidence noted in the FS file for SR 1342 as property lines and recorded agreement.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word 050001_ls_sr.doc

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 050001_ls_fs.dgn

* 050001_ls_dtl.dgn

* 050001_ls_tnl.tin

* 050001_ls_gpk.gpk

* 050001_ls_DeedBook.pdf

* 050001_ls_sr.doc

* 050001_ls_poc.mdb

PROJECT SITE PHOTOGRAPHS

Looking North                                                                        Looking South

Looking West (Downstream)                                                 Looking East (Upstream)

West Elevation                                                                       East Elevation