FINAL SURVEY REPORT									    January 23, 2017

PROJECT No.      17BP.11.R.127

TIP No.                 050079

COUNTY:             AVERY

ROUTE:               SR 1341

LS ID No.:

DESCRIPTION:    BRIDGE NO. 79 OVER ELK RIVER ON SR 1341 (BANNER CREEK RD.)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT MONUMENT

"79-2" WITH A NORTHING OF 884748.750’ AND AN EASTING OF 1155347.528’.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99982294.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID 12B) AND BASED ON NCDOT MONUMENT “79-2”

(ELEV 3684.32’)

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (BL-3) and runs northeasterly along SR 1341 and NC 184 to station 13+87.54 (79-2).

The existing centerline was calculated but not staked for Line EL SR 1341 and Line EY NC 184.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file.

=========================================================================================

PLANIMETRIC DATA:

Planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 60 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

NC 811 designated underground utilities.  Underground utilities were encountered within the project limits and are noted as shown in the FS file.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The owner of the poles within the project limits is not apparent.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A Right-of-Way abstract was provided for this project dated November 2, 2016.  Existing right of way is shown by virtue of recorded right of way agreements on SR 1341 dedicated as 22.5’ left and right through the limits per project 6.722185.  One parcel was not acquired, therefore 16’ maintenance limits are shown through this location.  NC Highway 184 was mapped as 17’ each side of centerline by virtue of maintenance.  The southeast quadrant at the intersection of NC Highway 184 and SR 1341 has a 150’ X 50’ Sight Distance as per the abstract.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word 050079_ls_sr.docx

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 050079_ls_fs.dgn

* 050079_ls_dtl.dgn

* 050079_ls_tnl.tin

* 050079_ls_gpk.gpk

* 050079_ls_DeedBook.pdf

* 050079_ls_sr.doc

* 050079_ls_poc.mdb

PROJECT SITE PHOTOGRAPHS

Looking North                                                                        Looking South

Looking West (Downstream)                                               Looking East (Upstream)

East Elevation                                                                       West Elevation