FINAL SURVEY REPORT									    January 20, 2017

PROJECT No.      17BP.11.R.138

TIP No.                 050117

COUNTY:             AVERY

ROUTE:               SR 1127

LS ID No.:

DESCRIPTION:    BRIDGE NO. 117 OVER HENSON CREEK ON SR 1127 (LITTLE HENSON CREEK RD.)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(NA 2011) STATE PLANE COORDINATES FOR THE NCDOT MONUMENT

"050117-1" WITH A NORTHING OF 843333.258’ AND AN EASTING OF 1102733.187’.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99982867.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID 12B) AND BASED ON NCDOT MONUMENT “050117-1”

(ELEV 2944.16’)

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (BL-3) and runs easterly along SR 1127 to station 8+05.77 (050117-1).

Baseline BY begins at station 5+00 (BY-4) and runs southeasterly along SR 1126 to station 10+04.70

(050117-2).

The existing centerline was calculated but not staked for Line EL SR 1127 and Line EY SR 1126.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file.

=========================================================================================

PLANIMETRIC DATA:

Planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 70 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

NC 811 designated underground utilities.  Underground utilities were encountered within the project limits and are noted as shown in the FS file.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The owner of the poles within the project limits appears to be AT&T.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A Right-of-Way abstract was provided for this project dated October 25, 2016.  Existing right of way is shown by maintenance limits and mapped at 17’ each side of centerline along SR 1127 (Little Henson Creek Road) which is Line –EL-. SR 1126 (Henson Creek Road) shown as line –EY- in the FS file was abstracted as 17’ each side of centerline.  The above two distances were mapped per the right of way abstract received.  At the intersection of SR 1126 & SR 1127, the pavement flares out to a width that is outside of the abstracted distances and the recommendation suggests a variable width maintenance.  Therefore, TGS Engineers shows return radii that fit the existing maintenance offsets for the two roads as abstracted.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word 050117_ls_sr.docx

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 050117_ls_fs.dgn

* 050117_ls_dtl.dgn

* 050117_ls_tnl.tin

* 050117_ls_gpk.gpk

* 050117_ls_DeedBook.pdf

* 050117_ls_sr.doc

* 050117_ls_poc.mdb

PROJECT SITE PHOTOGRAPHS

Looking East                                                                           Looking West

Looking South (Downstream)                                               Looking North (Upstream)

North Elevation                                                                      South Elevation