FINAL SURVEY REPORT									    January 13, 2017

PROJECT No.      17BP.11.R.137

TIP No.                 940093

COUNTY:             WATAUGA

ROUTE:               SR 1109

LS ID No.:

DESCRIPTION:    BRIDGE NO. 93 OVER LAUREL FORK ON SR 1109 (BAIRDS CREEK ROAD)

=========================================================================================

PERSONNEL:

NCDOT

WILKESBORO FIELD OFFICE

PO BOX 250

801 STATESVILLE ROAD

WILKESBORO, NC 28659

(336)903-9501

LARRY R. ABSHER

PRIVATE ENGINEERING FIRM

TGS ENGINEERS – Firm License Number C-0275

804-C NORTH LAFAYETTE STREET

SHELBY, NORTH CAROLINA 28150

(704)476-0003

LARRY D. SCISM, PLS

=========================================================================================

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE

NAD 1983(CORS96) STATE PLANE COORDINATES FOR THE NCGS MONUMENT

"MARJONS 1971" WITH A NORTHING OF 882813.127’ AND AN EASTING OF 1172749.269’.

THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.99986638.

ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD88 (GEOID 09) AND BASED ON NCDOT MONUMENT “BY7-69” (ELEV 2841.24’) ESTABLISHED ON PROJECT R-2566B.

=========================================================================================

ALIGNMENTS AND BENCHMARKS:

Baseline BL begins at station 5+00 (BY7-69) and runs northerly along SR 1109 through BL-1 to station 11+01.28 (BL-2).

There is no baseline BY alignment.

The existing centerline was calculated but not staked for Line EL SR 1109 and Line EY Arvil Greer Road.

Elevations were obtained for each baseline control point and one benchmark was established.

Baseline control data transmitted in the graphics FS file.

=========================================================================================

PLANIMETRIC DATA:

Planimetric mapping and field classification by TGS Engineers. Data portrayed in the graphics FS file.

=========================================================================================

DTM DATA:

The DTM data collected and compiled by TGS Engineers. Data portrayed in the graphics DTL file and the binary TIN file.

Dissolve Option – Side

Side Length – 60 feet

Minimum Linear Distance – 30 feet

Arc Stroke Tolerance – 0.15 feet

=========================================================================================

NON-GRAVITY UTILITY DATA:

Underground utilities were encountered within the project limits and are noted as shown in the FS file.  A telephone line is attached to the upstream side (east) of the bridge.

=========================================================================================

GRAVITY UTILITIES AND PIPE INVERTS:

Gravity storm water pipes were encountered within the survey limits.

Location, size, material type and elevations were obtained and transmitted graphically in the FS file.

=========================================================================================

POLE DATA:

Utility poles within project limits were classified and information documented and transmitted graphically in the FS file.

The owner of the poles within the project limits is BREMCO for power and Skyline for telephone.

=========================================================================================

PROPERTY AND EXISTING RIGHT OF WAY:

Deed research was performed by TGS Engineers.

Deed copies were scanned and transmitted in the Adobe Deed.pdf file.

Property lines were established based upon surveyed and located field evidence and recorded documents.

A Right-of-Way abstract has not yet been provided.  Right of way was mapped as 30’ left and right of centerline (60’ Total) per abstract on project R-2566B received in 2012 per recorded agreements.

Property mapping was completed and transmitted in the graphics FS file.

=========================================================================================

SITE PHOTOGRAPHS:

Photographs of the existing bridge, roadway and stream were taken and transmitted in the Microsoft Word 940093_ls_sr.doc

=========================================================================================

THE FOLLOWING FILES WERE TRANSMITTED TO NCDOT:

* 940093_ls_fs.dgn

* 940093_ls_dtl.dgn

* 940093_ls_tnl.tin

* 940093_ls_gpk.gpk

* 940093_ls_DeedBook.pdf

* 940093_ls_sr.doc

* 940093_ls_poc.mdb

PROJECT SITE PHOTOGRAPHS

Looking North                                                                 Looking South

Looking West (Downstream)                                          Looking East (Upstream)

West Elevation                                                                       East Elevation