Final Survey Report

october 2017

TIP u-2719

LS# 05-16-223

pROJECT# 35869.1.2

wake COUNTY

Description:

gel

i-440/us 1 south of sr 1313 (walnut street) to sr 5039 (north of jones franklin road).

stantec

I-440 (CLIFF BENSON BELTLINE) FROM NORTH OF JONES FRANKLIN ROAD

TO NORTH OF LIGON STREET.

esp

APPROXIMATELY 1500' SOUTH OF HILLSBOROUGH STREET TO APPROXiMATELY 2500' NORTH OF LAKE BOONE TRAIL.

# Alignment

Baseline –BL- Beginning Station:		u-2719-39, Station 5+00.00

Baseline Alignment Design File:		u-2719_NCDOT_FS.dgn

BEST-FIT ALIGNMENT DESIGN FILE:		u-2719_NCDOT_FS.DGN

PROJECT .GPK FILE:				u-2719_LS_GPK.GPK

HORIZONTAL DATUM:				NAD 83/2011

MONUMENT USED FOR PROJECT CONTROL:    NCDOT   u-2719 az-1

NORTHING		EASTING                   ELEVATION

741380.671 (ft)        2083505.762 (FT)               485.723’  (NCDOT)

AVERAGE COMBINED FACTOR:		0.99990018  (1/x=1.00009983)

VERTICAL DATUM:				NAVD 88

safety and traffic control

there were many problems with signage on this project.  Stay alert was contracted to provide work zone perminant signs to avoid having to put up and take down signs every day.  stay alert was several weeks late in installing these signs which required us to put up our own until the permanent signs were installed.  once the signs were installed they did not meet the quality needed for a safe working environment so we had to pull crews from the project until the issues were resolved.  signs were leaning over, signs were installed at angles not perpendicular to roadway, signs were not in proper location with the proper message, and signs were installed behind tree limbs were they were obstructed enough where the traveling public could not read them.  once all the issues were resolved the use of permanent signs did tremendously help with the flow of the project.

# EXISTING ALIGNMENT DATA

u-2719_NCDOT_FS.DGN

THE EXISTING ALIGNMENT WAS GENERATED BY A BEST FIT OF THE EXISTING CENTERLINE by the scan firm used for this project.

# UTILITY DATA

U-2719_NCDOT_FS.DGN

GRAVITY UTILITIES PRESENT AT THE SITE WERE LOCATED BY CONVENTIONAL METHODS.  the structures located at the i-40/i440 intersection were “new construction” structures.  all other structures not located in this area are currently under construction and were not located.

there were multiple storm sewer structures in the western blvd. area that could not be fully surveyed due collaspe of slab tops and being full of debris.  there were safety concerns at these locations and ncdot, wake county and city of raleigh were all notified.

# PlANIMETRIC - PHOTOGRAMMETRIC DATA

phOTOGRAMETRIC DATA was classified and supplemented by NCDOT USING CONVENTIONAL SURVEY METHODS.

# PROPERTY DATA

U-2719_NCDOT_FS.DGN

THIS FILE INCLUDES EXISTING RIGHT OF WAY and PROPERTY LINES. deed/plat books were prepared by GEL ENGINEERING, stantec and esp.  there was no r/w abstract provided for this project by ncdot.  the property and r/w was computed using a combination of existing monumentation and deed/plat info along with existing alignment data.

U2719_gel_LS_POC.MDB

u2719_stantec_ls_poc.mdb

u2719_esp_ls_poc.mdb

THeSe FILEs CONTAIN PROPERTY OWNER  INFORMATION AND CONTACTS.  the survey limits for this project was changed several times over the duration of this project so the amount of property owners in this file does not represent the number of property owners that are included in the fs file.  there should be more owners in the mdb than inside the project limits due to the changing limits.

# DTM’S

DTM GRAPHIC FILE:                                 u-2719_ls _DTM.dgn

DTM TIN FILE:                                            u-2719_ls _TIN.TIn

PAVEMENT DTM’S were not in GEL’s submitted dtm file.  pavement dtm’s were  added to gel’s and esp’s existing files by stantec.  the tin file submitted by gel is only to be used to check obscured coverage and not used for final design.  a final tin is submitted by stantec that represents merged final pavement dtms.  on 5/9/17 we received confirmation from eddie green that ql2 lidar was acceptable to use in obscure areas that fell behind the sound wall on us 1/64 because any construction would not fall behind the sound walls.  on 6/14/17 we received confirmation that we were to get check shots on sidewalks to ensure that photogrammetry sidewalks were within tolerance.  the check shots can be found in the gpk file submitted.

dtm settings

curve stroke tolerance:  0.15

Minimum linear distance:  30.0

dissolve option:  side

side length:  150.0

# HYDROGRAPHIC DATA

u-2719_NCDOT_FS.DGN

HYDRO DATA WAS COLLECTED AS PER NCDOT HYDRO GUIDELINES.  multiple attempts were made to locate pipes marked “could not locate” in the fs file.

# POLE DATA

u-2719_LS_PDF.TXT

POLE DATA WAS OBTAINED BY GEL ENGINEERING, stantec and esp

# SUE DATA

u-2719_NCDOT_FS.DGN

NON GRAVITY UTILITIES PRESENT WERE COLLECTED UNDER A DIFFERENT ASSIGNMENT however the underground utilities have been merged into the submitted fs file.

# BLN DATA

u-2719_LS_baseline.TXT

baseline DATA WAS OBTAINED BY GEL ENGINEERING, stantec and esp and approved by ncdot.

# COMMENTS

THIS IS AN ENGLISH PROJECT.

scale 1” = 50’

NO property strip map was prepared

gel CREW

PC                 michael green

TRANSIT     sYED qUIADRI

brain meilinger

scott trowbridge

ROD             WILLIE MILLER

MICHAEL GREEN

TRAVIS CONN

brian meilinger

sammy short

scott trowbridge

jack eddy

harold burgos

CADD OPERATOR(S)

GEORGE HART

Stantec

TOM BURNS

MICHAEL HELMUTH

DAN GUNNOE

DANIEL ROBERTSON

DAN GROVE

MARK LEWIS

KENNETH HAGGERTY

COLIN DAWES

esp

A.D. KEANE, PLS		SURVEY DEPARTMENT MANAGER

T.E.S. GIBBS, PLS		SURVEY PROJECT MANAGER