August 13, 2012

MEMORANDUM TO:	Jay A. Bennett, P.E.

State Roadway Design Engineer

ATTENTION:	Brenda L. Moore, P.E.

Roadway Design Project Engineer

### FROM:	Njoroge W. Wainaina, P.E.

State Geotechnical Engineer

STATE PROJECT:	38439.1.1 (B-4621)

FEDERAL PROJECT:	BRNHS-220B (4)

### COUNTY:	Rockingham

DESCRIPTION:	Bridge No. 150 over US 220 Business on US 220

SUBJECT:	Geotechnical Report – Design and Construction Recommendations

The Geotechnical Engineering Unit has completed the subsurface investigation for this project and submits the following recommendations.  No plans will be submitted

### I.	Slope and Embankment Stability

A.	Slope Designs

Recommend that all embankment slopes be constructed at a ratio of 2:1 (H:V) or flatter.

- Undercut (Soft Foundation Soils)

Recommend 100 cubic yards of undercut for embankment stability as a contingency item to be used at the discretion of the Engineer.

- Geotextile for Soil Stabilization

A quantity of 100 square yards of geotextile for soil stabilization should be included in the project contract as a contingency.

- Subgrade Stability

- Undercut for Subgrade Stability

A quantity of 100 cubic yards of undercut is recommended for inclusion in the contract as a contingency item, to be used in areas for undercut at the discretion of the Engineer.

- Aggregate Subgrade

A quantity of 100 cubic yards of shallow undercut is recommended for inclusion in the contract as a contingency item, to be used at the discretion of the Engineer.

A quantity of 200 tons Class IV material is recommended for inclusion in the contract as a contingency item, to be used at the discretion of the Engineer.

A quantity of 300 square yards of Geotextile for Soil Stabilization is recommended for inclusion in the contract to be used at the discretion of the Engineer.

- Geotextile for Soil Stabilization

Recommend a quantity of 100 square yards of Geotextile for Soil Stabilization is recommend for inclusion in the contract as a contingency item to be used at the discretion of the Engineer.

- Subsurface Drainage - Underdrain

### Recommend a quantity of 50 linear feet of 6-inch perforated corrugated plastic pipe be included in the project contract as a contingency item to be used at the discretion of the Project Engineer.  Construction of underdrains shall follow Standard Specifications, Section 815, “Subsurface Drainage”. Include Roadway Standard Drawing 815.03, “Pipe Underdrain and Blind Drain”.

### III.	Borrow Specifications

- Borrow Criteria

Common borrow for embankment construction to subgrade shall meet Statewide criteria outlined in the Standard Specifications, Article 1018-2(A).

### B.	Select Granular Material

Select Granular Material for embankment construction on geotextile for soil stabilization and/or backfill in water shall meet the criteria outlined in Standard Specifications, Article 1016-3 Class II or III.  Include 200 cubic yards of this material in the project contract as a contingency item.  The backfill material should be placed to a height of three (3) feet above geotextile for soil stabilization or water level.

### C.	Shrinkage Factor

Recommend a 20% shrinkage factor be used for earthwork calculations.

IV.	Miscellaneous

A.	Reduction of Unclassified Excavation - Clearing and Grubbing

The reduction of unclassified excavation available for use on the project is anticipated to be negligible.

Prepared by, 						Prepared by,

David L. Teague, P.E. 				Christina M. Bruinsma, L.G.

Geotechnical Design Engineer			Project Geological Engineer

### NWW/DLT/cmb

Summary of Quantities

| # Item Number | # Description | # Quantity | # Comment |
| --- | --- | --- | --- |
| 0036000000-E | Undercut Excavation | 200 CY | See Section I. B., Section II. A. |
| 0038000000-E | Shallow Undercut | 100 CY | Section II. B. |
| 0196000000-E | Geotextile for Soil Stabilization | 500 SY | See Section I. C. and Section II. B. & C. |
| 0195000000-E | Select Granular Material | 200 CY | See Section III. B. |
| 0080000000-E | Class IV Subgrade Stabilization | 200 TONS | See Section II. B. |
| 2044000000-E | 6” Perforated Subdrain Pipe | 50 LF | See Section II. D. |
| These only impact Earthwork totals |  |  |  |
|  | Reduction of Unclassified – Clearing and Grubbing | N/A | See section IV. A. |
|  | Shrinkage Factor | 20% | See section III. C. |