U-5818 DRAINAGE REDLINE & CSR REVIEW

The below comments represent a review of the provided U-5818 Drainage Redlines (PSH-4-7) as well as provided storm drainage calculations, CSRs and HECRAS.

Comments are provided for each of the following:

- General Project Comments

- Specific Plan Sheet Comments, including comps

- General CSR Comments

- Specific CSR Comments

- HECRAS Comments

This review does not represent a detailed review or acceptance of every aspect of the drainage design, nor is it it’s intent. The full accuracy and completeness of the hydraulic design and its calculations are still the sole responsibility of the Designer of Record.

I. GENERAL PROJECT COMMENTS

- Does the Hydraulic Data for the Existing Culverts need to be shown on the Profiles?

- For all existing Box Culverts – label ‘Retain” or “Retain & Extend”.

- The project is a minor collector, curb and gutter with a design speed of 50 mph. Confirm that all the spreads meet the criteria in the Table 10-1 in the latest NCDOT 2016 Guidelines.

- If paved/conc ditches are used behind retaining walls, make sure any needed proposed drainage is accounted for.

- DDE may be required for the several ditches shown leaving the highway and that are not shown on the cross sections.

- Confirm that all ditch slopes shown in the profiles correspond to the correct slope based on number of decimal places indicated for the % slope.

II. SPECIFIC PLAN SHEET COMMENTS, INCLUDING COMPS

SHEET 4

- Y 459+00 RT – Label “Retain” notes on existing system.

- Y 459+50 RT – It’s unclear if this is a ditch, expressway gutter, etc.; the cross sections are unclear as well.

- Y 459+00 RT – The ditch calculation appears to show you are at the limit of maximum allowable shear stress; confirm and line if necessary.

- Y 465+00 LT – Due to the additional widening thru this stretch, is SBG now warranted?

- Y 465+00 LT – The new fill slope appears to be an encroachment in the floodplain of the adjacent stream. Is any slope protection needed near the toe? Does the slope stake affect the water surface elevations of the stream, outside the R/W?

- RP C 16+50 RT – May need additional easement to cover the lateral 3 ft base ditch near the culvert inlet extension.

- RP D 13+50 LT – Str 0419 to Str 0420 – May need to call for Elbows.

- RP D 12+70 – At the ends of pipes to be filled with flowable fill, you might want to consider calling for a “plug” as well.

- RP D 14+00 RT – Will the ditch formed between the 2 ramps drain to Str 0418, or is a “grade to drain” note needed?

- L 11+75 RT – Str 0409 needs to be labeled “2-GI-D”. The depth appears to be that of a “D”.

- L 12+30 LT – Is this some sort of proposed retaining wall between the 2 box culverts? Any constructability issues or concerns with the base of the wall so near the stream? Will a scour analysis be required for the wall, due to proximity to the stream? Were 1.5:1 rip rap plated slopes considered thru this area?

SHEET 5

- L 24+00 LT – Were 1.5:1 rip rap plated slopes considered in this area? This appears to be an extremely long wingwall or is it a proposed retaining wall? With the base of the wall located in the stream determine if a scour analysis is warranted or not. Is a paved/conc ditch needed on the roadway side of the top of wall?

- Y 475+50 RT – The proposed slope stake appears to cover over the end of the existing 30” CMP. Does it need to be extended?

- L 28+25 LT – Str 0518 to Str 0519 – you are bringing a proposed pipe under the proposed bridge fill and then under/thru a proposed retaining wall. Consider revising layout to a design that would bring the system in front of the retaining wall, rather than behind/thru it. You would still tie to the existing storm drain – but in front of wall rather than behind. Then remove or fill with flowable fill the portion of existing pipe left under the proposed bridge fill.

- RP B 11+50 LT – There appear to be existing rip rap check dams in the ditch. Are these permanent ditch checks for water quality? If so, may want to consider labeling “Retain” on the 3 shown in this area.

- L 30+00 to 30+50 – on Profile – Proposed slope shown appears to be off.

- L 32+50 LT – Str 0521 to Str 0522 – are elbows needed?

- Y 482+50 RT – Is outlet protection needed at the end of the pipe extension?

- Y 479+00 RT – Is any proposed drainage needed in front of and at the end of the face of proposed retaining wall?

- Y 481+00 RT – The existing contours may be indicating a possible migrating stream bank, outside bend and headed towards the roadway.  Was this observed in the field and confirmed as well?  If so, with the additional widening occurring in this area is any slope protection warranted?

SHEET 6

- Y 37+00 LT – I would place a note in the Remarks column for Str 0603, stating that the drainage area and resulting spread is based off the area for the entire existing drainage system north of here. This would explain why the spread is so bad as documented.

SHEET 7

- No Comments

III. GENERAL CSR COMMENTS

- The following items are listed in the latest 2016 Drainage Guidelines, which could be considered for incorporation:

    - Under Recommended Structure on Front – they say to show Length of extension.

    - Under Data on US and DS Structures – they say to show distance to the US and DS structures if less than 1000 feet away.

    - Under Floodplain Profile – they say to show PI Data for roadway.

    - Under Floodplain Profile – they say to show normal WS elevation.

    - Under Plan View – they say to provide land use descriptions in all 4 quadrants.

    - Under Front of CSR – need to show Longitude and Latitude in provided spaces.

    - Under Plan View – show name of stream in Plan.

IV. SPECIFIC CSR COMMENTS

CSR 11+57.95 L

- Front of CSR – Confirm that this SR Road is Statewide Tier.

- Back of CSR – Allowable HW – since you don’t know how the array of upstream systems are affected by this Box Culvert’s hydraulic performance, you might want to consider revising the allowable HW to the “Existing 100 YR”.

- Back of CSR – Under Required Outlet Protection – since you aren’t showing any rip rap in the stream bed, per plan; maybe the note needs to be revised to include “Banks Only”; if that is the intent.

- Back of Report – Under the HDS-5 Calculations for the existing box culvert – it appears that the 500yr Tailwater is actually controlled by the downstream backwater of the existing box culvert under RP C. Confirm and revise if needed.

- Inside on Roadway Template View – the Retain and Extend note says for a 7’x6’ culvert size; revise since this is a 6’x6’.

- Inside in Plan View – show any additional Easement or R/W needed for outlet channel.

- Inside – Need to include Details and Quantities for Outlet Channel Improvements.

CSR 16+31.90 RP C

- Back of CSR in Design Table – confirm correct rounding of all numbers taken from HECRAS.

- Inside in Plan View – there is a radius at the culvert outlet end on the roadway shoulder; confirm this shifted shoulder catches behind existing tip of wing in the radius – looking down stream, right side of culvert.

- Inside – Need to include Details and Quantities for the Inlet Channel Improvements.

- Inside on Roadway Template View – need to include the Size Culvert and Retain and Extend note – as part of the summary data at the top of the view.

- Inside on the Performance Table – double check the Natural elevations shown; they appear to have been taken at a different section than the Existing and Proposed elevations.

- Inside on the Performance Table – double check the rounding of all numbers taken from HECRAS.

CSR. 25+49.32 L

- Front of CSR – Confirm that this SR Road is Statewide Tier.

- Back of CSR – With your design, you exceeded your Allowable HW; therefore, maybe “Allowable” is truly more than what you stated was allowed; since you are OK exceeding it. You could consider changing what is shown as “allowable”.

- Back of CSR – Finish populating “approx. ?? feet upstream” in the Design Table.

- Back of CSR in Design Table – The outlet control numbers shown appear to be those from Section 1102 and not 913 as stated. Confirm and revise if necessary.

- Back of CSR in Design Table – Populate the HW column under Outlet Control.

- Inside Report - Need to include Details and Quantities for the Inlet Channel Improvements – if you intend to do anything here.

V. HECRAS COMMENTS

- Could consider upgrading to HECRAS 5.0.3, since NCDOT Hydraulics is now accepting this Version.

- 16+31.90 RP C Culvert – For the Existing Model – you used a Ke=0.2 and an Inlet Chart of 8-1. Looking at the inlet photo it appears you have something closer to a square top in the opening, and not a bevel. Confirm that you are using the correct Ke.

- You could consider adding XS 913 and XS 2088 in the Existing Model as well – then you don’t have to add any cross sections – between Existing and Proposed models. This would also allow you to make the WS comparisons in the CSRs closer to the Culvert Faces.

- 25+39.42 L Culvert – For the Existing Model -  you used a Ke=0.2 and an Inlet Chart of 8-2. Confirm that you are using the correct Ke. Did the existing box culvert have a small chamfer or the newer larger Beveled Inlet?

- Why did you use a culvert concrete mannings n value of 0.013 for one box culvert and then 0.015 for the other box culvert?

- For the 25+39.42 L Culvert – Confirm you are modeling the proposed culvert with 0.75 foot of blockage. It would be beneficial to add this to the narrative, to state what you did and why.

- Check XS 1518 in both the existing and proposed models – you appear to have resulting water surface elevations that are flip flopping – ie 50 yr shown below 25 yr, etc.