NCDOT Final Survey Report for Bridge U-5818

February 29, 2016

Project No.:  44390.1.1

TIP No.:  U-5818

County:  McDowell

LS No.:  13-15-101

Job ID No.:  u5818-con-wet

Project Description:  SR 1001 (Sugar Hill Rd.) and I-40 Ramps

Survey limits:  Limits are defined by a design file provided by the Roadway Design Department of Wetherill Engineering named “U5818 SURVEY LIMITS_REV.dgn” and contain approximately 3900’ along SR 1001 (Sugar Hill Rd.), 3200’ along I-40, all entrance and exit ramps for I-40 at the Sugar Hill Interchange (totaling 1950’) and 1900’ along the service road (Condrey Rd.).  This is an English project and will be mapped at a 1”=50’ Scale in Microstation using NCDOT Location & Surveys Workspace.

Date of Survey:  Field work was performed November 2015 - February 2016

Surveyed by:  Wetherill Engineering, Inc.

Control:  Control for this project was established by NCDOT.

Control for this project is North Carolina 3200 State Plane Coordinates NAD83/NA2011 Horizontal Datum and NAVD88 Vertical Datum.

Six Benchmarks were established on this project by Wetherill Engineering.

Wetherill made the Baseline text file from the control provided by NCDOT.

Utilities:  GEL Engineering painted, surveyed and mapped the SUE for this project.  Wetherill merged the SUE data into the FS design file.

DTM (Digital Terrain Modeling):  NCDOT Photogrammetry provided DTM’s for this project.  ESP provided mobile scan mapping for the pavement and beginning/end of ramps for I-40.  Wetherill collected and mapped pavement features (except I-40 and beginning/end of ramps), Hydro T-Lines and obscure areas.  Wetherill merged all DTM data into the final DTL design file. A TIN file was made using Side Option: Dissolve; Stoking Options: Curve Stroke tolerance was 0.10’ and Minimum Linear Distance was 30’; Side Length was 100’.

Property & Right-of-Way:  Wetherill performed courthouse research on approximately 25 properties and populated the Property Owner database.  Letters were mailed to owners prior to beginning surveys.

Property lines were established using evidence collected in the field and deed/plat descriptions.  The existing R/W was established by using old plans along with field evidence/best-fit alignments as per the R/W Abstract provided by NCDOT.

Best-Fit Alignments:  Best-Fit alignments were made for –L- and all –Y- lines within the project limits.  Sugar Hill Rd. (-EL-) existing alignment was made using a combination of best fit routines from centering up the road on the south side of I-40, and using old plans provided on the north side of I-40.  I-40 (-EY-), I-40 West off ramp (-ERPA-), I-40 West on ramp (-ERPB-), I-40 East off ramp (-ERPC-) and I-40 East on ramp (-ELPC-) existing alignments were created using old plans provided along with field evidence (existing R/W monuments and/or centerline points).  Worley Rd. (-EY2-) and Condrey Rd. (-EY3-) existing alignments were created using best fit routines and centerline points.  All existing alignments were stationed starting at 10+00.00 except for I-40 (-EY-), which was stationed according to the old plans as request by NCDOT L&S.  Note: this project does not contain an (-EY1-) alignment.

Additional Comments:

Although NCDOT Photogrammetry provided the shell mapping for the FS file, the gas station near the beginning of the project on Sugar Hill Rd. was under construction at the time of the flight.  Additional features were collected during the survey to accurately show any changes to this area and was added to the final FS file.

All gravity utilities and pipe inverts were collected and mapped as per NCDOT Hydro guidelines.  Old plans were very helpful in providing insight to some of the drainage systems and were used for Quality Control.  There is a box culvert near the beginning of the project along Sugar Hill Rd. that crosses Sugar Hill Rd. and originates from the new gas station property.  The end of this box culvert could not be located on the gas station side of the Sugar Hill Rd.  Crews talked with the construction supervisor to inquire about this issue.  They were informed that the end of the culvert was tied into a number of hidden junction boxes under the parking lot of the gas station.  They were informed that Miller Engineering may be of assistance.  Miller Engineering was contacted and asked if they could provide as-built drawings of the drainage system.  Wetherill was informed that no as-built drawing existed, therefore, a note was placed in FS file stating such.

Utility poles were field collected and Photogrammetry mapped poles were replaced in the FS to show the proper symbol for all poles within the project limits.  A pole data file was created and transmitted.

Consultant delivered the following:

- Baseline text file

- Pole Data Report

- Final Survey Report

- Propcon database

- FS Design file

- DTM design file

- Project TIN file

- Project GPK file

- Deed Book/Plat Book (Digital PDF format)

- Certification Letter

- Final Progress Report

Report prepared by:

Tony Alford, PLS

Wetherill Engineering, Inc.

Survey Manager

559 Jones Franklin Road, Suite 164

Raleigh, NC  27606

Phone:  919.851.8077

Fax:  919.851.8107

wei@wetherilleng.com

559 Jones Franklin Road, Suite 164

Raleigh, NC  27606

Phone:  919.851.8077

Fax:  919.851.8107

wei@wetherilleng.com