Final Survey Report

October 2017

TIP I-5711

LS# 07-17-039

pROJECT# 50401.1.FS1

wake COUNTY

Description: I-40/85 at SR 1007 (Mebane Oaks Road) in Mebane

# Alignment

Baseline –BL- Beginning Station:		I-5711-39, Station 5+00.00

Baseline Alignment Design File:		I-5711_NCDOT_FS.dgn

BEST-FIT ALIGNMENT DESIGN FILE:		I-5711_NCDOT_FS.DGN

PROJECT .GPK FILE:				I-5711_LS_GPK.GPK

HORIZONTAL DATUM:				NAD 83/NA 2011

MONUMENT USED FOR PROJECT CONTROL:    NCDOT   I-5711-2

NORTHING		EASTING                   ELEVATION

844997.878 (ft)        1919627.354 (FT)               669.170’ (NCDOT)

SITE SCALE FACTOR:		1.0000524010

VERTICAL DATUM:	               NAVD 88

# EXISTING ALIGNMENT DATA

I-5711_NCDOT_FS.DGN

THE EXISTING ALIGNMENT WAS GENERATED BY A COMBINATION OF THE BEST FIT OF THE EXISTING CENTERLINE by the scan firm used for this project AND EXISTING HIGHWAY PLANS WITH CORRESPONDING MONUMENTATION.

# UTILITY DATA

I-5711_NCDOT_FS.DGN

GRAVITY UTILITIES PRESENT AT THE SITE WERE LOCATED BY CONVENTIONAL METHODS BY NCDOT.

# PlANIMETRIC - PHOTOGRAMMETRIC DATA

phOTOGRAMETRIC DATA was classified and supplemented by NCDOT USING CONVENTIONAL SURVEY METHODS.

# PROPERTY DATA

I-5711_NCDOT_FS.DGN

THIS FILE INCLUDES EXISTING RIGHT OF WAY and PROPERTY LINES. deed/plat books were prepared by GEL ENGINEERING.  there was no r/w abstract provided for this project by ncdot.  the property and r/w was computed using a combination of existing monumentation and deed/plat info along with existing alignment data.

I-5711_LS_POC.MDB

THIS FILE CONTAINS PROPERTY OWNER  INFORMATION AND CONTACTS.

# DTM’S

DTM GRAPHIC FILE:                                 I-5711_ls _DTM.dgn

DTM TIN FILE:                                            I-5711_ls _TIN.TIn

A COMBINATION OF FIELD SURVEY DATA AND PHOTOGRAMMETRIC DATA WAS USED TO PRODUCE THIS FILE.  SHOTS WERE GOTTEN ON DITCHES COVERED BY VEGITATION OR WATER.

# HYDROGRAPHIC DATA

I-5711_NCDOT_FS.DGN

HYDRO DATA WAS COLLECTED AS PER NCDOT HYDRO GUIDELINES.  fLUME CATCH BASIN INVERTS SHOWN ARE THE ELEVATION AT THE POINT OF ENTRY INTO THE FLUME.  tOP INVERTS ON CATCH BASINS AND DROP INLETS WERE OBTAINED USING SCAN DATA.

# POLE DATA

I-5711_LS_PDF.TXT

POLE DATA WAS OBTAINED BY GEL ENGINEERING.

# SUE DATA

I-5711_NCDOT_FS.DGN

NON GRAVITY UTILITIES PRESENT WERE COLLECTED UNDER A DIFFERENT ASSIGNMENT however the underground utilities have been merged into the submitted fs file.

# BLN DATA

I-5711_LS_baseline.TXT

baseline DATA WAS OBTAINED BY GEL ENGINEERING and approved by ncdot.

# COMMENTS

THIS IS AN ENGLISH PROJECT.

NO property strip map was prepared

CREW

PC                 michael green

TRANSIT     sYED qUIADRI

brain meilinger

scott trowbridge

ROD             WILLIE MILLER

MICHAEL GREEN

TRAVIS CONN

brian meilinger

sammy short

scott trowbridge

jack eddy

harold burgos

CADD OPERATOR(S)

GEORGE HART

JASON PREVATTE