U-2729 100% Plan Comments

- Need to add survey control/RW sheets to index once they come in.

- Sheet 1A – Utility Owners – Add AT&T, Windstream, Spectrum, and Piedmont Natural Gas

  - Additional utility owners added to 1A sheet

- Still a lot of question marks for ending sheet numbers; will need to be provided on final submittal.

- Letting on sheet 1A seems faint.

  - Letting Date weight Increased to show up more clearly.

- Typical section No. 21 – Pavement Schedule is still labeled preliminary and doesn’t correspond or cover all of the designations shown on the typical (e.g. no “M’s” are shown on typical, E2 is not in schedule, full depth dimension is not shown, etc..).  One more thing, since the shoulders are 6”, shouldn’t we use a trench section instead of shoulder borrow on the inside? Remove the Type B reference to UTBWC. Sorry I provided an old detail.

  - Updated Milling courses to match pavement schedule(Changed from “M” to “V”), added E2, Changed Shoulder to Trench Section, removed Type B note.

- Sheet 3G-1 is now missing from plans?

  - 3G-1 Sheets readded to plan set

- PSH 4 – Let’s add Beg/End stations for the CA

  - Begin and End CA labels with stationing added around RAB

- PSH 5 – Shouldn’t Begin Construction for the ramps and loops be removed since adding US 52 resurfacing in?

  - Begin Construction labels removed for ramps and loops

- PSH 5 – (internal discussion AL: You responded that a note was added to retain the concrete ditch between RPA and LPA; however, I don’t see the note and it doesn’t appear that this is even possible judging by the cross sections. Connie: this was for the area of the concrete ditch that was missing from the plans. This was added back in and the note is near the GR anchor unit.) In addition, the cross sections for these alignments need to be looked at more closely.  The interior ditch and slope information along with supers for RPA disappear at 19+00 -RPA.  Also, the ditch that is produced appears extremely shallow from  19+50 – 20+50.

- PSH 5 – The following comments aren’t addressed:

Were you able to confirm drainage? Mike Plummer with HDR is working with WSFCUC on the fencing issue and he will have an answer by 3/6. I will call him for an update.

- PSH 6 - DOT needs to provide info for Truist entrance. Provided 1/3/23, but proposed driveway is not shown.

- PSH 6 - Structure 651 – I believe the intent was to install a doghouse TBJB over the existing 18”RCP and tie 651 into it.  The 15” RCP run between 651 and 643 would then be eliminated.  This change has not been incorporated.

- PSH 6 - Turn off all of the existing invert layers for storm and sewer.

  - All Invert labels turned off

- PSH 6 – Structure 638 still calls to retain box, but the comment was to replace it.

- PSH 6 – Need a driveway for parcel 23

- Revise sidewalk for bus shelter area so pedestrians can walk around the shelters. It is ok to come to the back of curb with the sidewalk and the pad for the 2 shelters should be 7’ x 30’. The will be placing both shelters parallel to the road. Also need to connect the sidewalk that goes to the restaurant. The fiberglass pole is in the center of the proposed ROW. We were going to leave that pole in and grade around it. I think shifting the sidewalk toward the curb as mentioned above will remedy this.

  - Sidewalk and Bus Shelter shifted and revised. Sidewalk retied to restaurant parking lot.

- PSH 6 – was drainage to the south checked to address flooding comment?

- PSH 7 – DOT needs to provide site plan for Parcels 18 & 19. Provided 1/3/23. Proposed driveways not shown.

  - Proposed Development Driveways added and shown as dashed in Plans

- PSH 5 and XSC 18 – Need to make sure we have positive drainage from 5+50 to end where berm has been revised and is tying into existing fill slope.

- PSH 7, 8 and 9 – The notes to see PSH 8A and 8B are confusing since the actual PSH is numbered 8.

  - Notes revised to point to just sheet 8

- US 52 – Need to make sure that there’s enough room to rebuild interior shoulders without disturbing the guardrail. R-2247EB is showing replacing guardrail through the length of resurfacing.