NCDOT Final Survey Report for R-5681 Conventional

August 31, 2018

Project No.:  47427.1.1

TIP No.:  R-5861

County:  Cherokee

LS No.:  14-17-116

Job ID No.:  r5861-con-wet

Project Description:  US 19/129 Cherokee County from Georgia State Line to US 64

Survey limits:  Limits are defined by the design file entitled “R5861_SurveyLimitMap_171012.dgn”.  In general, the length of the project is 3.8 miles with a variable corridor width ranging from 300’ to 600’, as per the limits file.  This is an English project and will be mapped at a 1”=50’ Scale in Microstation using NCDOT Location & Surveys Workspace.

Date of Survey:  Field work was performed April 2018 – September 2018

Surveyed by:  Wetherill Engineering, Inc.

Control:  Control for this project was established by Wetherill Engineering from GPS points provided established by Wetherill Engineering during the Photo Panel phase.

Baseline points were established conventionally by turning two sets of angles for each point and closed on the GPS pair at the other end of the project.  The raw closure for this project was 1/20,000+ on the traverse loops.  The closure reports are included in the transmittal.

Elevations were established by running digital level loops from the control provided by NCDOT.  The level loop was processed and adjusted in a Level Loop spreadsheet.  The Level spreadsheets were included in the transmittal.  All Levels are based on GPS Point R-5861-5, which is the point that the project is localized about.

Control for this project is North Carolina 3200 State Plane Coordinates NAD83/NA2011 Horizontal Datum and NAVD88 Vertical Datum.  Geoid used was Geoid 12B.

Ten (10) benchmarks were set, collected and leveled to from the nearest baseline point.  All benchmarks were Benchlite nails set in trees.

DATUM DESCRIPTION:

THE LOCALIZED COORDINATE SYSTEM DEVELOPED FOR THIS PROJECT IS BASED ON THE NAD 83/NA2011 STATE PLANE COORDINATES FOR THE NCDOT GPS MONUMENT "R-5861-5" WITH A NORTHING OF 500131.564’, EASTING OF 477308.680’ ELEV= 1684.35’.  THE AVERAGE COMBINED FACTOR USED ON THIS PROJECT (GROUND TO GRID) IS 0.999806199; (1/x = 1.000193839).  ALL LINEAR DISTANCES ARE LOCALIZED HORIZONTAL DISTANCES.

THE VERTICAL DATUM IS NAVD 88. ELEVATIONS ARE BASED ON NCDOT GPS MONUMENT "R-5861-5" (ELEV. 1684.35’).

Utilities:  Wetherill performed the SUE Level BC under a separate request.

DTM (Digital Terrain Modeling):  NCDOT Photogrammetry provided DTM’s and QL-2 Lidar in the obscured areas for this project.  Wetherill Engineering surveyed Hydro T-Lines within the project limits.  All identified obscured areas were collected.  Pavement was provided by NCDOT and was mobile scanned.  Wetherill merged all DTM data into the final DTL design file. A TIN file was made using Side Option: Dissolve; Stoking Options: Curve Stroke tolerance was 0.10’ and Minimum Linear Distance was 30’; Side Length was 75’.

Property & Right-of-Way:  Wetherill performed courthouse research on approximately 147 properties and populated the Property Owner database.  Letters were mailed to owners prior to beginning surveys.

Property lines were established using evidence collected in the field and deed/plat descriptions.  The existing R/W was established by offsetting the Best Fit alignment in most areas and placed by offsetting the existing alignments at a width specified by NCDOT R/W Abstract, which was provided.

The North Carolina/Georgia State line was established from Deeds/Plats, property irons and R/W monuments found during the survey.

Best-Fit Alignments:  Best-Fit alignments were made for –L- and all –Y- lines within the project limits.  US19/129 – Blairsville Hwy. (-EL-), SR 1664 – Jim Cordell Rd. (-EY1-), SR 1590 – Moccasin Creek Rd. (-EY2-), SR 1556 – Martins Creek Rd. (-EY3-), SR 1583 – Lance Rd. (-EY4-), SR 1643 – Price Weaver Rd.  (-EY5-), SR 2583 – Lance Rd. (-EY6-), SR 1667 – Taylor Henson Rd. (-EY7-), SR 1580 – Hedden Rd. (-EY8-), SR 1667 – Taylor Henson Rd. (-EY9-) and US 64/74 (-EY10-) were computed by centering up the roadway and using best-fit routines and design alignments from NCDOT plans.  All existing alignments were stationed starting at 10+00.00.

Additional Comments:

Gravity utilities and pipe inverts were collected and mapped as per NCDOT Hydro guidelines.  Any features found in the field that was not shown on mapping was collected and added to the FS file.

Utility poles were field checked and Photogrammetry mapped poles were replaced in the FS to show the proper symbol for all poles within the project limits.

Field crews went door to door during classification and tried to obtain the approximate locations of septic tanks and septic fields from the property owners.  Approximate septic locations were shown on the FS mapping according to their findings.  Not all owners were home and some that were didn’t know where the septic locations were located.

Consultant delivered the following:

- Baseline text file

- Final Survey Report

- Propcon database

- FS Design file

- DTM design file

- Project TIN file

- Project GPK file

- Deed Book/Plat Book (Digital PDF format)

- Level loop spreadsheet

- Raw Traverse Closure Report

- RW02 Control Sheets

- Certification Letter

- Final Progress Report

Report prepared by:

Tony Alford, PLS

Wetherill Engineering, Inc.

Survey Manager

559 Jones Franklin Road, Suite 164

Raleigh, NC  27606

Phone:  919.851.8077

Fax:  919.851.8107

wei@wetherilleng.com

559 Jones Franklin Road, Suite 164

Raleigh, NC  27606

Phone:  919.851.8077

Fax:  919.851.8107

wei@wetherilleng.com