# Review Comments

| Submittal: | B-5831 90% Plans |  | Designer: | TGS Engineers | Submittal Date: | 08/15/24 |
| --- | --- | --- | --- | --- | --- | --- |
| Reviewer: | S. Merritt (RDY), M. Kamprath (HYD) |  | Organization: | Summit and VHB | Review Date: | 08/28/24 |
| Plans | Specifications | Calculations | Other |  |  |  |

| No. | DWG/Detail/ Page/Para | Comment | Response | Date | Disposition | Date |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | REVIEWER DISCLAIMER BELOW |  |  |  |  |
|  |  | Roadway Comments |  |  |  |  |
| 1 | TSH | Please update/verify WBS/FA Data in top right. Current Project Information on Sharepoint shows updates needed to: ROW FA: NHP-0077(006) CONST WBS: 45784.3.1. Please consider adding these to existing information. | Updated WBS/FA data in top right per comment. | 1/29/25 |  |  |
| 2 | TSH | From the revised May RDM Guidelines 13.2.10, please consider keeping municipal boundaries note contrary to 2018 Final Plans Checklist. | Added Municipal Boundaries note back to Title Sheet per comment. | 1/29/25 |  |  |
| 3 | 1A (Typical Sections) | In Index of sheets, please update typical sheet ending at 2A-5 to 2A-2 instead. (Is correct in provided 1A Spreadsheet) | Updated per comment. | 1/29/25 |  |  |
| 4 | 1A | Please update first XSC XPL sheet to start at X-2 consistent with plans. (Is correct in provided 1A Spreadsheet) | Updated per comment. | 1/29/25 |  |  |
| 5 | 1A | Please add Standard Drawings 840.31 and 840.32. | Added per comment. | 1/29/25 |  |  |
| 6 | 2B-1 | Please consider rotating Trenchless Installation 24” Welded Steel callout 90 degrees with leader line so callouts so do not conflict with note for TB2FI around -XOVR1-- Sta. 21+50.00. | Rotated text and added leader line per comment. | 1/29/25 |  |  |
| 7 | 2B-2 to 2B-3 | Were matchlines missed between 2B-2 and 2B-3? | No. 2B-1 & 2B-2 show XOVR1 and 2B-3 & 2B-4 show XOVR2. No matchlines between 2 & 3 needed. | 1/29/25 |  |  |
| 8 | 2B-2 | -X0VR2- Callouts are cut off from logo border. Please consider moving callouts and leaders. | Those callouts are shown on 2B-3 as intended for XOVR2. No change. | 1/29/25 |  |  |
| 9 | 2B-3 | -X-0VR2- Begin Construction is at 11+00.00. Is there any work being called out between 11+00 -11+50? | XOVR2 work should begin at 11+50 – the 11+00 STA was left over from a previous iteration. Corrected to STA 11+50. | 1/29/25 |  |  |
| 10 | 2B-3 | Please consider shifting profile cell slightly left to fall in line with grade and stationing labels of profile sheet grid. | Good catch. Shifted slightly per comment. | 1/29/25 |  |  |
| 11 | 2B-4 & 04 | Was PDE considered for installation and maintenance of DRN Str 0408/riprap outlet at Walters Property? | Yes, there was a R/W Revision dated 9/10/2024 adding a PDE at this location. | 1/29/25 |  |  |
| 12 | 2B-4 | Please consider providing a superelevation callout at the beginning of the detail sheet. | Added SE callout per comment. | 1/29/25 |  |  |
| 13 | 2B-4 | Clean up comments at -XOVR2- Sta, 30+00.. | Cleaned up callout text per comment. | 1/29/25 |  |  |
| 14 | 2B-4 | Please consider sliding barrier callout away from direction callout at -XOVR2- Sta, 30+00.. | Same fix as comment No. 13. | 1/29/25 |  |  |
| 15 | 04 | Please add shoulder taper rates/callouts. | Added shoulder taper rates and callouts where needed. | 1/29/25 |  |  |
| 16 | 04 | Please consider moving existing super callouts after super arrows and repositioning -L- 20+00 callout at Begin Project. | Moved SE callouts and repositioned -L- 20+00 cardinal station per comment. | 1/29/25 |  |  |
| 17 | 05 | Please consider adding excavation callout for cross-hatching. | Added (Typ.) Excavation callout per comment. | 1/29/25 |  |  |
| 18 | 05 | Please consider adding an existing superelevation callout/label or note for RT side of -Y2-. | Added existing SE label to the right side of -Y2- per comment. | 1/29/25 |  |  |
| 19 | 05 | Please verify need for TCE at End of -Y1- right. | Verified. TCE needed for contractor access across railroad and Town of Elkin property to the proposed/existing bridge. | 1/29/25 |  |  |
| 20 | 05 | Please consider adding callouts for Riprap and type at bridge for embankments. | Added (Typ.) riprap callout per comment. | 1/29/25 |  |  |
| 21 | 06 | Please consider adding BM data to -Y1- profile sheet. | This BM data is already referenced off -L- and there is no elevation work associated with Y1. No change. | 1/29/25 |  |  |
| 22 | X-5 to X-7 | Were fold lines considered on X-5 to X-7? | Not considered since only 2 cross sections are affected. The intent is clear. | 1/29/25 |  |  |
| 23 | Estimate | Please consider verifying clearing needed at -Y2- Sta. 14+60+/- to 15+70+/- LT to be adjusted to C/G and LS Grading worksheets and estimate | Good catch. Revised quantity sheets adding the clearing & grubbing quantity off -Y2-. | 1/29/25 |  |  |
| 24 | Estimate/DSS | Please verify masonry drainage structure quantity for consistency with DSS and estimate. Appear to be one off. | Agreed. Changed count to 28 from 27. However, the DSS changed whenever Temp. Drainage for the crossovers was added. | 1/29/25 |  |  |
| 25 | Estimate/DSS | Please verify quantities with grates/covers in estimate and DSS for consistency. (840.20) | Verified. The estimate had not been updated to match the most recent iteration of the DSS. They now match. | 1/29/25 |  |  |
| 26 | Estimate/Qty Package | Please consider matching 13” PCC Pavement say value with estimate value. (With and without dowels) | Per comment, updated said quantity sheets “SAY” value to match the estimate value. | 1/29/25 |  |  |
|  |  | Hydro Comments |  |  |  |  |
| 1 | 2B-1 | Is a false sump needed at 0402? | No. It is a paved median with no false sumps currently. | 1/29/25 |  |  |
| 2 | 2B-1 | Add false sump detail to this sheet or note where the detail can be found. | Detail added | 1/29/25 |  |  |
| 3 | 2B-1 | Note ditch grades on the ditch profile. | Ditch grades labeled | 1/29/25 |  |  |
| 4 | 2B-1 | Why is the last section of 2’ base ditch beginning at Sta. 17+92 on a reverse grade (going uphill)? | We created a sag at 17+92 and picking up the drainage coming from just beyond. | 1/29/25 |  |  |
| 5 | 2B-3 | Note ditch grades on the ditch profiles. | Ditch grades labeled | 1/29/25 |  |  |
| 6 | 2B-3 | At Sta. 14+00 of the ditch profile there is also a PI call-out that can be deleted. | Removed | 1/29/25 |  |  |
| 7 | 2B-3 | Profile stations don’t quite line up with the grid. Adjust. | Adjusted reference file | 1/29/25 |  |  |
| 8 | 2B-1 thru 2B-4 | Check need for traffic bearing structure for 2B02, 0402, 2B07, 2B10, 2B11, 2B12 and 2B13. They appear to be behind barrier or adequate distance from travel lane to not require TB. | Should keep as traffic bearing for the potential of construction equipment traveling in proximity. | 1/29/25 |  |  |
| 9 | 3D-1 | Temporary drainage needs to be included in the drainage summary. | Temporary drainage added to drainage summary sheet. | 1/29/25 |  |  |
| 10 | 3D-2 | Show elbows for pipe from 0605 to 0606. | Elbows added | 1/29/25 |  |  |
| 11 | 4 | Note the removal of existing DIs and CMPs on the left side where new system will be constructed. | Noted removal | 1/29/25 |  |  |
| 12 | 4 | Under Detail 2 check the stationing for the false sumps. Why aren’t they the same? | Corrected | 1/29/25 |  |  |
| 13 | 4 | Why is a HW shown at 0408 on a 24” pipe outlet? | Proposing an endwall since there is one there now. | 1/29/25 |  |  |
| 14 | 6 | Note removal of existing DIs at 0600, 0601, and 0602. | Removal of existing DI’s labeled | 1/29/25 |  |  |
|  |  | Reviewer Disclaimer |  |  |  |  |
|  |  | These review comments are not intended to replace the Designer’s QC process. The sole responsibility of errors and omissions is still on the sealing Engineer of Record. |  |  |  |  |