EXPLANATION OF SURFACE TYPES

| 00 | PRIMITIVE: On the official state system, unimproved, with no maintenance, natural surface, barely passable but usable by 4-wheel vehicles, and publicly traveled by small numbers of vehicles. |
| --- | --- |
| 10 | UNIMPROVED: A road, composed wholly (or with minor local exceptions) of natural ground of the region traversed, and that may or may not have been bladed. Does not conform in respect to alignment, grade, and drainage, (at least to the definition of "Graded and Drained,") and on which the only work done by public authority is that required to maintain a condition of bare passability for horse-drawn or motor vehicles. |
| 20 | GRADED & DRAINED: A road of natural earth (or with minor exceptions) aligned and graded to permit reasonably convenient use by motor vehicles. Drained by longitudinal and transverse drainage systems, sufficiently to prevent serious impairment of the road by surface water. |
| 30 | SOIL SURFACED-UNTREATED: Topsoil, sand, sand-clay, natural or artificial; also stabilized earth (stabilized with sand, selected materials, tar, Portland cement, chemicals or other admixtures). Not bituminous surface treated. |
| 41 | GRAVEL OR STONE-UNTREATED: Crushed, screened or natural gravel, crushed stone, slag, shale, shell, chert, marl, crushed brick, shist, disintegrated granite, iron ore, chats, and other similar fragmental material. Not bituminous surface treated. |

BITUMINOUS SURFACE-TREATED:

| 51 | Topsoil or Sand - Clay: Same as 30 except Bituminous Surface treated with a mat of about one inch or less in thickness. |
| --- | --- |
| 52 | Gravel or Stone: Same as 41 except Bituminous Surface-treated with a mat of about one inch or less in thickness. |
| 54 | Bituminous: Bituminous Surface-treatment over existing types 63, 65, or 66. |

MIXED BITUMINOUS:

A road, the wearing surface of which is one inch or more in compacted thickness, composed of gravel, stone, sand, or similar material, mixed with bituminous material under partial control as to grading and proportions.  Examples are mixed-in-place sand asphalt, or bituminous sand-road mix.  (Does not include hot premixed sand asphalt).

| 60 | Non-rigid Base: A mixed bituminous road the base course of which is of types other than 70, 80, 90, and the combined compacted thickness of surface and base is less than 7 inches, or the design is such as to produce a road having a characteristically low or non-uniform load-bearing capacity. |
| --- | --- |
| 61 | Rigid Base: A mixed bituminous road on a base of types 70, 80, 90. Also on any other type of base where the combined compacted thickness of surface and base is 7 inches or more, or where by reason of the presence of natural foundation materials which meet base requirements, the road has a characteristically high uniform load-bearing capacity. |