BITUMINOUS PENETRATION:

A gravel or stone road consisting of a base course and a wearing course one inch or more in compacted thickness, bound with bituminous material introduced by downward or upward penetration.

| 62 | Rigid Base: A bituminous penetration road on any base of types 70, 80, or 90. Also on any other type of base where the combined compacted thickness of surface and base is 7 inches or more, or where by reason of the presence of natural foundation materials which meet base requirements, the road has a characteristically high uniform load-bearing capacity. |
| --- | --- |
| 64 | Non-rigid Base: A bituminous penetration road the base course of which is other than type 70, 80, or 90, and the combined thickness of surface and base is less than 7 inches, or the design is such as to produce a road having a characteristically low or non-uniform load-bearing capacity. |

63  BITUMINUOS CONCRETE e.g. I1, I2, HDS: Bituminous concrete on

non-rigid base.  Bituminous concrete includes sheet asphalt, topeka, rock      asphalt, and similar bituminous mixes with precise control as to grading, proportions, and temperature, etc.

67 Bituminous concrete or sand asphalt, hot premixed over existing Portland cement concrete pavement.

SAND ASPHALT-HOT PREMIXED:  Premixed sand asphalt prepared under

reasonably precise specifications, and exact control as to grading

and proportions, on the following bases:

65 Sand asphalt, hot premixed on sand asphalt base, or on any other base or type, except on types 66 or 67.

66 Sand asphalt, hot premixed on existing bituminous concrete pavement.

70- 76 PORTLAND CEMENT CONCRETE: A wearing surface of plain or reinforced

Portland Cement Concrete.  Does not include Portland Cement stabilization.  Includes P.C. Concrete covered with a seal

coat or bituminous surface-treated, or other layer less than 1" in thickness.  Does not include P.C. concrete covered with retread, sand-mix, or sand asphalt over 1" thick.

80  BRICK:  Paving brick, with or without a bituminous mat less than one

inch in compacted thickness.

90  BLOCK:  Paving block (stone block, wooden block, asphalt

block) or other form of block, except vitrified paving brick.