63  BITUMINUOS CONCRETE e.g. I1, I2, HDS: Bituminous concrete on

non-rigid base (of substantial thickness).  Bituminous concrete

includes sheet asphalt, topeka, rock asphalt, and similar bituminous

mixes with precise control as to grading, proportions, and tempera-

ture, ect.

67 Bituminous concrete or sand asphalt, hot premixed over existing Portland cement concrete pavement.

SAND ASPHALT-HOT PREMIXED:  Premixed sand asphalt prepared under

reasonably precise specification, and exact control as to grading

and proportions, on any base as follows:

65 Sand asphalt, hot premixed on sand asphalt base, or on any other base or type, except on types 66 or 67.

66 Sand asphalt, hot premixed on existing bituminous concrete pavement.

70- 76 PORTLAND CEMENT CONCRETE:  Wearing surface of plain or reinforced

Portland Cement Concrete, with or without a bituminous mat of less

than one inch in compacted thickness.  Does not include Portland

Cement Stabilization.  Includes P.C. Concrete covered with a seal

coat or Bituminous Surface-treated, or any layer less than about 1"

Does not include P.C. Concrete Covered with Retread, Sand-Mix, or

Sand Asphalt over 1" thick.

80  BRICK:  Paving brick, with or without a bituminous mat less than one

inch in compacted thickness.

90  BLOCK:  Paving block (either stone block, wooden block, asphalt

block) or other form of block, except vitrified paving brick, with

or without a bituminous mat less than one inch in compacted

thickness.